3GPP TSG-RAN WG2 Meeting #100		R2-1712667

Reno, Nevada, USA, 26 November – 2 December, 2017

Agenda Item:	10.4.1.3.1

Source: 		Intel Corporation

Title: 		RRC signaling for PDCP data recovery

Document for:			Discussion and Decision

# Introduction

In the last meeting, it was briefly discussed how the PDCP recovery action is triggered as shown in the agreement below:

5	Introduce an explicit bit to indicate that RLC is to be re-established (to be used whenever MAC is reset).

FFS: How to trigger the PDCP recovery actions given the agreement 5 to be checked

In this contribution, the FFS point on the triggering of the PDCP data recovery is discussed.

# Discussion

## When PDCP data recovery is performed

PDCP data recovery is performed:

- During bearer type change from split bearer to non-split bearer when one of the split leg is released

- During PSCell change when there is no key change (i.e. As per the agreement in the Chair’s notes:  Handling 2 is supported (RA access, MAC reset, RLC re-established, PDCP recovery (for AM DRB), No security key change) are allowed in the specification for PSCell change. Trigger conditions for PDCP recovery will be captured in RRC spec. If PDCP is in master node then MN is involved)

Also, if RAN 3 reply back to RAN 2 that the 2c/2x bearer is supported (i.e. the anchor node is not changed but the logical channel is switched from MCG to SCG or vice versa), PDCP data recovery may also be needed.

In UP, PDCP data recovery is also being discussed to be used for UL switching (FFS on UE behaviour upon UL path switch (e.g. retransmissions and data recovery)). This is related to whether PDCP data recovery is needed or whether it will follow the existing LTE DC (where the old leg will continue RLC (re)transmission). This is not further discussed in the contribution until further clarity is made from UP session.

Observation: PDCP data recovery can be performed in the following cases:

- During bearer type change from split bearer to non-split bearer when one of the split leg is released

- During PSCell change when there is no key change (i.e. handling 2 in the offline discussion where RLC is re-established and MAC is reset)

- If 2c/2x bearer is supported where anchor node is not changed but the logical channel is changed from MCG to SCG or vice versa.

## How to specify the triggering of the PDCP data recovery

There are 2 ways to trigger PDCP data recovery procedure:

- Explicit RRC signalling

A flag is included for every DRB. When this flag is present for a DRB, the UE will perform PDCP data recovery for the DRB.

- Implicit based on the event (e.g. release of MCG or SCG logical channel configuration, PSCell change without key change)

It is included in procedure text as part of the event when MCG or SCG logical channel configuration is released or when PSCell change without key change.

Option (a) is similar to the reestablishPDCP flag which is included for each DRB to perform PDCP reestablishment during bearer type change between MCG (split) and SCG (split) bearer. Similar explicit signalling can be introduced for each DRB to trigger PDCP data recovery for bearer type change from split to non-split bearer or between 2c and 2x bearer.

Applying Option (a) for SN change without security change will mean that all the DRBs and/or SRB that have SCG logical channel configuration will need to be signalled in radio bearer configuration in order to provide the explicit signalling to perform PDCP data recovery. Alternatively, applying Option (b) will require further procedure text in the specification. As Option (a) is more aligned to PDCP re-establishment for security algorithm change (where explicit signalled re-establishPDCP is set for each radio bearer where the PDCP is anchored at SN), we prefer to align the same for PDCP data recovery.

Hence it is proposed that:

Proposal: For each configured DRB, an explicitly signalled indication is included to trigger the UE to perform PDCP data recovery. No implicit procedural rules are specified for triggering of PDCP data recovery.

The procedure text changes and an example of ASN.1 signalling for the inclusion of the signalling to trigger the UE to perform PDCP data recovery are shown in Annex A.

# Conclusion and proposals

It is requested that RAN 2 discuss and agree on the following proposal and observation:

Observation: PDCP data recovery can be performed in the following cases:

- During bearer type change from split bearer to non-split bearer when one of the split leg is released

- During PSCell change when there is no key change (i.e. handling 2 in the offline discussion where RLC is re-established and MAC is reset)

- If 2c/2x bearer is supported where anchor node is not changed but the logical channel is changed from MCG to SCG or vice versa.

Proposal: For each configured DRB, an explicitly signalled indication is included to trigger the UE to perform PDCP data recovery. No implicit procedural rules are specified for triggering of PDCP data recovery.

The procedure text and the ASN.1 in Annex A should be adopted, if it is not already so in the running CR.

# Annex A

##### 5.3.5.5.4	DRB addition/ modification

Editor’s Note: TODO: Add handling of the new QoS concept (mapping of flows; configuration of QFI-to-DRB mapping; reflective QoS…) but keep also EPS bearer handling for EN-DC.

The UE shall:

1>	for each drb-Identity value included in the drb-ToAddModList that is not part of the current UE configuration (DRB establishment including the case when full configuration option is used):

3>	establish a PDCP entity and configure it in accordance with the received pdcp-Config;

3> configure the PDCP entity with the security algorithms according to securityConfig and apply the keys (KUPenc) associated with the KeNB/S-KgNB as indicated in keyToUse;

2>	if the RRCReconfiguration message includes the fullConfig IE:

3>	associate the established DRB with corresponding included eps-BearerIdentity;

2>	else:

3>	indicate the establishment of the DRB(s) and the eps-BearerIdentity of the established DRB(s) to upper layers;

1>	for each drb-Identity value included in the drb-ToAddModList that is part of the current UE configuration:

2>	if reestablishPDCP is set

3>	re-establish the PDCP entity of this DRB as specified in 38.323, [REF], section 5.1.2;

2> else if recoveryPDCP is set

3>	perform PDCP data recovery procedure on the PDCP entity of this DRB as specified in 38.323, [REF], section 5.5;

2>	if the pdcp-Config is included:

3>	reconfigure the PDCP entity in accordance with the received pdcp-Config;

NOTE:	Removal and addition of the same drb-Identity in a single radioResourceConfigDedicated is not supported. In case drb-Identity is removed and added due to handover or re-establishment with the full configuration option, the eNB can use the same value of drb-Identity.

NOTE:	When determining whether a drb-Identity value is part of the current UE configuration, the UE does not distinguish which RadioBearerConfig and DRB-ToAddModList that DRB was originally configured in.  To re-associate a DRB with a different key (KeNB to S-KeNB or vice versa), the network provides the drb-Identity value in the (target) drb-ToAddModList and sets the reestablish flag. The network does not list the drb-Identity in the (source) drb-ToReleaseList.

NOTE:	When setting the reestablishPDCP flag for a radio bearer, the network ensures that the RLC receiver entities do not deliver old PDCP PDUs to the re-established PDCP entity. It does that e.g. by triggering a synchronized reconfiguration of the cell group hosting the old RLC entity or by releasing the old RLC entity.

### 6.3.2	Radio resource control information elements

#### –	RadioBearerConfig

The IE RadioBearerConfig is used to add, modify and release signalling- and/or data radio bearers. Specifically, this IE carries the parameters for PDCP and, if applicable, SDAP entities for the radio bearers.

RadioBearerConfig information element

-- ASN1START

-- TAG-RADIO-BEARER-CONFIG-START

RadioBearerConfig ::=					SEQUENCE {

srb-ToAddModList					SRBToAddModList			OPTIONAL, -- Cond HO-Conn

srb-ToReleaseList					INTEGER(3)				OPTIONAL, -- Cond HO-Conn

drb-ToAddModList					DRB-ToAddModList		OPTIONAL, -- Cond HO-toNR

drb-ToReleaseList					DRB-ToReleaseList		OPTIONAL, -- Need M

securityConfig						SecurityConfig			OPTIONAL  -- Cond TBD

}

SRB-ToAddModList ::=					SEQUENCE (SIZE (1..2)) OF SRB-ToAddMod

SRB-ToAddMod ::=						SEQUENCE {

srb-Identity								SRB-Identity,

-- may only be set if the cell groups of all linked logical channels are reset or released

reestablishPDCP						ENUMERATED{true}		OPTIONAL, -- Cond HO

pdcp-Config							PDCP-Config				OPTIONAL, -- Cond PDCP

...

}

DRB-ToAddModList ::=					SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod

DRB-ToAddMod ::=						SEQUENCE {

cnAssociation						CHOICE {

-- The EPS bearer ID determines the EPS bearer when NR connects to EPC using EN-DC

epsBearerIdentity					INTEGER(0..15)	OPTIONAL, -- Cond EPS-DRB-Setup

--	The SDAP configuration determines how to map QoS flows to DRBs when NR connects to the 5GC

sdap-Config							SDAP-Config		OPTIONAL -- Cond NGC

}

drb-Identity						DRB-Identity,

-- may only be set if the cell groups of all linked logical channels are reset or released

Reestablish-PDCP					ENUMERATED{true}	OPTIONAL, -- Cond HO

Recovery-PDCP						ENUMERATED{true}	OPTIONAL, -- Need N

pdcp-Config							PDCP-Config			OPTIONAL  -- Cond PDCP

}

DRB-ToReleaseList ::=					SEQUENCE (SIZE (1..maxDRB)) OF DRB-Identity

SecurityConfig							SEQUENCE {

securityAlgorithmConfig					SecurityAlgorithmConfig		OPTIONAL,	-- Need M

keyToUse								ENUMERATED{KeNB,sKgNB}		OPTIONAL,	-- Need M

...

}

-- TAG-RADIO-BEARER-CONFIG-STOP

-- ASN1STOP

RadioBearerConifg field descriptions

drb-Identity In case of DC, the DRB identity is unique within the scope of the UE i.e. an MCG DRB can not use the same value as split DRB. For a split DRB the same identity is used for the MCG- and SCG parts of the configuration.

cnAssociation Indicates if the bearer is associated with the eps-bearerIdentity (when connected to EPC) or sdap-Config (when connected to 5GC).

keyToUse Indicates if the bearer econfigured with this list is using KeNB of sKKgNB for deriving ciphering and integrity protection keys.

srb-Identity Value 1 is applicable for SRB1 only. Value 2 is applicable for SRB2 only. Value 3 is applicable for SRB3 only.

Recovery-PDCP Trigger UE to perform PDCP data recovery on the PDCP entity as specified in 38.323, [REF], section 5.5