3GPP TSG-RAN WG2 #100	R2-1712848

Reno, USA, 27th November – 1st December 2017

Agenda Item: 	10.2.7

Source:	Apple

Title:	TDM pattern coordination for single UL transmission

Document for:		Discussion

# 1	Introduction

For single UL transmission, according to RAN1 progress (as below), the single uplink transmission in EN-DC is based on network scheduling.

- When the UE is configured with multiple UL carriers on different frequencies (where there is at least one LTE carrier and at least one NR carrier of a different carrier frequency), but the UE operates on only one of the carriers at a given time among a pair of LTE and NR carriers - UE can be configured with - Case 1: DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-PCell - For scheduling/HARQ timing of LTE FDD carrier, DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-PCell is applied - UE is allowed to transmit NR UL signals at least in the subframe(s) where LTE UL transmission is not allowed according to the DL-reference UL/DL configuration - FFS whether or not a UE-specific subframe offset for the DL-reference UL/DL configuration can be configured considering system resource utilization and potential spec impact - Case 2: Release 15 LTE-FDD HARQ timing - No impact on LTE RAN1 specifications - Note: it doesn’t necessarily imply that UE has to support both cases

To avoid eNB and gNB scheduling on the same subframe, TDM pattern coordination between eNB and gNB is required. This contribution gives our view on the TDM pattern coordination.

# 2    Discussion

Single uplink transmission is achieved based on network scheduling. From UE perspective, in NR side there is no UE impact; and in LTE side there is also no UE impact for the case 2. But from network perspective, in order to provide correct implementation, TDM pattern for UL transmission between eNB and gNB is required for single UL transmission. Since the single uplink transmission mode enabling is based on UE capability and configured serving cell set, the TDM pattern should be UE specific.

Observation 1: TDM pattern for single UL transmission between nodes is UE specific.

Observation 2: TDM pattern coordination is required when the single uplink transmission mode is enabled.

Due to the single uplink transmission mode and TDM pattern are tightly related to the configured serving cell set, TDM pattern coordination should be in the MN/SN procedure which is associated to the serving cell set change.

Proposal 1: TDM pattern coordination between MN/SN should follow the principle of EN-DC MN/SN procedure,

-Proposal 1.1. In case of the configured serving cell set change, SN can provide the suggestion and MN makes the decision.

-Proposal 1.2. MN or SN can request the TDM pattern change, and the receipient can accept or reject it.

Since the cell related information exchanged in the network interface is in the inter-node RRC message, considering the inter-node RRC message also includes some configuration associated (i.e. RRM related) informaiton, TDM pattern related information should also be included in the inter-node RRC message.

Proposal 2: TDM pattern related information should be in the inter-node RRC message.

According to the procedure described in TS 37.340, table-1 gives the potential TDM coordination procedure between MN and SN.

Table-1. TDM pattern coordination (based on the procedure in TS 37.340)

| EN-DC Procedure | TDM coordination between MN/SN |
| --- | --- |
| SN addition | 1/ The single uplink transmission mode and configured serving cell set can be determined when MN receives “SgNB Addition Request Acknowledge” from SN. (step 2) 2/ SN can provide the suggested TDM pattern together in “SgNB Addition Request Acknowledge” msg; (step 2) 3/ MN provides the final TDM pattern to SN in “SgNB Reconfiguration Complete” msg. (step 5) |
| MN initiated SN modification | 1/ The single uplink transmission mode and configured serving cell set can be determined when MN receives “SgNB Modification Request Acknowledge” from SN. (step 2) 2/ SN can provide the suggested TDM pattern together in “SgNB Modification Request Acknowledge” msg; (step 2) 3/ MN provides the final TDM pattern to SN in “SgNB Reconfiguration Complete” msg. (step 6) |
| SN initiated SN modification | 1/ The single uplink transmission mode and configured serving cell set can be determined when MN receives “SgNB Modification Required” from SN. (step 1) 2/ SN can provide the suggested TDM pattern together in “SgNB Modification Required” msg based on current MN configuration; (step 1) 3/ MN provides the final TDM pattern to SN in “SgNB Modification Confirm” msg. (step 6) |
| MN/SN initiated SN change | 1/ The single uplink transmission mode and configured serving cell set can be determined when MN receives “SgNB Addition Request Acknowledge” from SN. 2/ SN can provide the suggested TDM pattern together in “SgNB Addition Request Acknowledge” msg; 3/ MN provides the final TDM pattern to SN in “SgNB Reconfiguration Complete” msg. |
| Inter-MN HO with/without SN change | 1/ The single uplink mode and configured serving cell set can be determined when MN receives “SgNB Addition Request Acknowledge” from SN. 2/ SN can provide the suggested TDM pattern together in “SgNB Addition Request Acknowledge” msg; 3/ MN provides the final TDM pattern to SN in “SgNB Reconfiguration Complete” msg. |

According to current RAN3 CR [1], the final TDM pattern is in SCG-ConfigInfo, and the suggested TDM pattern is in SCG-Config, and the corresponding TP based on [2] is provided in Annex.

Proposal 3: It is proposed to add TDM pattern in SCG-ConfigInfo and SCG-Config.

For the bitmap length of TDM pattern, 10-bit length is assumed in the following TP. If needed, the length could be updated according to the RAN1 suggestion.

# 3    Conclusion

According to the anlaysis in section 2, it is proposed that

Proposal 1: TDM pattern coordination between MN/SN should follow the principle of EN-DC MN/SN procedure,

-Proposal 1.1. In case of the configured serving cell set change, SN can provide the suggestion and MN makes the decision.

-Proposal 1.2. MN or SN can request the TDM pattern change, and the receipient can accept or reject it.

Proposal 2: TDM pattern related information should be in the inter-node RRC message.

Proposal 3: It is proposed to add TDM pattern in SCG-ConfigInfo and SCG-Config.

# 4    References

- Draft TS 36.423 CR r15

- R2-17xxxxx		Report of the e-mail [99bis#25][NR] on Inter-node RRC messages	RAN2#100

# 5    Annex (TP for TDM pattern)

#### –	SCG-Config

This message is used to transfer the SCG radio configuration as generated by the SgNB.

Direction: Secondary eNB to master eNB

SCG-Config message

-- ASN1START

SCG-Config ::=						SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE{

scg-Config-r15					SCG-Config-r15-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

SCG-Config-r15-IEs ::=				SEQUENCE {

scg-CellGroupdConfig				OCTET STRING (CONTAINING RRCReconfiguration)		OPTIONAL,

scg-RB-Config						OCTET STRING (CONTAINING RadioBearerConfiguration)	OPTIONAL,

tdmPattern-r15						BIT STRING(SIZE (10))				OPTIONAL,

configRestrictModReq-r15			ConfigRestrictModReqSCG-r12			OPTIONAL,

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

ConfigRestrictModReqSCG-r15-IEs ::=				SEQUENCE {

requestedBC-List-NR-r15							SEQUENCE OF INTEGER					OPTIONAL,

requestedBPC-List-NR-r15						SEQUENCE OF INTEGER					OPTIONAL,

...

}

-- ASN1STOP

SCG-Config field descriptions

scg-CellGroupConfig Contains the RRCReconfiguration message, used to (re-)configure the SCG configuration upon SCG establishment or modification, as generated (entirely) by the target SgNB

scg-RB-Config Contains the IE RadioBearerConfig, used to establish or reconfigure the SCG configuration, used to (re-)configure the SCG RB configuration upon SCG establishment or modification, as generated (entirely) by the target SgNB

configRestrictModReq Used by SN to re-negotiate SCG configuration restrictions previously set by MN to ensure UE capabilities are respected. E.g. used to request configure an NR band combination which use MN has previously forbidden.

tdmPattern Used in single uplink mode. Field encoded as a bit map, where bit N is set to "0" if the subframe is used for upink transmission in MN, and set to "1" if the subframe is used for upink transmission in SN.

#### –	SCG-ConfigInfo

This message is used by MNB to request the SgNB to perform certain actions e.g. to establish, modify or release an SCG. The message may include additional information e.g. to assist the SgNB to set the SCG configuration.

Direction: Master eNB or gNB to secondary gNB

SCG-ConfigInfo message

-- ASN1START

SCG-ConfigInfo ::=					SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE{

scg-ConfigInfo-r15					SCG-ConfigInfo-r15-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

SCG-ConfigInfo-r15-IEs ::=			SEQUENCE {

eutra-CapabilityInfo-r12		OCTET STRING (CONTAINING UECapabilityInformation)	OPTIONAL,

candidateCellInfoList			CandidateCellInfoList								OPTIONAL,

srb-ToAddModList				SRB-ToAddModInfoList								OPTIONAL,

drb-ToAddModList				DRB-ToAddModInfoList								OPTIONAL,

drb-ToReleaseList-r12			DRB-IdList											OPTIONAL,

configRestrictInfo-r12			ConfigRestrictInfoSCG-r12							OPTIONAL,

drx-InfoMCG-r12					DRX-Info											OPTIONAL,

sourceConfigSCG-r15				OCTET STRING (CONTAINING RRCReconfiguration)		OPTIONAL,

tdmPattern-r15						BIT STRING(SIZE (10))				OPTIONAL	nonCriticalExtension			SEQUENCE {}							OPTIONAL

}

SRB-ToAddModInfoList ::=				SEQUENCE (SIZE (1..2)) OF SRB-ToAddModInfo

SRB-ToAddModInfo ::=					SEQUENCE {

split									BOOLEAN,

ul-Duplication							BOOLEAN

}

DRB-ToAddModInfoList ::=				SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddModInfo

DRB-ToAddModInfo::=				SEQUENCE {

eps-BearerIdentity-r12			INTEGER (0..15)				OPTIONAL,

drb-Identity-r12				DRB-Identity,

drb-Type-r12					ENUMERATED {mcgSplit, scgSplit, scg}		OPTIONAL,

ul-path							ENUMERATED {mcg, scg, split}				OPTIONAL,

ul-Duplication					BOOLEAN,

ul-DataSplitThresh				ENUMERATED {ffs1, ffs2, ffs3}				OPTIONAL,

-- FFS how to signal ul information

...

}

DRB-IdList ::=				SEQUENCE (SIZE (1..maxDRB)) OF DRB-Identity

ConfigRestrictInfoSCG ::=			SEQUENCE {

restrictedBandCombinationNR-r15			INTEGER							OPTIONAL,

restrictedBasebandCombinationNR-NR-r15	SEQUENCE OF INTEGER				OPTIONAL,

maxMeasFreqsSCG-NR-r15					INTEGER							OPTIONAL,

...

}

DRX-Info ::=					SEQUENCE {

cycle							INTEGER,

offset							INTEGER

}

-- ASN1STOP

SCG-ConfigInfo field descriptions

drb-ToAddModInfoList Includes DRBs the SgNB is requested to establish or modify (DRB type change).

drb-ToReleaseListSCG Includes DRBs the SeNB is requested to release.

scg-RadioConfig Includes the current dedicated SCG configuration.

scg-ConfigRestrictInfo Includes fields for which SgNB is explictly indicated to observe a configuration restriction.

restrictedBandCombinationNR Indicates restrictions regarding the NR BCs the SN can configure by signalling the LTE BC selected by MN. The SN may configure any EN-BC including the indicated LTE BC selected by MN.

restrictedBasebandCombinationNR Indicates restrictions regarding the NR BPCs the SN can/ cannot configure i.e. by signalling the list of NR BPC the SN may configure.

tdmPattern Used in single uplink mode. Field encoded as a bit map, where bit N is set to "0" if the subframe is used for upink transmission in MN, and set to "1" if the subframe is used for upink transmission in SN.