3GPP TSG-RAN WG2 #100	R2-1712849

Reno, USA, 27th November – 1st December 2017

Agenda Item: 	10.2.7

Source:	Apple

Title:	Stage 2 TP for single UL transmission

Document for:		Discussion

# 1	Introduction

In this contribution, the stage 2 text proposal to TS37.340 on single UL transmission is presented.

# 2    Stage 2 TP on Single UL	Transmission

--------------<Start of Change>-------------

5	Layer 1 related aspects

5.x Support for Single Uplink Transmission in EN-DC

For EN-DC band combinations with inter-modulation (IMD) issues, UE is allowed to operate on only one of the carriers at a given time among a pair of LTE and NR carriers. In order to do that, UE indicates in UE capability that simultaneous UL transmission is not supported for certain band combinations that are categorized as difficult band combinations in [TS 38.101]. UE shall support simultaneous UL tranmsision on the non-difficult band combination. Within difficult band combinations, easy channel combinations are decided by the formulas in [TS 38.XXX]. UE shall support simultaneous UL tranmsission on the easy channel combinations.

Single uplink transmission is up to network implementation. From network perspective, TDM pattern coordination between eNB and gNB is required for good network implementation. From UE perspective, there is no UE impact in NR side, and in LTE side there are two cases:

- Case 1: DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-PCell

  - For scheduling/HARQ timing of LTE FDD carrier, DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-PCell is applied;

  - UE is allowed to transmit NR UL signals at least in the subframe(s) where LTE UL transmission is not allowed according to the DL-reference UL/DL configuration;

  - FFS whether or not a UE-specific subframe offset for the DL-reference UL/DL configuration can be configured considering system resource utilization and potential spec impact

- Case 2: Release 15 LTE-FDD HARQ timing.

--------------<End of Change>-------------