3GPP TSG-RAN WG2 #100	Tdoc R2-1712918

Reno, Nevada, USA, 27th November – 1st December 2017

Agenda Item:	9.7.3

Source:	Ericsson

Title:	Number of DRBs in E-UTRA connected to 5GC

Document for:	Discussion, Decision

# Introduction

This contribution discusses number of DRBs supported when LTE is connected to 5GC.

# Discussion

When LTE UE is connected to 5GC, the UP and CP protocol stack is expected to be following:

Figure 1	Illustration of UP and CP protocol stacks when Rel-15 E-UTRA UE is connected to 5GC

When UE is connected to 5GC, NR PDCP is used together with LTE RLC and LTE MAC. Therefore, UE is required to be able to configure NR PDCP with NR RRC configuration. RLC and MAC are configured with LTE RRC configuration. Having possibility to configure either NR RRC elements results that there is no limitation on number of DRBs from PDCP configuration perspective. However, the limitations are introduced by LTE RRC configuration, which only expands to DRB space of LTE RRC.

DRB configuration of E-UTRA – 5GC is limited by DRB space of LTE RRC configuration

Currently, the discussion is ongoing for LTE Rel-15 [1] whether LTE supports more than 8 DRBs. The number of DRBs may be increased for LTE and side-effect of this discussion may be that the LCID space for the LTE-Rel-15 is extended. The Limiting factor in the number of DRBs when connected to 5GC is the LTE MAC LCID space. Therefore, the LTE-Rel-15 UE connected to 5GC should follow the number of DRBs that is settle in the LTE discussion.

LTE MAC LCID space is setting the limitation for the number of DRBs supported by UE connected to 5GC

Number of DRBs for E-UTRA connected to 5GC is aligned with the number Rel-15 supports.

# Conclusion

In section 2 we made the following observations:

Observation 1	DRB configuration of E-UTRA – 5GC is limited by DRB space of LTE RRC configuration

Observation 2	LTE MAC LCID space is setting the limitation for the number of DRBs supported by UE connected to 5GC

Based on the discussion in section 2 we propose the following:

Proposal 1	Number of DRBs for E-UTRA connected to 5GC is aligned with the number Rel-15 supports.

# References

R2-1712957. Number of DRBs in E-UTRA. Ericsson. Reno, Nevada, USA, 27th Nov– 1st Dec 2017

R2-1712956. LCID space extension. Ericsson. Reno, Nevada, USA, 27th Nov– 1st Dec 2017