3GPP TSG-RAN WG2 #100	Tdoc R2-1712936

Reno, Nevada, USA, 27th November – 1st December 2017

Agenda Item:	10.3.2.5

Source:	Ericsson

Title:	RLC Status Report Format

Document for:	Discussion, Decision

# Introduction

RAN2#99bis agreed following:

Agreements:

- The AMD PDU formats in the draft TS are confirmed, and corresponding Editor’s note is removed

- No further optimizations on current STATUS PDU format are pursued

This contribution discusses about non-optimal design that ended up into the current Status Report Format. Even though, the working agreement is stating that no further optimizations on current Status Report Format are pursued, there should be still room for critical improvement reducing the overhead of the format.

# Discussion

The status report format can be seen to be constructed from static part and dynamic part. The static part of the format is minimum response which is needed to be constructed when there are no NACK_SNs. The static part currently contains DC, CPT, ACK_SN, E1 and R fields. Figure 2 is illustrating the RLC Status report format.

Static part of the RLC Status report format is minimum response when there are no NACK SNs.

Figure 2: Illustration of 12-bit RLC status report format with static part and dynamic part.

There are two status report formats, one with 12-bit SN and another with 18-bit SN. Both formats have currently E1-field which is used to indicate whether NACK_SN will follow. As shown in the contributions [1][2], E1-bit is not mandatory because the size of the static part of the status report block does not change. Therefore, observing the status report PDU size, parser may derive whether the status report format has NACK_SNs. In other words, the receiver knows total length of the status report format from the L-field of MAC sub-header. If the Status Report size equals to the static part (DC/CPT/ACK_SN) of the status report format, the NACK_SN field does not follow. Therefore, the E1-bit is not needed if there is no NACK_SN field.

E1-bit is not mandatory for static part of the Status Report format

However, removing the E1-bit from the consecutive blocks after the static part does not introduce overhead benefits because the blocks are byte-aligned. Therefore, whenever there is NACK_SN, the E1-bit may indicate that the consecutive NACK_SN will follow.

E1-bit is not mandatory but it helps parser processing when E1-bit is included whenever NACK_SN is included.

E1-bit is used only when there is NACK_SN, i.e. it is not used after ACK_SN

# Text Proposal for TS 38.322

#### 6.2.1.5	STATUS PDU

STATUS PDU consists of a STATUS PDU payload and a RLC control PDU header.

RLC control PDU header consists of a D/C and a CPT field.

The STATUS PDU payload starts from the first bit following the RLC control PDU header, and it consists of one ACK_SN and one , zero or more sets of a NACK_SN, an E1, an E2 and an E3, and possibly a pair of a SOstart and a SOend or a NACK range field for each NACK_SN.

Figure 6.2.1.5-1: STATUS PDU with 12 bit SN

Figure 6.2.1.5-2: STATUS PDU with 18 bit SN

# Conclusion

In section 2 we made the following observations:

Observation 1	Static part of the RLC Status report format is minimum response when there are no NACK SNs.

Observation 2	E1-bit is not mandatory but it helps parser processing when E1-bit is included whenever NACK_SN is included.

Observation 3	Figure 3 format requires only 2 bit-shifting patterns, one for 12-bit SN and another for 18-bit SN. This reduces parser implementation complexity.

Observation 4	Figure 3 supports main formats required for RLC Status reporting while respecting agreements in the previous meetings.

Based on the discussion in section 2 we propose the following:

Proposal 1	E1-bit is used only when there is NACK_SN, i.e. it is not used after ACK_SN

# References

R2-1711268. Remaining details of RLC STATUS PDU format. Nokia. TSG-RAN WG2 #99bis Prague, Czech Republic, 9th – 13th October 2017

R2-1711789. Presence of E1 in RLC Status Report. Samsung. TSG-RAN WG2 #99bis Prague, Czech Republic, 9th – 13th October 2017