3GPP TSG-RAN WG2 #100	Tdoc R2-1712951

Reno, Nevada, USA, 27th November – 1st December 2017

Agenda Item:	9.19

Source:	Ericsson

Title:	FGI20 limitation for DRBs

Document for:	Discussion, Decision

# Introduction

RAN2 received LS from SA2 on the number of DRBs in RAN2#99bis meeting in Prague (R2-1710245).

SA2 had discussed about the impacts of the maximum number of LTE bearers to the end user services implemented by LTE/EPS networks. Based on RAN2 discussion, the following agreements were made and LS sent back to SA2 (in R2-1712066):

It is possible to extend the number of DRBs  to 15 in Rel-15.  However, extending to 15 would have implication in RAN2 specs to extend the MAC header to support more logical channels which has an overhead impact.

The extension of DRB to the number less than above is possible without having to extend the MAC header and still keeping a few “reserved” bits in the MAC specification for further use.

Currently all UEs support 8 DRBs.  If the number of DRBs is extended, the number of DRBs supported will need to be signalled as part of UE AS capability and this will also need to be visible in the CN.

The current 5AM and 3UM bearers comes from the FGI bit 20.  More flexible combinations of number of AM and UM bearers is feasible with UE capability signalling.

The above response is valid for LTE connected to EPC as well as EN-DC (architecture option 3).

The number of DRBs in NR will be in the range of 16..32 but RAN2 has not concluded on the final number.

# Discussion

The LS R2-1710245 mentioned that the currently supported options for “E-UTRA connected to EPC” allow either 8 Acknowledged Mode (AM) or 5 AM + 3 Unacknowledged Mode (UM) bearers per UE. That is correct understanding as TS 36.331 FGI table in Annex B1 states:

| 7 | - RLC UM | - can only be set to 0 if the UE does neither support VoLTE nor MCPTT | Yes, if UE supports VoLTE, MCPTT, or both. Yes, if UE supports SRVCC to EUTRAN from GERAN. | No |
| --- | --- | --- | --- | --- |

| 20 | If bit number 7 is set to 0: - SRB1 and SRB2 for DCCH + 8x AM DRB If bit number 7 is set to 1: - SRB1 and SRB2 for DCCH + 8x AM DRB - SRB1 and SRB2 for DCCH + 5x AM DRB + 3x UM DRB NOTE: UE which indicate support for a DRB combination also support all subsets of the DRB combination. Therefore, release of DRB(s) never results in an unsupported DRB combination. | - Regardless of what bit number 7 and bit number 20 is set to, UE shall support at least SRB1 and SRB2 for DCCH + 4x AM DRB - Regardless of what bit number 20 is set to, if bit number 7 is set to 1, UE shall support at least SRB1 and SRB2 for DCCH + 4x AM DRB + 1x UM DRB | Yes | No |
| --- | --- | --- | --- | --- |

It is quite common assumption that the UE is configured with at least one UM bearer e.g. if it is receiving voice call. In that case, the UE supports only 5 AM bearers.  However, one use case could be that UE is configured e.g. with 1 UM bearers and 7 AM bearers. This kind of configuration could be supported from signalling point of view but FGI20 indicates that this case is not supproted.

We consider that introducing new capability to increase number of DRBs is quite straightforward change as it has limited specification impacts. Also benefit of that approach is that Logical Channel IDs are not consumed and thus also overhead is avoided.

Simplest way would be just to introduce a new capability bit that overrides FGI20 limitation. If the UE indicates that bit, then it should support any combinations of AM and UM bearers, up to 8 DRBs in total.

Introduce a new capability to overcome FGI20 limitation

If new capability is set, then the UE supports any combinations of AM and UM bearers, up to 8 DRBs in total

As this change is quite simple and only requires new capability bit, it could be even discussed if this change can be introduced in earlier releases like Rel-14.

Discuss in which release new capability bit is introduced like Rel-14

- CRs for Rel-14 are provided in R2-1712952 and R2-1712953.

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	Introduce a new capability to overcome FGI20 limitation

Proposal 2	If new capability is set, then the UE supports any combinations of AM and UM bearers, up to 8 DRBs in total

Proposal 3	Discuss in which release new capability bit is introduced like Rel-14