3GPP TSG-RAN WG2 #100	Tdoc R2-1713612

Reno, USA, 27th November – 1st December 2017

Agenda Item:	10.2.2.3

Source:	Ericsson, Nokia, Nokia Shanghai Bell, Vodafone

Title:	Support of ECN in NR

Document for:	Discussion, Decision

# Introduction

At RAN2#99 it was decided not to support an enhanced ECN mechanism as described in [1]. The question to support a basic ECN mechanism (similar to the one in LTE) was left open. In this paper we suggest to introduce a basic ECN mechanism to be captured in TS 38.300.

# Discussion

## ECN operation in LTE and NR

In this section we present the typical ECN operation in LTE and also NR. In Figure 1 a typical setup is shown. First of all, it should be noted that packets are queued on the IP layer. This decision is left for implementation, but this is the typical setup. At 1 the eNB detects congestion and the eNB sets the ECN bits of the first IP packet in the queue before transmitting it. How the eNB decides congestion has occurred and sets the ECN bits is left for eNB implementation, but typical triggers can be queueing time or queue length. The eNB can set the ECN bits because the IP packets have not yet been ciphered by PDCP. At 2 the ECN marked IP packets are received in the IP layer of the UE and the ECN markings are echoed back to the server, as application or transport layer feedback. At 3 the server takes the corresponding action, e.g. reduces the sending rate.

Figure 1 – ECN operation in LTE and NR

As it was agreed not to support an enhanced ECN mechanism in NR, we propose to support ECN in NR in the same way as in LTE.

As in LTE, NR supports ECN through a text in TS 38.300.

Adopt the text proposal below.

# Text proposal

This is the text proposal to TS 38.300.

## 2	References

[xx]	IETF RFC 3168 (09/2001): "The Addition of Explicit Congestion Notification (ECN) to IP".

## X.Y	Explicit Congestion Notification

The gNB and the UE support of the Explicit Congestion Notification (ECN) is specified in Section 5 of [xx] (i.e., the normative part of [xx] that applies to the end-to-end flow of IP packets), and below. ECN is beneficial especially for latency sensitive interactive applications such as chat and gaming as well as for real-time voice and video, this because loss as a congestion signal is avoided, losses that would otherwise necessitate retransmission of packets with additional application delay as a result.

# Conclusion

Based on the discussion in this paper we propose the following:

Proposal 1	As in LTE, NR supports ECN through a text in TS 38.300.

Proposal 2	Adopt the text proposal below.

# References

R2-1709469, "Efficient support for ECN in NR", Ericsson, RAN2#99, Berlin, Germany, 21st – 25th August 2017