3GPP TSG-RAN WG2 Meeting #100			     R2-1713657

Reno, U.S.A., November 27 – December 1, 2017

Agenda item:	10.3.2.6 (NR_newRAT-Core)

Source:	LG Electronics Inc.

Title: 	Simplification of RLC entity re-establishment

Document for:	Discussion and Decision

1.	Introduction

In the email discussion “[99bis#59] [NR UP/RLC] Open issues related to RLC”, the following proposals are made and the new section for RLC entity handling is added.

Proposal 1 RLC entity release procedure is specified in RLC specification (triggering discarding of all RLC SDUs and PDUs). Proposal 2 Restructure RLC sections, i.e. common section “RLC entity handling” with subsections “RLC establishment”, “RLC re-establishment”, “RLC release”.

In RLC entity handling section, the RLC entity establishment and RLC entity release subsections specify common behavior for all RLC modes, e.g. TM, UM, and AM. On the other hand, the RLC entity re-establishment subsection specifies the UE behavior for each RLC mode separately. However, we think that RLC entity re-establishment subsection can be aligned with other two subsections. In this contribution, we discuss simplification of RLC entity re-establishment briefly and provide the TP.

2.	Discussion

In RLC entity handling section, a single unified wording would be used for the triggering condition of each operation because the RLC entity establishment, re-establishment, and release operations are only triggered by RRC, i.e., “When upper layers request an RLC entity establishment/re-establishment/release”.

In the RLC entity re-establishment subsection, even though discarding operation in the current TP is classified by RLC and transmitting/receiving mode, the intention of the wording for discarding operation is to discard all possible remaining RLC SDUs, RLC SDU segments, and RLC PDUs at the transmitting/receiving side of each RLC mode. In other words, nothing should be remained after RLC entity re-establishment. Thus, we think that if a compact expression is possible to cover all cases of discarding operation, it would be good to have a compact wording, i.e., “discard all RLC SDUs, RLC SDU segments, and RLC PDUs, if any”.

In RLC entity release subsection, we think that same things for discarding operation can be adopted as in the RLC entity re-establishment subsection, i.e., “discard all RLC SDUs, RLC SDU segments, and RLC PDUs, if any”.

Proposal 1.	The RLC entity re-establishment subsection should be simplified.

TP for the proposal is provided in the Annex which is based on the TP for outcome of “[99bis#59] [NR UP/RLC] Open issues related to RLC”.

Proposal 2.	Adopt the TP in the Annex to the running TS 38.322.

3.	Conclusion

In this contribution, we discussed to simplify the RLC entity re-establishment subsection and provided the TP according to the proposal.

Proposal 1.	The RLC entity re-establishment subsection should be simplified.

Proposal 2.	Adopt the TP in the Annex to the running TS 38.322.

Annex. Text Proposal to TS 38.322

5.1 RLC entity handling 5.1.1 RLC entity establishment When upper layers request an RLC entity establishment, the UE shall: - establish a RLC entity; - set the state variables of the RLC entity to initial values; - follow the procedures in subclause 5.2. 5.1.2 RLC entity re-establishment When upper layers request an RLC entity re-establishment, the UE shall: - discard all RLC SDUs, RLC SDU segments, and RLC PDUs, if any; - stop and reset all timers; - reset all state variables to their initial values. 5.1.3 RLC entity release When upper layers request an RLC entity release, the UE shall: - discard all RLC SDUs, RLC SDU segments, and RLC PDUs, if any; - release the RLC entity.