3GPP TSG-RAN2 Meeting #100			     R2-1713668

Reno, USA, 27th November – 1st December, 2017           (Revision of R2-1709071)

Agenda item:	   10.3.4.3 (NR_newRAT-Core)

Source:	LG Electronics Inc.

Title: 		Further discussion on SDAP Configuration

Document for:	Discussion and Decision

1.	Introduction

During the e-mail discussion [99bis#18] L2 parameters in RRC, RAN2 discussed on SDAP configuration, and the following SDAP-Config information element was captured in text proposal for L2 parameter contents. However, RAN2 haven’t yet made a decision on whether configuration on SDAP header needs to be separate for UL and DL, and whether reflective QoS field is needed.

SDAP-Config information element

-- ASN1START

-- TAG-SDAP-CONFIG-START

SDAP-Config ::=						SEQUENCE {

-- TODO: Definition of PDUsessionID to be added

pduSession							PDUsessionID,

-- FFS: separate configuration for UL and DL

sdap-Header-DL 						ENUMERATED {present, absent},

sdap-Header-UL 						ENUMERATED {present, absent}

defaultDRB							BOOLEAN,

reflectiveQoS						BOOLEAN,	-- It is FFS whether this field is needed

-- FFS: Is the simple list sufficient? Replace by add/mod/release list? Or bitmap?

mappedQoSflows						SEQUENCE (SIZE (0..maxNrofQFIs)) OF QFI 									OPTIONAL, -- Need N

...

}

QFI :: = 							INTEGER (0..maxQFI)

-- TAG-SDAP-CONFIG-STOP

This document discusses the FFS fields highlighted above.

2.	Discussion

According to TS 38.300, the main services and functions of the SDAP layer, which is newly defined for the NR QoS framework, include:

- Mapping between a QoS flow and a DRB;

- Marking QoS flow ID in both DL and UL packets.

QoS flow is the finest granularity of QoS differentiation for the NR QoS framework, and is identified by QoS Flow ID. Traffic with the different QoS Flow ID value within a PDU session requires the different traffic forwarding treatment. For such different traffic forwarding treatment, UL QoS flow ID is sent in SDAP header with UL packet so that gNB can determine the QoS marking which is carried in an N3 encapsulation header of the UL packet. The gNB and UPF are going to treat the received UL packet based on the QoS marking (QoS Flow ID) in the N3 encapsulation header.

However, if there is only one UL QoS flow mapped to a DRB, the gNB can infer UL QoS Flow ID from the DRB over which the UL packets have been received. In that case, it is not necessary to send UL QoS Flow ID with the UL packets. The gNB may configure the SDAP not to attach SDAP header to SDAP SDUs to be submitted to the DRB.

Observation 1-1: For different traffic forwarding treatment, gNB needs to know UL QoS flow ID of the received UL packet.

Observation 1-2: If there is only one UL QoS flow mapped to a DRB, the gNB can infer the UL QoS Flow ID from the DRB over which the UL packets have been received.

Contrary to the uplink, for per-QoS flow differentiation, DL QoS flow ID does NOT need to be sent in SDAP header with DL packet because the traffic forwarding treatment on the radio interface is done on a per DRB basis. However, DL QoS flow ID can be included in the SDAP header when AS or/and NAS-level reflective QoS is triggered. As mentioned in LS reply on NAS Reflective QoS [1], the information that needs to be provided to UE along with the DL packet to enable NAS-level reflective QoS includes: 1) QoS Flow ID, and 2) indication that the DL packet is subject to NAS-level reflective QoS. More specifically, RQA (Reflective QoS Attribute) within a QoS flow’s QoS profile provided to the gNB indicates that some (not necessarily all) traffic carried on the QoS flow can be subject to NAS-level reflective QoS. For the QoS flow, packets that are subject to NAS-level reflective QoS processing in the UE are indicated with the Reflective QoS Indication (RQI) in the N3 encapsulation header.

However, processing for both NAS and AS reflective QoS is not required for the QoS flow which meets the following all conditions:

- Condition #1: RQA indicates that all traffic carried on the QoS flow is not subject to NAS-level reflective QoS.

- Condition #2: gNB decides to use only explicit RRC configuration for providing the QoS flow’s AS mapping information to the UE.

If all DL QoS flow(s), which is mapped to a DRB, meets the above conditions, the gNB may configure the SDAP not to attach SDAP header to SDAP SDUs to be submitted to the DRB.

Observation 2-1: For reflective QoS processing, UE needs to know DL QoS flow ID of the received DL packet and/or whether or not the DL packet is subject to reflective QoS.

Observation 2-2: If 5G-CN and gNB decide not to use NAS and AS reflective QoS for a QoS flow, the gNB does not need to send DL QoS flow ID and/or indicator for the QoS flow.

Depending on these observations, UL and DL SDAP header do not need to be sent in the different scenarios respectively. Therefore, we propose the following:

Proposal 1: Whether or not a SDAP header is present is independently configured for UL and DL.

On top of that, we don’t think that additional field indicating whether or not reflective QoS is used for QoS flows is needed. UE can naturally know whether the UE should perform the operation related to the reflective QoS, by using the information on whether or not SDAP header is present.

Proposal 2: Whether or not reflective QoS is used for QoS flows does NOT need to be configured by RRC.

3.	Proposal

In this document, we present our view on SDAP configuration. We have following proposals:

Proposal 1: Whether or not a SDAP header is present can be independently configured for UL and DL.

Proposal 2: Whether or not reflective QoS is used for QoS flows does NOT need to be configured by RRC.

References

[1] S2-1705244/R2-1707651	LS reply on NAS Reflective QoS		SA2