3GPP TSG-RAN WG2 Meeting #100		R2-1713674

Reno, NV, USA, 27th November - 1st December 2017

Agenda Item:	10.4.1.4.2

Source: 	MediaTek Inc.

Title:  	NR Filter Coefficients for Different Frequencies

Document for:	Discussion and decision

# Introduction

In email discussion [99bis#22] (filter coefficients), RAN2 discussed the configuration of filter coefficients for different frequencies. However, no conclusion could be made. In this paper, we discuss the need of configuring filter coefficients for different frequencies.

# Discussion

## Need of different filter coefficients

In NR, different frequency ranges has been defined.

Table 1: Definition of frequency ranges

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1 | 450 MHz – 6000 MHz |
| FR2 | 24250 MHz – 52600 MHz |

The channel variation in FR1 (lower frequency) may be much slower than in FR2 (higher frequency), as illustrated in the figure below.

Fig 1.	Channel variation for FR1 and FR2 in NR

Therefore, we should at least allow different filtering coefficients to be configured for the two frequency ranges.

Observation 1:	Channel variation in FR1 (lower frequency) may be much slower than in FR2 (higher frequency).

Proposal 1:	We should allow different filtering coefficients to be configured for FR1 and FR2.

## Configuration of filter coefficients

To configure different filtering coefficients to be configured for FR1 and FR2, we may introduce in measurement configuration two sets of filter coefficients. Since the range of FR2 is large, the network may even want to configure different filter coefficients for different frequencies in FR2. For future extension, we suggest that multiple sets of filter coefficients are defined in measurement configuration (measConfig), and each measObject is associated with a set of filter coefficients. In this way, a set of filter coefficients may be applied to multiple frequencies.

Proposal 2:	Multiple sets of filter coefficients are defined in measConfig, and each measObject is associated with a set of filter coefficients.

In the Annex, we provide proposed ASN.1 update in TS 38.331, based on the RRM TP under discussion [1].

# Conclusion

We have the following observation:

Observation 1:	Channel variation in FR1 (lower frequency) may be much slower than in FR2 (higher frequency).

It is proposed to discuss and decide on the following proposals:

Proposal 1:	We should allow different filtering coefficients to be configured for FR1 and FR2.

Proposal 2:	Multiple sets of filter coefficients are defined in measConfig, and each measObject is associated with a set of filter coefficients.

# Reference

- R2-17xxxxx Summary of email discussion [99bis#20][NR] TP on RRM, Ericsson

- 3GPP TS 36.133, Requirements for support of radio resource management, Release 15

# Annex: Proposed ASN.1 update in 38.331

#### –	MeasConfig

The IE MeasConfig specifies measurements to be performed by the UE, and covers intra-frequency, inter-frequency and inter-RAT mobility as well as configuration of measurement gaps.

MeasConfig information element

-- ASN1START

-- TAG-MEAS-CONFIG-START

MeasConfig ::=							SEQUENCE {

-- Measurement objects

measObjectToRemoveList					MeasObjectToRemoveList											OPTIONAL,

measObjectToAddModList					MeasObjectToAddModList											OPTIONAL,

-- Reporting configurations

reportConfigToRemoveList				ReportConfigToRemoveList										OPTIONAL,

reportConfigToAddModList				ReportConfigToAddModList										OPTIONAL,

-- Measurement identities

measIdToRemoveList						MeasIdToRemoveList												OPTIONAL,

measIdToAddModList						MeasIdToAddModList												OPTIONAL,

-- Other parameters

quantityConfigList						SEQUENCE (SIZE (1..maxQuantityConfigs)) OF QuantityConfig		OPTIONAL

-- s-Measure config

s-MeasureConfig							CHOICE {

ssb-rsrp								RSRP-Range,

csi-rsrp								RSRP-Range

} 																										OPTIONAL,

-- Placehold for measGapConfig

measGapConfig							MeasGapConfig													OPTIONAL

}

MeasObjectToRemoveList ::=				SEQUENCE (SIZE (1..maxNrofObjectId)) OF MeasObjectId

MeasIdToRemoveList ::=					SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasId

ReportConfigToRemoveList ::=			SEQUENCE (SIZE (1..maxNrofReportConfigId)) OF ReportConfigId

-- TAG-MEAS-CONFIG-STOP

-- ASN1STOP

MeasConfig field descriptions

measGapConfig FFS Definition of measurement gaps in NR.

measIdToAddModList List of measurement identities

measIdToRemoveList List of measurement identities to remove.

measObjectToAddModList List of measurement objects to add and/or modify.

measObjectToRemoveList List of measurement objects to remove.

reportConfigToRemoveList List of measurement reporting configurations to remove.

s-MeasureConfig The setting of s-MeasureConfig to ssb-rsrp indicates that the UE shall compare the configured threshold with the measured PCell RSRP based on SS/PBCH block, while the setting of s-MeasureConfig to csi-rsrp indicates that the UE shall compare the configured threshold with the measured PCell RSRP based on CSI-RS. If the UE is in EN-DC, the UE shall compare the configured threshold with the measured PSCell RSRP based on SS/PBCH block or CSI-RS.

quantityConfigList List of measurement quantity configurations.

#### –	MeasObjectNR

The IE MeasObjectNR specifies information applicable for SS/PBCH block(s) intra/inter-frequency measurements or CSI-RS intra/inter-frequency measurements.

MeasObjectNR information element

-- ASN1START

-- TAG-MEAS-OBJECT-NR-START

MeasObjectNR ::=							SEQUENCE {

carrierFreq									ARFCN-ValueNR,

--RS configuration (e.g. SMTC window, CSI-RS resource, etc.)

referenceSignalConfig						ReferenceSignalConfig														OPTIONAL,

--Consolidation of L1 measurements per RS index

absoluteThresholdCellQuality			SEQUENCE {

absThreshSS-BlocksConsolidation			ThresholdNR																	OPTIONAL,

absThreshCSI-RS-Consolidation			ThresholdNR																	OPTIONAL

}																														OPTIONAL,

--Config for cell measurement derivation

maxBeamsCellQuality						SEQUENCE {

nroSS-BlocksToAverage					INTEGER (1..maxNroSS-BlocksToAverage)										OPTIONAL,

nroCSI-RS-ResourcesToAverage			INTEGER (1..maxNroCSI-RS-ResourcesToAverage)								OPTIONAL

}

OPTIONAL,

--Frequency-specific offsets (only for events A3, A6)

offsetFreq									Q-OffsetRangeList,

-- Cell list

cellsToRemoveList							CellIndexList																OPTIONAL,

cellsToAddModList							CellsToAddModList															OPTIONAL,

-- Black list

blackCellsToRemoveList						CellIndexList																OPTIONAL,

blackCellsToAddModList						BlackCellsToAddModList														OPTIONAL,

-- White list

whiteCellsToRemoveList						CellIndexList																OPTIONAL,

whiteCellsToAddModList						WhiteCellsToAddModList														OPTIONAL

-- Associated quantity config

quantityConfigId							QuantityConfigId			OPTIONAL,

}

-- TODO: To be updated based on email discussion[99#31][NR] Additional information for SSB and CSI-RS config (Ericsson)

ReferenceSignalConfig::=     			 SEQUENCE {

ssb-MeasurementTimingConfiguration		SSB-MeasurementTimingConfiguration										OPTIONAL,

ssbPresence								CHOICE {

present									SEQUENCE {

frequencyOffset							TYPE_FFS!

-- FFS: How to give offset and timing if there are multiple SSBs on a carrier, or coding details details of the offset

},

notPresent								SEQUENCE {

-- FFS: How to inform the UE where else to find the SSB. FFS whether to indicate here a carrier or a cell ID or multiple cell IDs

},

}

-- CSI-RS resources to be used for for CSI-RS based RRM measurements

csi-rs-ResourceConfig-Mobility			SEQUENCE (SIZE (1..maxNrofCSI-RS-Resources))	OF CSI-RS-ResourceConfig-Mobility	OPTIONAL -- Need N

-- Indicates whether the UE can utilize serving cell timing to derive the index of SS block transmitted by neighbour cell:

useServingCellTimingForSync				BOOLEAN

}

-- A measurement timing configuration

SSB-MeasurementTimingConfiguration ::= 	SEQUENCE {

-- Primary measurement timing configuration. Applicable for intra- and inter-frequency measurements.

smtc1									SEQUENCE {

-- Timing (periodicity and offset) of the half frames for receptions of SS/PBCH blocks for SMTC-Config:

ssb-Timing								TYPE_FFS!,

-- Duration of the half frames for SMTC-Config:

ssb-Duration							TYPE_FFS!

},

-- Secondary measurement timing confguration for explicitly signalled PCIs. Supported only for intra-frequency measurements.

smtc2 									SEQUENCE {

-- PCIs that are known to follow this SMTC.

pci-List								SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysicalCellId		OPTIONAL

-- Periodicity for the given PCIs. Timing offset and Duration as provided in smtc1.

periodicty							TYPE_FFS!

}																													OPTIONAL -- Cond IntraFreq

}

CSI-RS-ResourceConfig-Mobility ::= 		SEQUENCE {

csi-rs-ResourceId-RRM					CSI-RS-ResourceId-RRM,

cellId									PhysicalCellId,

-- subcarrier spacing of CSI-RS. It can take the same values available also for the data channels and for SSB

subcarrierSpacing						SubcarrierSpacing,

-- Contains periodicity and slot offset for periodic/semi-persistent CSI-RS (see 38.211, section x.x.x.x)

slotConfigPeriodicity					ENUMERATED {sf5, sf10, sf20, sf40, [sf80, sf160]},

slotConfigOffset						INTEGER (0..XX),

-- Number of ports for CSI-RS (see 38.211, section x.x.x.x)

nrofAntennaPorts						ENUMERATED{X,...},

-- Resource Element mapping pattern for CSI-RS (see 38.211, section x.x.x.x)

resourceElementMappingPattern			TYPE_FFS!,

-- TODO: ADD DESCRIPTION (see 38.211, section x.x.x.x)

csi-rs-TransmissionBW					ENUMERATED {x,y,z, ...},

-- TODO: ADD DESCRIPTION (see 38.215, section x.x.x.x)

csi-rs-MeasurementBW					ENUMERATED {x,y,z, ...},

-- TODO: ADD DESCRIPTION (see 38.211, section x.x.x.x)

sequenceGenerationConfig				TYPE_FFS!

}

CSI-RS-ResourceId-RRM ::= 				INTEGER (0..maxNrofCSI-RS-ResourcesRRM-1)

Q-OffsetRangeList ::=					SEQUENCE {

rsrpOffsetSSB							Q-OffsetRange				DEFAULT dB0,

rsrqOffsetSSB							Q-OffsetRange				DEFAULT dB0,

sinrOffsetSSB							Q-OffsetRange				DEFAULT dB0,

rsrpOffsetCSI-RS						Q-OffsetRange				DEFAULT dB0,

rsrqOffsetCSI-RS						Q-OffsetRange				DEFAULT dB0,

sinrOffsetCSI-RS						Q-OffsetRange				DEFAULT dB0

}

ThresholdNR ::=							SEQUENCE{

threshold-RSRP							RSRP-Range					OPTIONAL,

threshold-RSRQ							RSRQ-Range					OPTIONAL,

threshold-SINR							SINR-Range					OPTIONAL

}

CellsToAddModList ::=					SEQUENCE (SIZE (1.. maxNrofCellMeas)) OF CellsToAddMod

CellsToAddMod ::=						SEQUENCE {

cellIndex								INTEGER (1.. maxNrofCellMeas),

physCellId								PhysCellId,

cellIndividualOffset					Q-OffsetRangeList

}

BlackCellsToAddModList ::=				SEQUENCE (SIZE (1.. maxNrofCellMeas)) OF BlackCellsToAddMod

BlackCellsToAddMod ::=					SEQUENCE {

cellIndex								INTEGER (1.. maxNrofCellMeas),

physCellIdRange							PhysCellIdRange

}

WhiteCellsToAddModList ::=				SEQUENCE (SIZE (1.. maxNrofCellMeas)) OF WhiteCellsToAddMod

WhiteCellsToAddMod ::=					SEQUENCE {

cellIndex								INTEGER (1.. maxNrofCellMeas),

physCellIdRange							PhysCellIdRange

}

-- TAG-MEAS-OBJECT-NR-STOP

-- ASN1STOP

MeasObjectNR field descriptions

blackCellsToAddModList List of cells to add/ modify in the black list of cells.

blackCellsToRemoveList List of cells to remove from the black list of cells.

carrierFreq Identifies NR carrier frequency for which this configuration is valid.

cellIndex Entry index in the cell list. An entry may concern a range of cells, in which case this value applies to the entire range.

cellIndividualOffset Cell individual offsets applicable to a specific cell.

cellsToAddModList List of cells to add/ modify in the cell list.

cellsToRemoveList List of cells to remove from the cell list.

nroCSI-RS-ResourcesToAverage Indicates the maximum number of measurement results per beam based on CSI-RS resources to be averaged. The same value applies for each detected cell in that carrierFreq.

nroSS-BlocksToAverage Indicates the maximum number of measurement results per beam based on SS/PBCH blocks to be averaged. The same value applies for each detected cell in that carrierFreq.

offsetFreq Offset values applicable to the carrier frequency.

physCellId Physical cell identity of a cell in the cell list.

physCellIdRange Physical cell identity or a range of physical cell identities.

absThreshCSI-RS-Consolidation Absolute threshold for the consolidation of measurement results per CSI-RS resource(s) from L1 filter(s). The values above the threshold are used as input to the derivation of cell measurement results as described in 5.5.3.3 and the L3 filter(s) per CSI-RS resource as described in 5.5.3.2.

absThreshSS-BlocksConsolidation Absolute threshold for the consolidation of measurement results per SS/PBCH block(s) from L1 filter(s). The values above the threshold are used as input to the derivation of cell measurement results as described in 5.5.3.3 and the L3 filter(s) per SS/PBCH block index as described in 5.5.3.2.

whiteCellsToAddModList List of cells to add/modify in the white list of cells.

whiteCellsToRemoveList List of cells to remove from the white list of cells.

quantityConfigId The identification of quantityConfig as specified in the corresponding measConfig, associated with this measObject,