3GPP TSG-RAN WG2 Meeting #100                                                           				            R2-1713851

Reno, USA, 27th November – 1st December 2017                                     	       Revision of R2-1713228

Agenda item:	10.3.1.6

Source:	CMCC

Title:	Common C-RNTI for common PDCCH

Document for:      Discussion

Introduction

In 6.1.2.3	of TR 38.802, the requirements for control channel are depicted.  And Common PDCCH is introduced as follows [1],

NR supports a group common PDCCH carrying at least slot format related information. If the UE does not receive the group common PDCCH the UE should be able to receive at least PDCCH in a slot, at least if the gNB did not transmit the group common PDCCH. The network will inform through RRC signalling the UE whether to decode the group common PDCCH or not. Note that common does not necessarily imply common per cell. Also, the term group common PDCCH refers to a channel (either a PDCCH or a separately designed channel) that carries information intended for the group of UEs. Slot format related information and defined as information from which the UE can derive at least which symbols in a slot that are DL, UL (for Rel-15), and other, respectively. The UE will have the possibility to determine whether some blind decodings can be skipped based on information on a group common PDCCH (if present). When monitoring for a PDCCH, the UE should be able to process a detected PDCCH irrespective of whether the group common PDCCH is received or not.

Obviously, we can conclude from the text that common PDCCH shall be supported by RRC and MAC. In this contribution, we discuss that a new type of RNTI should be introduced for common PDCCH.

In legacy LTE, there are many types of RNTI in TS 36.321 as follows [2],

7.1	RNTI values

RNTI values are presented in Table 7.1-1 and their usage and associated Transport Channels and Logical Channels are presented in Table 7.1-2.

Table 7.1-1: RNTI values.

| Value (hexa-decimal) | RNTI |
| --- | --- |
| 0000 | N/A |
| 0001-003C | RA-RNTI, C-RNTI, Semi-Persistent Scheduling C-RNTI, Temporary C-RNTI, TPC-PUCCH-RNTI and TPC-PUSCH-RNTI (see note) |
| 003D-FFF3 | C-RNTI, Semi-Persistent Scheduling C-RNTI, Temporary C-RNTI, TPC-PUCCH-RNTI and TPC-PUSCH-RNTI |
| FFF4-FFFC | Reserved for future use |
| FFFD | M-RNTI |
| FFFE | P-RNTI |
| FFFF | SI-RNTI |

NOTE:	A UE uses the same C-RNTI on all Serving Cells.

Table 7.1-2: RNTI usage.

| RNTI | Usage | Transport Channel | Logical Channel |
| --- | --- | --- | --- |
| P-RNTI | Paging and System Information change notification | PCH | PCCH |
| SI-RNTI | Broadcast of System Information | DL-SCH | BCCH |
| M-RNTI | MCCH Information change notification | N/A | N/A |
| RA-RNTI | Random Access Response | DL-SCH | N/A |
| Temporary C-RNTI | Contention Resolution (when no valid C-RNTI is available) | DL-SCH | CCCH |
| Temporary C-RNTI | Msg3 transmission | UL-SCH | CCCH, DCCH, DTCH |
| C-RNTI | Dynamically scheduled unicast transmission | UL-SCH | DCCH, DTCH |
| C-RNTI | Dynamically scheduled unicast transmission | DL-SCH | CCCH, DCCH, DTCH |
| C-RNTI | Triggering of PDCCH ordered random access | N/A | N/A |
| Semi-Persistent Scheduling C-RNTI | Semi-Persistently scheduled unicast transmission (activation, reactivation and retransmission) | DL-SCH, UL-SCH | DCCH, DTCH |
| Semi-Persistent Scheduling C-RNTI | Semi-Persistently scheduled unicast transmission (deactivation) | N/A | N/A |
| TPC-PUCCH-RNTI | Physical layer Uplink power control | N/A | N/A |
| TPC-PUSCH-RNTI | Physical layer Uplink power control | N/A | N/A |
| G-RNTI | Dynamically scheduled SC-PTM transmission | DL-SCH | SC-MTCH |

Discussion

In LTE, the following UE identities are used at cell level [3],

-	C-RNTI: unique identification used for identifying RRC Connection and scheduling;

-	Semi-Persistent Scheduling C-RNTI: unique identification used for semi-persistent scheduling;

-	Temporary C-RNTI: identification used for the random access procedure;

-	TPC-PUSCH-RNTI: identification used for the power control of PUSCH;

-	TPC-PUCCH-RNTI: identification used for the power control of PUCCH;

-	SL-RNTI: identification used for sidelink communication scheduling;

-	Random value for contention resolution: during some transient states, the UE is temporarily identified with a random value used for contention resolution purposes.

The following identities are used for SC-PTM transmission [3]:

-	SC-RNTI:	Identifies transmissions of the SC-MCCH;

-	SC-N-RNTI: Identifies SC-MCCH change notification;

-	G-RNTI:	Identifies transmissions of a SC-MTCH.

UE uses different types of RNTIs to identify specific functions at the physical layer. The explicit definition of RNTI can reduce the complexity of blind detection of UEs. Therefore, the definition of RNTI in NR shall follow the same principle as in LTE.

Observation 1: It is necessary to define different types of RNTIs in NR.

As indicated in TR 38.802, a group common PDCCH, which refers to a channel that carries the information for a group of UEs, is introduced in NR. In our opinion, this function is applicable for the transmission to a group of UEs instead of transmitting dedicated DCI to each UE. The introduction of common PDCCH is useful for scenarios such as mMTC with large amount of connections and small burst packets. For example, the network side can use common PDCCH to trigger a group of terminals to report status information in an industrial scenario. Obviously, the mechanism of common PDCCH is an efficient manner to reduce the occupancy of physical channels.  As a result, a type of RNTI, which is used for scrambling the common PDCCH, should be introduced in NR.

It should be noted that LTE has introduced the G-RNTI, which is used for SC-PTM scenario to identify the transmission of a SC-MTCH. As we can see from above, the G-RNTI adopted in LTE is also intended for a group of UEs. Therefore, it is necessary for RAN2 to discuss whether similar mechanism could be reused in NR.

Proposal 1: RAN2 is kindly asked to explicitly define a type of RNTI for scrambling the common PDCCH in NR.

Proposal 2: RAN2 is kindly asked to discuss whether similar mechanism (e.g. the G-RNTI in LTE) could be reused in NR.

Conclusions

In this contribution, the observation and proposals  are listed as below:

Observation 1: It is necessary to define different types of RNTIs in NR.

Proposal 1: RAN2 is kindly asked to explicitly define a type of RNTI for scrambling the common PDCCH in NR.

Proposal 2: RAN2 is kindly asked to discuss whether similar mechanism (e.g. the G-RNTI in LTE) could be reused in NR.

Reference

[1] 3GPP TR 38.802 V2.0.0 (2017-03)

[2] 3GPP TS 36.321 V14.2.0 (2017-03)

[3] 3GPP TS 36.300 V14.1.0 (2016-12)