3GPP TSG-RAN WG2 #100	R2-1714047

Reno, USA, November 27th - December 1st 2017

Agenda item:	10.3.1.4.2

Source:	Nokia

Title:	TP on beam recovery

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

In RAN2#100 common session, the following was captured in the minutes:

Agreements

1	The reception of the gNB response to beam recovery request sent on RACH is based on the monitoring of a PDCCH addressed to C-RNTI within a time duration configured by RRC.

2	Beam recovery can take place on a candidate beam (e.g. beams above threshold) with dedicated PRACH resources either associated with an SSB or CSI-RS resource.

FFS In which spec the criteria for a candidate beam for beam recovery is specified

3: 	When more than one beam is a valid candidate, it is up to UE implementation to select the beam.

FFS Whether we need to configure candidate beams for recovery with a mixture of SSB based and CSI-RS resource based beams. Any RAN1 agreement on this can be checked

FFS Behaviour in case the beam recovery attempt is not successful

FFS Whether beam recovery is supported using CBRA.

Furthermore, the UP session discussed the issue further and agreed the following.

Agreements

- Beam failure recovery using a dedicated PRACH preamble is specified in the MAC and triggered upon indication from Physical layer.  RAN2 assumes that the PHY layer does the detection of beam failure.

- Beam selection is specified in the MAC similar to the HO case

- The UE uses contention free when there is a beam associated to a dedicated “preamble/resource” and the beam is above a threshold.  Otherwise use contention based.

R2-1714047	TP on beam recovery 	Nokia

=>	Review the two TPs from R2-1713380	and R2-1712870 to determine what is the best way to capture

[CB 516]

# 2	TP into TS 38.321

*** Start of change ***

## 5.1	Random Access procedure

### 5.1.1	Random Access procedure initialization

The Random Access procedure described in this subclause is initiated by a PDCCH order, by the MAC entity itself, by beam failure indication from lower layer, or by RRC for the events in accordance with TS 38.300 [2]. There is only one Random Access procedure ongoing at any point in time in a MAC entity. The Random Access procedure on an SCell other than PSCell shall only be initiated by a PDCCH order with ra-PreambleIndex different from 0b000000.

NOTE:	If the MAC entity receives a request for a new Random Access procedure while another is already ongoing in the MAC entity, it is up to UE implementation whether to continue with the ongoing procedure or start with the new procedure (e.g. for SI request).

Editor's note: Editor thinks RAN1 agreements imply PDCCH order as in LTE even though 38.213 does not capture it yet. To be confirmed by RAN2.

RRC configures the following parameters for the Random Access procedure:

-	prach-ConfigIndex: the available set of PRACH resources for the transmission of the Random Access Preamble;

-	ra-PreambleInitialReceivedTargetPower: initial preamble power;

-	ssb-Threshold: an RSRP threshold for the selection of the SS block and corresponding PRACH resource;

-	ra-PreamblePowerRampingStep: the power-ramping factor;

-	ra-PreambleIndex: Random Access Preamble;

-	ra-PreambleTx-Max: the maximum number of preamble transmission;

-	the groups of Random Access Preambles (i.e. Random Access Preambles group A and Random Access Preambles group B) and the set of available Random Access Preambles in each group (SpCell only);

-	if Random Access Preambles group B exists:

-	ra-Msg3SizeGroupA: the threshold to determine the groups of Random Access Preambles;

-	the set of Random Access Preambles for SI request and corresponding PRACH resource(s), if any;

-	set of Random Access Preambles for beam failure recovery request and corresponding PRACH resource(s), if any;

-	ra-ResponseWindow: the time window to monitor RA response(s);

-	ra-ContentionResolutionTimer: the Contention Resolution Timer (SpCell only).

In addition, the following information for related Serving Cell is assumed to be available for UEs:

-	if Random Access Preambles group B exists:

-	PCMAX,c: the configured UE transmitted power of the Serving Cell performing the Random Access Procedure.

The following UE variables are used for the Random Access procedure:

-	PREAMBLE_INDEX;

-	PREAMBLE_TRANSMISSION_COUNTER;

-	PREAMBLE_POWER_RAMPING_COUNTER;

-	PREAMBLE_RECEIVED_TARGET_POWER;

-	PREAMBLE_BACKOFF;

-	TEMPORARY_C-RNTI.

The Random Access procedure is initiated as follows:

The MAC entity shall:

1>	flush the Msg3 buffer;

1> set the PREAMBLE_TRANSMISSION_COUNTER to 1;

1>	set the PREAMBLE_POWER_RAMPING_COUNTER to 1;

1>	set the PREAMBLE_BACKOFF to 0 ms;

1>	perform the Random Access Resource selection procedure (see subclause 5.1.2).

Editor's note: beamforming aspect may impact to RA procedure across subclause 5.1, but RAN1 inputs would be further required.

Editor's note: SUL may impact to RA procedure depending on the RAN1/2 discussion.

Editor's note: Differentiation of backoff parameter and/or power ramping will be supported, but details to be discussed next year.

### 5.1.2	Random Access Resource selection

The MAC entity shall:

1> if the Random Access procedure was initiated by a beam failure indication from lower layer and contention free PRACH resources associated with any of the SS blocks or CSI-RSs have been explicitly provided by RRC and at least one of the SS block with metric X above FFS-threshold-SSB amongst the associated SS blocks or the CSI-RSs with metric X2 above FFS-threshold-CSI-RS amongst the associated CSI-RSs is available:

2> select an SS block with SS- metric X above FFS-threshold-SSB amongst the associated SS blocks or a CSI-RS with metric X above FFS-threshold-CSI-RS amongst the associated CSI-RSs;

2> set the PREAMBLE_INDEX to a ra-PreambleIndex corresponding to the selected SS block or CSI-RS from the set of Random Access Preambles for beam failure recovery request;

Editor's note: metric X, metric X2, FFS-threshold-SSB, and FFS-threshold-CSI-RS should be agreed by RAN1.

Editor's note: Above assumes gNB can configure candidate beams for recovery with a mixture of SSB based and CSI-RS resource based beams.

1>	else if the ra-PreambleIndex has been explicitly provided by either PDCCH or RRC, and it is not 0b000000:

2>	set the PREAMBLE_INDEX to the signalled ra-PreambleIndex;

1>	else:

2>	select a SS block with SS-RSRP above ssb-Threshold;

Editor's note: RAN2 can discuss further whether ssb-Threshold is mandatory or not. Also, the case when multiple SS blocks are above the ssb-Threshold can be discussed later.

2>	if Msg3 has not yet been transmitted:

3>	if Random Access Preambles group B exists; and

3>	if the potential Msg3 size (UL data available for transmission plus MAC header and, where required, MAC CEs) is greater than ra-Msg3SizeGroupA and the pathloss is less than PCMAX,c (of the Serving Cell performing the Random Access Procedure) – ra-PreambleInitialReceivedTargetPower:

Editor's note: from the LS R1-1716932, Editor understands that only ra-PreambleInitialReceivedTargetPower is discussed in RAN1, so deltaPreambleMsg3, messagePowerOffsetGroupB in LTE are not captured. RAN2 confirmation requires.

4>	select the Random Access Preambles group B;

3> else:

4>	select the Random Access Preambles group A.

2> else (i.e. Msg3 is being retransmitted):

3>	select the same group of Random Access Preambles as was used for the preamble transmission attempt corresponding to the first transmission of Msg3.

2>	if the association between Random Access Preambles and SS blocks is configured:

3>	select a ra-PreambleIndex randomly with equal probability from the random access preambles associated with the selected SS block and the selected group;

2>	else:

3>	select a ra-PreambleIndex randomly with equal probability from the random access preambles within the selected group;

2>	set the PREAMBLE_INDEX to the selected ra-PreambleIndex;

Editor's note: the updates above are based on the following RAN1 agreements and the LS R1-1716932:
-	For contention-based random access, an association between an SS block in the SS burst set and a subset of RACH resources and/or preamble indices is configured by a set of parameters in RMSI.
-	For initial access, threshold for SS block selection for RACH resource association is configurable by network, where the threshold is based on RSRP.

1>	if an SS blocks is selected above and an association between PRACH occasions and SS blocks is configured:

2>	determine the next available PRACH occasion from the PRACH occasions corresponding to the selected SS block;

1>	else if a CSI-RS is selected above and an association between PRACH occasions and CSI-RSs is configured:

2>	determine the next available PRACH occasion from the PRACH occasions corresponding to the selected CSI-RS;

1>	else:

2>	determine the next available PRACH occasion;

Editor's note: the order of procedure (e.g. selection of SS block) can be re-arranged later.

1> perform the Random Access Preamble transmission procedure (see subclause 5.1.3).

### 5.1.3	Random Access Preamble transmission

The MAC entity shall, for each preamble:

1>	set PREAMBLE_RECEIVED_TARGET_POWER to ra-PreambleInitialReceivedTargetPower + DELTA_PREAMBLE + (PREAMBLE_POWER_RAMPING_COUNTER – 1) * powerRampingStep;

Editor's note: still the above equation needs to be confirmed by having RAN1 input.

Editor's note: the text should be clarified further regarding multiple preamble transmission case.

1>	except for contention free preamble for beam failure recovery request, compute the RA-RNTI associated with the PRACH in which the Random Access Preamble is transmitted as follows [TBD]:

Editor's note: RA-RNTI calculation equation will be placed here. At least time and frequency is used in the RA-RNTI formula.

1>	instruct the physical layer to transmit the preamble using the selected PRACH, corresponding RA-RNTI (if available), PREAMBLE_INDEX and PREAMBLE_RECEIVED_TARGET_POWER.

### 5.1.4	Random Access Response reception

Once the Random Access Preamble is transmitted, the MAC entity shall:

1>	if 'multiple preamble transmission' has been signalled:

Editor's note: RAN1 has not concluded whether to support multiple Msg1 transmissions so the relevant text can be removed after having RAN1 input. Also how to signal 'multiple preamble transmission' is FFS if supported, and the text can be improved after having RAN1 input.

2>	start the ra-ResponseWindow at the NR-UNIT that contains the end of the first preamble transmission plus [TBD] NR-UNITs;

2>	monitor the PDCCH of the SpCell for Random Access Response(s) identified by the RA-RNTI(s) while ra-ResponseWindow is running;

1>	else if Random Access Preamble for beam failure recovery request was selected by the MAC entity:

2>	start the ResponseWindowSize-BFR at the NR-UNIT that contains the end of the preamble transmission plus [TBD] NR-UNITs;

2>	monitor the PDCCH of the SpCell for downlink assignment(s) or uplink grant(s) identified by the C-RNTI while ResponseWindowSize-BFR is running;

1> else:

2>	start the ra-ResponseWindow at the NR-UNIT that contains the end of the preamble transmission plus [TBD] NR-UNITs;

Editor's note: the above if-statement can be combined since the difference is marginal.

Editor's note: The value of TBD duration is FFS. It is a fixed duration and should be as low as possible.

Editor's note: The term 'NR-UNIT' is used tentatively, and should be replaced later with e.g. subframe/TTI/slot after having RAN1 input.

2>	monitor the PDCCH of the SpCell for Random Access Response(s) identified by the RA-RNTI while ra-ResponseWindow is running;

Editor's note: handling of measurement gap may need to be specified to the above sentence (as in LTE).

1>	if a downlink assignment or uplink grant for this NR-UNIT has been received on the PDCCH addressed for the C-RNTI; and

1>	if the Random Access Preamble for beam failure recovery request was selected by the MAC entity:

2>	consider the Random Access procedure successfully completed.

1>	else if a downlink assignment for this NR-UNIT has been received on the PDCCH for the RA-RNTI and the received TB is successfully decoded:

2>	if the Random Access Response contains a Backoff Indicator subheader:

3>	set the PREAMBLE_BACKOFF to value of the BI field of the Backoff Indicator subheader using Table 7.2-1.

2>	else:

3>	set the PREAMBLE_BACKOFF to 0 ms.

2>	if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted PREAMBLE_INDEX (see subclause 5.1.3):

3>	consider this Random Access Response reception successful;

2>	if the Random Access Response reception is considered successful:

3>	if the Random Access Response includes RAPID only:

4>	consider this Random Access procedure successfully completed;

4>	indicate the reception of an acknowledgement for the SI request to upper layers;

3>	else:

4>	if 'multiple preamble transmission' has been signalled:

5>	stop transmitting remaining preambles, if any;

Editor's note: RAN1 has not concluded whether to support multiple Msg1 transmissions so the relevant text can be removed after having RAN1 input. Also how to signal 'multiple preamble transmission' is FFS if supported, and the text can be improved after having RAN1 input.

4>	apply the following actions for the serving cell where the Random Access Preamble was transmitted:

5>	process the received Timing Advance Command (see subclause 5.2);

5>	indicate the ra-PreambleInitialReceivedTargetPower and the amount of power ramping applied to the latest preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) * powerRampingStep);

5>	process the received UL grant value and indicate it to the lower layers;

4>	if the Random Access Preamble was not selected by the MAC entity among the common PRACH preambles:

5>	consider the Random Access procedure successfully completed.

4>	else:

5>	set the TEMPORARY_C-RNTI to the value received in the Random Access Response;

5>	if this is the first successfully received Random Access Response within this Random Access procedure:

6>	if the transmission is not being made for the CCCH logical channel:

7>	indicate to the Multiplexing and assembly entity to include a C-RNTI MAC CE in the subsequent uplink transmission;

6>	obtain the MAC PDU to transmit from the Multiplexing and assembly entity and store it in the Msg3 buffer.

1>	if ra-ResponseWindow expires and

if the Random Access Response containing Random Access Preamble identifiers that matches the transmitted PREAMBLE_INDEX has not been received; or

1>	if ResponseWindowSize-BFR expires and if no downlink assignment or uplink grant on the PDCCH addressed for the C-RNTI has been received:

2>	consider the Random Access Response reception not successful;

2>	increment PREAMBLE_TRANSMISSION_COUNTER by 1;

Editor's note: Need to have input from RAN1 on power ramping suspension (in LTE), and can be added later.

2>	if the notification of suspending power ramping counter has not been received from lower layers, and SS block selected for PRACH retransmission is not changed:

3>	increment PREAMBLE_POWER_RAMPING_COUNTER by 1;

Editor's note: The location of 'increment PREAMBLE_POWER_RAMPING_COUNTER by 1') can be discussed further.

2>	if PREAMBLE_TRANSMISSION_COUNTER = ra-PreambleTx-Max + 1:

3>	if the Random Access Preamble is transmitted on the SpCell:

4>	indicate a Random Access problem to upper layers;

3>	else if the Random Access Preamble is transmitted on a SCell:

4>	consider the Random Access procedure unsuccessfully completed;

2>	if in this Random Access procedure, the Random Access Preamble was selected by MAC among the common PRACH preambles:

3>	select a random backoff time according to a uniform distribution between 0 and the PREAMBLE_BACKOFF;

3>	delay the subsequent Random Access Preamble transmission by the backoff time;

2>	perform the Random Access Resource selection procedure (see subclause 5.1.2).

The MAC entity may stop ra-ResponseWindow (and hence monitoring for Random Access Response(s)) after successful reception of a Random Access Response containing Random Access Preamble identifiers that matches the transmitted PREAMBLE_INDEX.

HARQ operation is not applicable to the Random Access Response transmission.

*** End of change ***

*** Start of change ***

## 5.X	Beam Failure Recovery Request procedure

The beam failure recovery request procedure is used for indicating to the serving gNB of a new SSB or CSI-RS when beam failure is detected on the serving SSB(s)/CSI-RS(s). Beam failure is detected by the lower layers and indicated to the MAC entity.

The MAC entity shall:

1>	if beam failure indication has been received from lower layers:

2>	start beamFailureRecoveryTimer;

2>	initiate a Random Access procedure (see subclause 5.1) on the SpCell.

1>	if the beamFailureRecoveryTimer expires:

2>	indicate beam failure recovery request failure to upper layers.

1>	if downlink assignment or uplink grant on the PDCCH addressed for the C-RNTI has been received:

2>	stop and reset beamFailureRecoveryTimer;

2>	consider the Beam Failure Recovery Request procedure successfully completed.

Editor's note: Details on the UE behaviour related to beamFailureRecoveryTimer is FFS.

*** End of change ***