# Offline discussion#26:

R2-1714130	Email Disc on [99bis#27][NR] L2/3 capabilities	Intel Corporation	discussion	Rel-15	NR_newRAT-Core

Agreements

1	UE capability parameters will be introduced for features that are optional and for features that are mandatory but required ability to indicate successful IOT (no need to decide whether a feature is optional or mandatory with IoT bit at the time the capability parameter is defined).

=>	Offline discussion to progress the lists. Aim to create a documents with lists that can be easily agreed plus list of FFS points that need more discussion (at future meeting). (Offline discussion #26, Intel)

[Offline discussion outputs]:

# Proposal#1: Mandatory features without the need of any UE capability signaling:

|  | Feature List |
| --- | --- |
| MAC | RA procedure on PCell RA procedure on PSCell for EN-DC* UE initiated RA procedure UE initiated RA for beam recovery NW initiated RA procedure (i.e. based on PDCCH) Support of ssb-Threshold and/or association between preambles/PRACH occasion and SS block Preamble grouping UL single TA maintenance UL multiple TA maintenance for EN-DC* HARQ operation for DL and UL LCH Prioritization Prioritized bit rate Multiplexing SR with single SR configuration BSR PHR 8bits L field 16bits L field *Note1: If EN-DC is supported |
| RLC | RLC TM RLC AM (with 18bits SN)* SDU discard Note1: No separate feature is considered on t-PollRetransmit, t-Reassembly and t-StatusProhibit. |
| PDCP | (de)Ciphering on DRB/SRB Integrity protection on SRB Timer based SDU discard Re-ordering and in-order delivery Status reporting Duplicate discarding 18bits SN |
| EN-DC Procedure | MCG DRB (with LTE PDCP) Joint processing on the combined RRC messages SN addition, modification and release via RRC connection reconfiguration Failure handling (including both MN and SN) MCG DRB (with NR PDCP) SCG DRB (with NR PDCP) |

# Proposal#2: Features with the need of a UE capability signaling (FFS whether to be mandatory or optional):

|  | Feature List |
| --- | --- |
| MAC | LCP restriction skipUplinkTxDynamic logicalChannelSR-ProhibitTimer Long DRX cycle Short DRX cycle SR with multiple SR configuration (number of supported SR configuration) Configured grant type 2 Number of configured grant configurations per cell group Multiple UL TA maintenance for NR CA |
| RLC | AM with 12bits SN UM with 6bits SN UM with 12bits SN |
| PDCP | ROHC profiles Max number of ROHC context sessions UL only ROHC drb-ContinueROHC Out-of-order delivery 12bits SN VoLTE over NR PDCP |
| EN-DC Procedure | SRB3 Split SRB 1/2 with UL TX on MCG or SCG Split DRB (with NR PDCP) with UL TX on MCG or SCG Split DRB (with NR PDCP) with UL TX on both MCG and SCG NR measurements and reporting (at least periodical reporting) in LTE connected mode Measurement reporting event B1 for NR in LTE connected NR intra-F and inter-F measurements and reporting (at least periodical reporting) NR measurement reporting for event A series SSTD measurements for EN-DC Direct SN addition and SCG DRB setup at first RRC reconfiguration after RRC connection setup Non-Default Bearer on SCG |

Proposal#3: RAN2 is asked to confirm after March (i.e. ASN.1 freeze), there is no mandatory feature w/o IOT bit in Rel-15, unless it is relevant to the idle mode.

# Proposal#4: Features needed further discussion:

|  | Feature List |
| --- | --- |
| EN-DC Procedure | Number of supported MCG, SCG, or split bearers |

# Proposal#5: Features needed RAN1/4 inputs or RAN2 decision:

|  | Feature List |
| --- | --- |
| MAC | Multiple Msg1 transmissions in contention-free RA (Need of RAN1 input) Number of parallel HARQ processes per HARQ entity (Need of RAN1 input) TTI bundling / Slot aggregation (Need of RAN1 input) Configured grant type 1 (Need of RAN1 input) |
| EN-DC Procedure | Max number of carriers across LTE and NR to be measured (Need of RAN4 input) NR SS based measurement and configuration and measurements (Need of RAN1 input) NR CSI-RS based measurement configuration and measurements (Need of RAN1 input) Independent measurement gap (Expect RAN2 decision at RAN2#100) Additional measurement gap (Expect RAN2 decision at RAN2#100) |

# Proposal#6: Features not required for Dec. 2017 version:

|  | Feature List |
| --- | --- |
| MAC | NR DC and NR-DC related capabilities |
| SDAP | SDAP related capabilities |
| PDCP | Integrity protection on DRB |
| EN-DC Procedure | Split SRB 1/2 with UL TX on both MCG and SCG Split DRB (with NR PDCP) with UL duplication |

Proposal#7: MAC, RLC capabilities are included in NR UE capability.

Proposal#8: PDCP capabilities are included in both LTE UE capability and NR UE capability.

Proposal#9: EN-DC capabilities are included in common MR-DC container (FFS on capabilities for NR measurements while the UE is in LTE connected state)