3GPP TSG-RAN WG2 #100	Tdoc R2-1712957

Reno, Nevada, USA, 27th November – 1st December 2017

Agenda Item:	9.19

Source:	Ericsson

Title:	Number of DRBs in E-UTRA

Document for:	Discussion, Decision

# Introduction

This contribution discusses about functional and specification impact caused by the increase of number of DRBs.

# Discussion

In the last RAN2#99bis meeting, the number of DRBs was discussed. The discussion concluded that the current specification is not limiting the number of UM and AM DRBs to 3 and 5, but the limit is introduced by the FGI capability (Bits 20 and 7). The FGI field is independent topic and it is discussed in the contribution [1]. There are three main options to choose when considering the increase of number of DRBs from the current 8 DRBs:

There are four main options on increasing the number of DRBs:

- Option 1: Number of DRBs is increased to 11.

- Option 2: Number of DRBs is increased to 15.

- Option 3: Number of DRBs is increased to more than 15.

- Option 4: Number of DRBs is not increased in Rel-15.

The appendix provides a comparison of DRB options with LCID space. The following subsections will analyse the impact of increasing the number of DRBs to LCID space as well as impact on current specifications.

## DRB range impact on LCID space

The size of the LCID space is coupled with the decision of the number of DRBs. The LCID field in the MAC header is 5 bits and hence can support 32 values. LCIDs are used to identify which logical channel and MAC control element are included in the MAC PDU. Currently there are 10 LCIDs marked as “Reserved” for DL and 7 for UL. Increasing the number of LCIDs to 11 (Option 1) would leave 4 “Reserved” UL LCIDs. Furthermore, increasing the number DRBs to 15 (Option 2) would result that all the remaining LCIDs in the UL would be consumed. Alternatively, if the number of DRBs is increased to beyond 15, LCID space must be increased (Option 3), which anyway will be needed at some point in time consider additions of new MAC CEs.

Based on the number of DRBs, following can be observed in LCID space.

DRB option has impact on LCID space

  - If number of DRBs is increased to 11, there are 4 “Reserved” LCIDs remaining.

  - If number of DRBs is increased to 15, there are 0 “Reserved” LCIDs remaining.

  - If number of DRBs is increased to more than 15, LCID space needs extension.

Historically, each release has been consuming LCIDs, and the rel-15 is not expected to be exception to this rule. When considering the increase of the number of DRBs, Reserved LCIDs should be left for future extensions and critical fixes. For instance, there should be reserved values left for MAC CEs. Following table summarizes the changes in LCID space reserved values.

Table 1 – Remaining LCIDs in MAC per release.

| Release | Remaining LCIDs |  |
| --- | --- | --- |
|  | DL | UL |
| Rel-11 | 16 | 14 |
| Rel-12 | 15 (-1) | 10 (-4) |
| Rel-13 | 13 (-2) | 10 (0) |
| Rel-14 | 10 (-3) | 7 (-3) |

Historically, each release has consumed multiple LCID fields (either UL or DL) and Rel-15 is not expected to be exception.

RAN2#97bis discussed about the LCID space and concluded that the issue may be discussed as it reappears. As this issue has reappeared already after a few months since the last discussion, there is no reason to postpone the decision to extend the LCID space anymore. The issue of LCID space running out should be resolved already for release-15.

There is no reason to postpone LCID space extension to the future release if DRB range is extended to support more than 8 DRBs.

If the number of DRBs is increased, LCID space is extended.

Contribution [2] is discussing about solutions to increase the number of supported LCID fields.

## Specification impact on increase of number of DRBs

If the number of DRBs is increased, it will have impact on at least to the following specifications.

| TS 36.321 MAC protocol specification | - Change in LCID space and formats |
| --- | --- |
| TS 36.331 RRC Protocol specification | - DRB range update - Capability bits |
| TS 36.306 UE radio access capabilities | - Capability to support extended DRB range |

RAN2 is kindly requested to discuss about increasing the number of DRBs with the following options.

- Option 1: Number of DRBs is increased to 11

- Option 2: Number of DRBs is increased to 15

- Option 3: Number of DRBs is increased to more than 15

- Option 4: Number of DRBs is not increased in Rel-15

# Conclusion

In section 2 we made the following observations:

Observation 1	There are four main options on increasing the number of DRBs:

Option 1: Number of DRBs is increased to 11.

Option 2: Number of DRBs is increased to 15.

Option 3: Number of DRBs is increased to more than 15.

Option 4: Number of DRBs is not increased in Rel-15.

Observation 2	DRB option has impact on LCID space

a.	If number of DRBs is increased to 11, there are 4 “Reserved” LCIDs remaining.

b.	If number of DRBs is increased to 15, there are 0 “Reserved” LCIDs remaining.

c.	If number of DRBs is increased to more than 15, LCID space needs extension.

Observation 3	Historically, each release has consumed multiple LCID fields (either UL or DL) and Rel-15 is not expected to be exception.

Observation 4	There is no reason to postpone LCID space extension to the future release if DRB range is extended to support more than 8 DRBs.

Based on the discussion in section 2, we propose following

Proposal 1	If the number of DRBs is increased, LCID space is extended.

Proposal 2	RAN2 is kindly requested to discuss about increasing the number of DRBs with the following options.

Option 1: Number of DRBs is increased to 11

Option 2: Number of DRBs is increased to 15

Option 3: Number of DRBs is increased to more than 15

Option 4: Number of DRBs is not increased in Rel-15

# References

R2-1712951, FGI20 limitation for DRBs, Ericsson, RAN2#100

R2-1712956, LCID space extension, Ericsson RAN2#100

# Appendix

Table 1: Comparison of DL LCID space impact on Number of DRBs selected

Table 2: Comparison of UL LCID space impact on Number of DRBs selected