3GPP TSG-RAN WG2 Meeting #100	R2-1713324

Reno, United States, 27 Nov - 1 Dec 2017

Agenda item:	9.4.4.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on New Measurement Events for Drones

Document for:	Discussion

Introduction

In RAN2#99bis meeting, the summary of email discussion for interference detection [1] has been presented, and some potential solutions have been concluded. One option is “Identify (airborne) UE causing interference – UE based: MR reporting enhancement”. Some companies proposed to introduce new events or new values to assist the NW. In this paper we further discuss the necessity of the proposed new measurement events.

Discussion

According to [1] four solutions are classified into measurement reporting enhancement with new events, and they are represented as follows:

S2: NOKIA

A new signaling events relying on RSRP and RSRQ measurements (see R2-170866 for details)

S7: INTEL

One potential solution for interference detection for both UL and DL are UE based detection. We can reuse existing measurement reporting mechanism by adding a new event. New event can sum the measurement of the neighboring cells to trigger the UE sends measurement report to the network. In this case, once the measurement report is sent, the network can send interference control indication to the UE.

S18: NTT

The proposal for the new event is to define an event that would only triggered a MR when the triggered cell satisfying the event is equal to N (N>1).

S26: E\\

It would be beneficial that measurement reports are triggered when the signal strength conditions resemble the airborne UE situation. For example, an airborne UE would likely see more cells with similar signal strength and an airborne UE would likely see more far away cells than a ground UE due to the difference in propagation conditions. Thus, for example, UE could trigger measurement report of e.g. X number of cells have signal strength above -90dBm and below -70dBm. Another possibility is to trigger report when UE sees certain cells, e.g. certain configured far away cells.

S2 Analysis

S2 proposes a measurement event relying on RSRP and RSRQ, and it is based on the observations that the downlink interference increases when a UE moves up in height and the RSSI for terrestrial users depends a lot on the location in the cell. In [2] measurement results are derived in real commercial LTE networks for two different locations and two different LTE network operators, then points are plotted by ΔRSRP and RSSI. And based on these results [2] makes the assumption that the points belonging to airborne UEs can be separated from the points belonging to terrestrial UEs by a separation line. So airborne UEs can be separated from terrestrial UEs by defining a separation line based on ΔRSRP and RSSI. Accordingly a new measurement event can be introduced when a UE “cross” the line, e.g. when a UE is assumed terrestrial and then its RSRP is lower than the separation line.

We notice that the measurement are performed in rural locations in which less buildings and obstacles exist, the air condition seems much “clean”, so the separation line could be obvious. And if the separation line can be also clear in UMa scenario still need to be proven by real field test, so we could not determine if S2 is effective in all scenarios.

Observation 1: the effectiveness of S2 still need to be proven, e.g. for UMa scenario.

Secondly for optimal performance the parameters of separation line need to be set differently for different area’s/cells, and a training process seems necessary and unavoidable. So extra field measurement has to be performed to collect training set, even for each cell, after the parameters are estimated the separation line can be drawn.

Observation 2: extra field measurement has to be performed for S2 to draw separation line, even for each cell.

S26 Analysis

Similar to S2, in [3] S26 is proposed to identify airborne UE by signal strength conditions. For example, UE could trigger measurement report of e.g. X number of cells have signal strength above -90dBm and below -70dBm. Another possibility is to trigger report when UE sees certain cells, e.g. certain configured far away cells.

Considering X number of cells have signal strength above -90dBm and below -70dBm, it seems that the current measurement report already can provide this kind of information, as the measurement results of neighbor cells are available in measurement report and the neighbor cells are sorted by signal strength quantity. And the trigger condition can be A4 event with a proper threshold.

Observation 3: the current measurement report already include the information which are highlighted by S26.

And for the possibility that UE sees certain far away cells, it seems unnecessary to design a new event because the neighbor cell can be listed in measurement report if its signal strength is high enough. And the “far away cell list” may change according to the UE’s location, so it cost extra signaling to indicate the modification of “far away cell list”. On the other hand, all UEs need to receive this kind of information for airborne UE identification, it cost extra power consumption for the terrestrial UEs.

Observation 4: it cost extra signalling overhead to indicate the modification of “far away cell list” for S26, and it cost extra power consumption for the terrestrial UEs to receive the “far away cell list”.

S7 Analysis

In [4] a UL interference detection method is proposed briefly to introduce a new event which sums the measurement of the neighbor cells to trigger the UE to send measurement report to the network. But what the detail of the event is and how to “sum” are still FFS. Ericsson provide some candidate options in [1], such as summing RSRP or counting number of cells. We think eNB can derive RSSI by processing RSRP and RSRQ, the RSSI value is nearly equal to summing RSRP of all detected cells. For counting cells the current measurement report also include the neighbour cell information, so the eNB can know the number of neighbour cells.

Observation 5: the detail of event proposed by S7 is still FFS and the information provided by candidate methods for summing measurement has already been available in measurement report or can be deduced by eNB.

S18 Analysis

Using the existing measurement reporting mechanism, the number of measurement reports may be high in the case of UAV UE. In [1] DCM propose to define a new event which only triggered a MR when the triggered cell satisfying the event is equal to N (N>1). In fact it is an enhancement for trigger condition, not a new event. We think this modification is necessary, and it can apply for all available measurement events, instead of introducing a new dedicated event.

Observation 6: it is necessary to introduce a minimum number of cells to trigger an event reporting to mitigate frequent reporting.

Observation 7: the enhancement for trigger condition can be applied for all available measurement events.

Based on the analysis above the introduction of new measurement events seems too much early because some details are still FFs, and the information provided by new events has already been available in current measurement report or can be deduced by eNB, so it is also unnecessary to introduce a new event. But no matter if new event will be introduced, the issue of frequent reporting can be mitigated by setting a minimum number of cells to trigger an event reporting. So we have the following proposal:

Proposal: introduce a minimum number of cells to trigger an event reporting to mitigate frequent reporting.

Conclusion

In this paper we further discuss the necessity of the new measurement events, and we have the following observations:

Observation 1: the effectiveness of S2 still need to be proven, e.g. for UMa scenario.

Observation 2: extra field measurement has to be performed for S2 to draw separation line, even for each cell.

Observation 3: the current measurement report already include the information which are highlighted by S26.

Observation 4: it cost extra signalling overhead to indicate the modification of “far away cell list” for S26, and it cost extra power consumption for the terrestrial UEs to receive the “far away cell list”.

Observation 5: the detail of event proposed by S7 is still FFS and the information provided by candidate methods for summing measurement has already been available in measurement report or can be deduced by eNB.

Observation 6: it is necessary to introduce a minimum number of cells to trigger an event reporting to mitigate frequent reporting.

Observation 7: the enhancement for trigger condition can be applied for all available measurement events.

So we propose:

Proposal: introduce a minimum number of cells to trigger an event reporting to mitigate frequent reporting.

Reference

[1]       R2-1711738 Summary of email discussion [99#37][LTE/UAV] DL and UL Interference detection, NTT DOCOMO INC.

[2]      R2-1708669 Air-borne UE identification mechanism, Nokia, Nokia Shanghai Bell

[3]      R2-1708664 Identify certification for drones, Ericsson

[4]      R2-1709038 UL interference from an aerial vehicle, Intel