3GPP TSG-RAN WG2 Meeting #101                                                                 R2-1801921

Athens, Greece, 26th February - 2nd March 2018

Source:	CATT

Title:	Consideration on PLMNs Connected to both 5GC and EPC

Agenda Item:	9.7.2

Document for:	Discussion and Decision

Introduction

In RAN2 #100 meeting, the following agreements on CN type selection were made:

Agreements

1	Introduce a new 5GC PLMN list containing PLMNs that can connect to 5GC (coding details to be worked offline to avoid any need for repeating a PLMN ID if already present in the legacy PLMN list)

2	"cellReservedForOperatorUse” is introduced for PLMNs which can connect to 5GC only.

3	5GC specific “cellReservedForOperatorUse” is introduced for PLMNs which can connect both EPC and 5GC.

4	UE AS indicates available CN types to upper layers for CN type selection.

About the PLMNs that can connect to both 5GC and EPC, how to introduce them in SIB 1 has not been discussed. The email discussion of 100#24 for TS 36.331 to capture the eLTE agreements has concluded to define a new 5GC PLMN list. Taking the email discussion of 100#24 as baseline, this contribution will discuss the approach on how to provide the PLMNs that connected to both 5GC and EPC by SIB1.

Discussion

PLMNs connected to both 5GC and EPC

As a separate 5GC PLMN list has been introduced into SIB1, the PLMNs connected to both 5GC and EPC should be present in both the legacy PLMN list and 5GC PLMN list is a natural and straight solution. But based on the agreement made in #100 meeting, repeating PLMN ID in legacy PLMN list and new 5GC PLMN list should be avoided. Hence the same PLMN ID present in two PLMN lists should be taken as the low-priority candidate option. Other options should be considered to reduce overhead.

- TAC and Cell ID

If a PLMN ID can’t be present in the legacy PLMN list and the 5GC PLMN list, first we should discuss what information in legacy PLMN list can be shared by this PLMN connected to 5GC. The PLMN information structure in legacy PLMN list is as following:

cellAccessRelatedInfo				SEQUENCE {

plmn-IdentityList					PLMN-IdentityList,

trackingAreaCode					TrackingAreaCode,

cellIdentity						CellIdentity,

cellBarred							ENUMERATED {barred, notBarred},

intraFreqReselection				ENUMERATED {allowed, notAllowed},

csg-Indication						BOOLEAN,

csg-Identity						CSG-Identity			OPTIONAL	-- Need OR

},

CellAccessRelatedInfo-r14 ::=	SEQUENCE {

plmn-IdentityList-r14				PLMN-IdentityList,

trackingAreaCode-r14				TrackingAreaCode,

cellIdentity-r14					CellIdentity

}

From the structure we can get that each plmn-IdentityList connected to EPC is corresponding to one trackingAreaCode  and cellIdentity. As for the new 5GC PLMN list also reuse this structure.

SystemInformationBlockType1-v15xy-IEs ::=	SEQUENCE {

cellAccessRelatedInfoFor-5GC-r15				SEQUENCE {

cellBarred-5GC-r15							ENUMERATED {barred, notBarred},

-- FFS on maximum number of PLMN lists for 5GC;  6 including EPC or indenpended

cellAccessRelatedInfoList-5GC-r15			SEQUENCE (SIZE (1..maxPLMN-1-r14)) OF

CellAccessRelatedInfo-r14

}

As the ASN.1 shows, each plmn-IdentityList in SIB1 should correspond to one trackingAreaCode and cellIdentity no matter they connect to EPC or 5GC.

For the PLMN which can connect to both EPC and 5GC, the values of trackingAreaCode and the cellIdentity of the PLMN can be set to different values when it connects to different CN. Thus the TACs and the cell IDs are not restricted to be the same for the PLMN connects to both EPC and 5GC.

Obversation1: Each PLMN list corresponds to one TAC and cellIdentity no matter they connect to EPC or 5GC. And for the PLMN connected to both 5GC and EPC, the corresponding TACs and cell IDs for different PLMN lists can be set to different values.

Different TAC/Cell ID for different CN

If the PLMN connected to both 5GC and EPC can’t be present in two PLMN lists, the trackingAreaCode and cellIdentity should be separately defined for this PLMN connected to 5GC. The existing structure of PLMN-Identity, TrackingAreaCode and CellIdentity are listed as following:

TrackingAreaCode ::=				BIT STRING (SIZE (16))

CellIdentity ::=					BIT STRING (SIZE (28))

PLMN-Identity ::=					SEQUENCE {

mcc									MCC					OPTIONAL,					-- Cond MCC

mnc									MNC

}

MCC ::=								SEQUENCE (SIZE (3)) OF

MCC-MNC-Digit

MNC ::=								SEQUENCE (SIZE (2..3)) OF

MCC-MNC-Digit

MCC-MNC-Digit ::=					INTEGER (0..9)

The size of TrackingAreaCode and cellIdentity are 44 bits, the least size of PLMN-Identity is 8bits, and the largest size of PLMN-Identity is 24bits. The overhead of TAC and cell ID is obvious larger than the PLMN ID. To avoid repeating a PLMN ID presented in legacy PLMN list, the TAC and cell ID should separately defined for this PLMN which may be the same as these of one PLMN in 5GC PLMN list. This causes the TAC and cell ID duplicated in the 5GC PLMN list.

Fig 1 TAC and CELL ID

Here is an example shown in figure 1, it assumes the PLMN-1 can be connect to both 5GC and EPC, then some indicator should be introduced to indicate UE this PLMN can also connect to 5GC. If for PLMN-1 connects to 5GC, the TAC and cell ID is same with PLMN-3 and PLMN-4 which is TAC-2/CELL ID-2, the overhead to indicate UE that PLMN-1 can connect to 5GC is at least 45bits as the green part shown. But if the PLMN can be present in two PLMN list as the red part in figure 1 illustrated, the overhead to indicate UE that PLMN-1 can connect to 5GC is at most 24bits. So comparing two options, for PLMNs connected to both 5GC and EPC which the TAC and cell ID are different for different CN type, the PLMN ID present in both legacy PLMN list and 5GC PLMN list is more efficient.

Proposal 1: For PLMNs connected to both 5GC and EPC which the TACs and cell IDs are different for different CN connections, the PLMN ID should be present in both legacy PLMN list and 5GC PLMN list is preferred considering the signaling overhead efficiency.

Same TAC/Cell ID for different CN

For PLMNs connected to both 5GC and EPC which the TACs and cell IDs are same for different CN types, i.e. the same TACs and cell IDs can also be avoided repeating, some enhancements can be considered to reduce signalling overhead.

Obversation2: When the TACs and the cell IDs of one PLMN connected to both 5GC and EPC are configured the same values for different CN connections, all the repetition of PLMN ID, TAC and the cell ID can be avoided.

In order to indicate UE which PLMNs in the legacy PLMN list are also connected to 5GC, a bitmap indication can be introduced. Each bit of the bitmap is corresponding to one PLMN in the legacy PLMN list, the length of the bitmap should cover all the legacy PLMNs, the LSB of the bitmap corresponds to the 1st PLMN in the 1st plmn-IdentityList included in SIB1. Value 1 indicates the corresponding PLMN can connect to 5GC, and value 0 indicates the corresponding PLMN can only connect to EPC. Hence UE can get the PLMN that connect to both 5GC and EPC through reading the bitmap. This bitmap should only be visible for 5GC capable UEs, the legacy UE will ignore this bitmap.

Proposal 2: A bitmap should be introduced to indicate UE which PLMN in the legacy PLMN list can also connect to 5GC. And this bitmap should be visible only for 5GC capable UEs.

Corresponding TP is provided in Annex for reference.

Due to the cellReservedForOperatorUse defined for each PLMN in legacy PLMN list only used as barring indicator for the PLMN connected to EPC, extend the information of the cellReservedForOperatorUse on the legacy PLMN list to cover 5GC barring indicator is not backward compatibility. Moreover 5GC barring indicator need to be introduced only for the PLMNs connected to both 5GC and EPC, i.e. the PLMNs indicated by the bitmap.

A separate barring indication list should be included to indicate the barring information for 5GC for each PLMN indicated by the bitmap. Similar cellReservedForOperatorUse can be reused, which can be named as cellReservedForOperatorUse-5GC. Each cellReservedForOperatorUse-5GC in the list is corresponding to one PLMN indicated by the bitmap. When the IE cellBarred-5GC-r15 is set to “notBarred”, and if the cellReservedForOperatorUse-5GC is set to “reserved”, it indicates that the cell is "reserved for operator use" for this PLMN connected to 5GC; and if the cellReservedForOperatorUse-5GC is set to “notReserved”, UE can treat this cell as candidate cell during cell (re)-selection for this PLMN connect to 5GC.

Proposal 3: A separate cellReservedForOperatorUse-5GC list should be introduced to indicate the 5GC barring indication for each PLMN indicated in the bitmap. The cellReservedForOperatorUse in the legacy PLMN list only used to indicate the cell barring indication that connected to EPC.

Impact on TS 36.331

UE reports the selected PLMN in RRCConnectionSetupComplete by selectedPLMN-Identity and the connectTo5GC-r15 indicated the CN type. The field description of selectedPLMN-Identity is as following:

RRCConnectionSetupComplete field descriptions

selectedPLMN-Identity Index of the PLMN selected by the UE from the plmn-IdentityList fields included in SIB1. 1 if the 1st PLMN is selected from the 1st plmn-IdentityList included in SIB1, 2 if the 2nd PLMN is selected from the same plmn-IdentityList, or when no more PLMN are present within the same plmn-IdentityList, then the PLMN listed 1st in the subsequent plmn-IdentityList within the same SIB1 and so on.

The selectedPLMN-Identity is the index of the PLMN selected by the UE from the plmn-IdentityList, As the new 5GC PLMN list introduced, some clarification needs to be made that whether the index of the PLMN for 5GC starts from 1 i.e. from the new 5GC PLMN list, or it is continued of the index of the PLMN for EPC, i.e. from the first PLMN list in SIB1.

How does the PLMN connected to both 5GC and EPC present in the SIB1 will impact the report of the selected PLMN. The following cases and potential reporting options are listed in the Table 1:

Table 1: schemes of selected PLMN ID reporting

| PLMN connected to both 5GC and EPC | PLMN number |  | selectedPLMN-Identity(When UE selects a PLMN from the 5GC PLMN list) | Extra bit(s) |
| --- | --- | --- | --- | --- |
| Case 1: extra indication to flag the PLMN in legacy PLMN list connecting 5GC or not, avoid PLMN ID repeating | Total PLMN number is 6 |  | Option 1): The index starts from the first legacy PLMN list in the SIB 1, i.e. index range is the legacy PLMN range + 5GC PLMN range. | no |
|  | Total PLMN number exceeds 6 | 5GC PLMN number doesn’t exceed 6 | Option 2): The index starts from 1 for each PLMN list, one extra bit is used to indicate which PLMN list is referred. | yes |
|  |  | 5GC PLMN number exceeds 6 | Option 3): The index starts from the first legacy PLMN list in the SIB 1, i.e. index range is the legacy PLMN range +5GC PLMN range. Extra bits are needed to provide MSBs of index, and existing bits are used for LSBs of index. | yes |
| Case 2: no extra indication, all present in both legacy and 5GC PLMN list | 5GC PLMN number exceeds 6 |  | Option 4): The index starts from 1, and current indicator is used to indicate which PLMN list is referred. And extra bits are needed to provide MSBs of index corresponding to 5GC PLMN list, and existing bits are used for LSBs of index. | yes |
|  | 5GC PLMN number doesn’t exceeds 6 |  | Option 5): The index starts from 1, and current indicator is used to indicate which PLMN list is referred. | No |

Proposal 4: RAN2 is requested to decide the number of 5GC PLMN and which option in Table 1 is adopted to report selected PLMN ID.

Conclusion

In this contribution, we propose:

Obversation1: Each PLMN list corresponds to one TAC and cellIdentity no matter they connect to EPC or 5GC. And for the PLMN connected to both 5GC and EPC, the corresponding TACs and cell IDs for different PLMN lists can be set to different values.

Obversation2: When the TACs and the cell IDs of one PLMN connected to both 5GC and EPC are configured the same values for different CN connections, all the repetition of PLMN ID, TAC and the cell ID can be avoided.

For PLMN connected to both 5GC and EPC which the TAC and CellIdentity are different for different CN:

Proposal 1: For PLMNs connected to both 5GC and EPC which the TACs and cell IDs are different for different CN connections, the PLMN ID should be present in both legacy PLMN list and 5GC PLMN list is preferred considering the signaling overhead efficiency.

For PLMN connected to both 5GC and EPC which the TAC and CellIdentity are same for different CN:

Proposal 2: A bitmap should be introduced to indicate UE which PLMN in the legacy PLMN list can also connect to 5GC. And this bitmap should be visible only for 5GC capable UEs.

Proposal 3: A separate cellReservedForOperatorUse-5GC list should be introduced to indicate the 5GC barring indication for each PLMN indicated in the bitmap. The cellReservedForOperatorUse in the legacy PLMN list only used to indicate the cell barring indication that connected to EPC.

Proposal 4: RAN2 is requested to decide the number of 5GC PLMN and which option in Table 1 is adopted to report selected PLMN ID.

Reference

[1] RAN2#99, Chairman notes

[2] TS 36.331

- Annex A:  TS 36.331

- –	SystemInformationBlockType1

SystemInformationBlockType1 contains information relevant when evaluating if a UE is allowed to access a cell and defines the scheduling of other system information. SystemInformationBlockType1-BR uses the same structure as SystemInformationBlockType1.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channels: BCCH and BR-BCCH

Direction: E-UTRAN to UE

SystemInformationBlockType1 message

-- ASN1START

SystemInformationBlockType1-BR-r13 ::=	SystemInformationBlockType1

SystemInformationBlockType1 ::=		SEQUENCE {

cellAccessRelatedInfo				SEQUENCE {

plmn-IdentityList					PLMN-IdentityList,

trackingAreaCode					TrackingAreaCode,

cellIdentity						CellIdentity,

cellBarred							ENUMERATED {barred, notBarred},

intraFreqReselection				ENUMERATED {allowed, notAllowed},

csg-Indication						BOOLEAN,

csg-Identity						CSG-Identity			OPTIONAL	-- Need OR

},

cellSelectionInfo					SEQUENCE {

q-RxLevMin							Q-RxLevMin,

q-RxLevMinOffset					INTEGER (1..8)			OPTIONAL	-- Need OP

},

p-Max								P-Max						OPTIONAL,			-- Need OP

freqBandIndicator					FreqBandIndicator,

schedulingInfoList					SchedulingInfoList,

tdd-Config							TDD-Config					OPTIONAL,	-- Cond TDD

si-WindowLength						ENUMERATED {

ms1, ms2, ms5, ms10, ms15, ms20,

ms40},

systemInfoValueTag					INTEGER (0..31),

nonCriticalExtension				SystemInformationBlockType1-v890-IEs	OPTIONAL

}

SystemInformationBlockType1-v890-IEs::=	SEQUENCE {

lateNonCriticalExtension			OCTET STRING (CONTAINING SystemInformationBlockType1-v8h0-IEs)			OPTIONAL,

nonCriticalExtension				SystemInformationBlockType1-v920-IEs	OPTIONAL

}

-- Late non critical extensions

SystemInformationBlockType1-v8h0-IEs ::=	SEQUENCE {

multiBandInfoList					MultiBandInfoList		OPTIONAL,	-- Need OR

nonCriticalExtension				SystemInformationBlockType1-v9e0-IEs	OPTIONAL

}

SystemInformationBlockType1-v9e0-IEs ::= SEQUENCE {

freqBandIndicator-v9e0				FreqBandIndicator-v9e0		OPTIONAL,	-- Cond FBI-max

multiBandInfoList-v9e0				MultiBandInfoList-v9e0		OPTIONAL,	-- Cond mFBI-max

nonCriticalExtension				SystemInformationBlockType1-v10j0-IEs	OPTIONAL

}

SystemInformationBlockType1-v10j0-IEs ::= SEQUENCE {

freqBandInfo-r10					NS-PmaxList-r10				OPTIONAL,	-- Need OR

multiBandInfoList-v10j0				MultiBandInfoList-v10j0		OPTIONAL,	-- Need OR

nonCriticalExtension				SystemInformationBlockType1-v10l0-IEs					OPTIONAL

}

SystemInformationBlockType1-v10l0-IEs ::= SEQUENCE {

freqBandInfo-v10l0					NS-PmaxList-v10l0			OPTIONAL,	-- Need OR

multiBandInfoList-v10l0				MultiBandInfoList-v10l0		OPTIONAL,	-- Need OR

nonCriticalExtension				SEQUENCE {}					OPTIONAL

}

-- Regular non critical extensions

SystemInformationBlockType1-v920-IEs ::=	SEQUENCE {

ims-EmergencySupport-r9				ENUMERATED {true}			OPTIONAL,	-- Need OR

cellSelectionInfo-v920				CellSelectionInfo-v920		OPTIONAL,	-- Cond RSRQ

nonCriticalExtension				SystemInformationBlockType1-v1130-IEs	OPTIONAL

}

SystemInformationBlockType1-v1130-IEs ::=	SEQUENCE {

tdd-Config-v1130				TDD-Config-v1130			OPTIONAL,	-- Cond TDD-OR

cellSelectionInfo-v1130			CellSelectionInfo-v1130		OPTIONAL,	-- Cond WB-RSRQ

nonCriticalExtension			SystemInformationBlockType1-v1250-IEs	OPTIONAL

}

SystemInformationBlockType1-v1250-IEs ::=	SEQUENCE {

cellAccessRelatedInfo-v1250					SEQUENCE {

category0Allowed-r12						ENUMERATED {true}		OPTIONAL	-- Need OP

},

cellSelectionInfo-v1250					CellSelectionInfo-v1250		OPTIONAL,	-- Cond RSRQ2

freqBandIndicatorPriority-r12			ENUMERATED {true}			OPTIONAL,	-- Cond mFBI

nonCriticalExtension			SystemInformationBlockType1-v1310-IEs	OPTIONAL

}

SystemInformationBlockType1-v1310-IEs ::=	SEQUENCE {

hyperSFN-r13								BIT STRING (SIZE (10))		OPTIONAL,	-- Need OR

eDRX-Allowed-r13							ENUMERATED {true}			OPTIONAL,	-- Need OR

cellSelectionInfoCE-r13					CellSelectionInfoCE-r13	OPTIONAL,	-- Need OP

bandwidthReducedAccessRelatedInfo-r13	SEQUENCE {

si-WindowLength-BR-r13					ENUMERATED {

ms20, ms40, ms60, ms80, ms120,

ms160, ms200, spare},

si-RepetitionPattern-r13				ENUMERATED {everyRF, every2ndRF, every4thRF,

every8thRF},

schedulingInfoList-BR-r13				SchedulingInfoList-BR-r13	OPTIONAL,	-- Cond SI-BR

fdd-DownlinkOrTddSubframeBitmapBR-r13	CHOICE {

subframePattern10-r13					BIT STRING (SIZE (10)),

subframePattern40-r13					BIT STRING (SIZE (40))

}																	OPTIONAL, 	-- Need OP

fdd-UplinkSubframeBitmapBR-r13			BIT STRING (SIZE (10)) 		OPTIONAL, 	-- Need OP

startSymbolBR-r13						INTEGER (1..4),

si-HoppingConfigCommon-r13				ENUMERATED {on,off},

si-ValidityTime-r13						ENUMERATED {true}	OPTIONAL,			-- Need OP

systemInfoValueTagList-r13				SystemInfoValueTagList-r13	OPTIONAL	-- Need OR

}																OPTIONAL,	-- Cond BW-reduced

nonCriticalExtension						SystemInformationBlockType1-v1320-IEs	OPTIONAL

}

SystemInformationBlockType1-v1320-IEs ::=	SEQUENCE {

freqHoppingParametersDL-r13				SEQUENCE {

mpdcch-pdsch-HoppingNB-r13				ENUMERATED {nb2, nb4}		OPTIONAL,	-- Need OR

interval-DLHoppingConfigCommonModeA-r13	CHOICE {

interval-FDD-r13					ENUMERATED {int1, int2, int4, int8},

interval-TDD-r13					ENUMERATED {int1, int5, int10, int20}

}																	OPTIONAL,	-- Need OR

interval-DLHoppingConfigCommonModeB-r13	CHOICE {

interval-FDD-r13					ENUMERATED {int2, int4, int8, int16},

interval-TDD-r13					ENUMERATED { int5, int10, int20, int40}

}																	OPTIONAL,	-- Need OR

mpdcch-pdsch-HoppingOffset-r13			INTEGER (1..maxAvailNarrowBands-r13)	OPTIONAL	-- Need OR

}																OPTIONAL,	-- Cond Hopping

nonCriticalExtension						SystemInformationBlockType1-v1350-IEs					OPTIONAL

}

SystemInformationBlockType1-v1350-IEs ::=	SEQUENCE {

cellSelectionInfoCE1-r13				CellSelectionInfoCE1-r13	OPTIONAL,	-- Need OP

nonCriticalExtension					SystemInformationBlockType1-v1360-IEs				OPTIONAL

}

SystemInformationBlockType1-v1360-IEs ::=	SEQUENCE {

cellSelectionInfoCE1-v1360				CellSelectionInfoCE1-v1360	OPTIONAL, 	-- Cond QrxlevminCE1

nonCriticalExtension						SystemInformationBlockType1-v1430-IEs		OPTIONAL

}

SystemInformationBlockType1-v1430-IEs ::=	SEQUENCE {

eCallOverIMS-Support-r14				ENUMERATED {true}			OPTIONAL,	-- Need OR

tdd-Config-v1430						TDD-Config-v1430			OPTIONAL,	-- Cond TDD-OR

cellAccessRelatedInfoList-r14			SEQUENCE (SIZE (1..maxPLMN-1-r14)) OF

CellAccessRelatedInfo-r14	OPTIONAL,	-- Need OR

nonCriticalExtension					SystemInformationBlockType1-v15xy-IEs					OPTIONAL

}

SystemInformationBlockType1-v15xy-IEs ::=	SEQUENCE {

cellAccessRelatedInfoFor-5GC-r15				SEQUENCE {

cellBarred-5GC-r15							ENUMERATED {barred, notBarred},

-- FFS on maximum number of PLMN lists for 5GC;  6 including EPC or indenpended

cellAccessRelatedInfoList-5GC-r15			SEQUENCE (SIZE (1..maxPLMN-1-r14)) OF

CellAccessRelatedInfo-r14

cellAccessRelateInfoOnLegacyList-5GC-r15 CellAccessRelateInfoOnLegacyList-5GC-r15   OPTIONAL,    --NEED OP

}				OPTIONAL,	-- NEED OP

--FFS on whether CellAccessRelatedInfo-r14 can be reused. It depends on whether TAC, CellIdentity are same between EPC and 5GC

nonCriticalExtension					SEQUENCE {}					OPTIONAL

}

PLMN-IdentityList ::=					SEQUENCE (SIZE (1..maxPLMN-r11)) OF PLMN-IdentityInfo

PLMN-IdentityInfo ::=					SEQUENCE {

plmn-Identity							PLMN-Identity,

cellReservedForOperatorUse				ENUMERATED {reserved, notReserved}

}

CellAccessRelateInfoOnLegacyList-5GC-r15    ::=	 SEQUENCE{

plmn5GC-BitMap                            BIT STRING (SIZE (maxPLMN-r11)),

cellReservedForOperatorUse-5GC-List       SEQUENCE (SIZE (1..maxPLMN-11)) OF

cellReservedForOperatorUse-5GC

}

cellReservedForOperatorUse-5GC ::= ENUMERATED {reserved, notReserved}

SchedulingInfoList ::= SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo

SchedulingInfo ::=	SEQUENCE {

si-Periodicity						ENUMERATED {

rf8, rf16, rf32, rf64, rf128, rf256, rf512},

sib-MappingInfo						SIB-MappingInfo

}

SchedulingInfoList-BR-r13 ::= SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo-BR-r13

SchedulingInfo-BR-r13 ::=	SEQUENCE {

si-Narrowband-r13				INTEGER (1..maxAvailNarrowBands-r13),

si-TBS-r13						ENUMERATED {b152, b208, b256, b328, b408, b504, b600, b712,

b808, b936}

}

SIB-MappingInfo ::= SEQUENCE (SIZE (0..maxSIB-1)) OF SIB-Type

SIB-Type ::=						ENUMERATED {

sibType3, sibType4, sibType5, sibType6,

sibType7, sibType8, sibType9, sibType10,

sibType11, sibType12-v920, sibType13-v920,

sibType14-v1130, sibType15-v1130,

sibType16-v1130, sibType17-v1250, sibType18-v1250,

..., sibType19-v1250, sibType20-v1310, sibType21-v1430}

SystemInfoValueTagList-r13 ::=		SEQUENCE (SIZE (1..maxSI-Message)) OF SystemInfoValueTagSI-r13

SystemInfoValueTagSI-r13 ::=		INTEGER (0..3)

CellSelectionInfo-v920 ::=			SEQUENCE {

q-QualMin-r9						Q-QualMin-r9,

q-QualMinOffset-r9					INTEGER (1..8)						OPTIONAL	-- Need OP

}

CellSelectionInfo-v1130 ::=			SEQUENCE {

q-QualMinWB-r11						Q-QualMin-r9

}

CellSelectionInfo-v1250 ::=			SEQUENCE {

q-QualMinRSRQ-OnAllSymbols-r12						Q-QualMin-r9

}

CellAccessRelatedInfo-r14 ::=	SEQUENCE {

plmn-IdentityList-r14				PLMN-IdentityList,

trackingAreaCode-r14				TrackingAreaCode,

cellIdentity-r14					CellIdentity

}

-- ASN1STOP

| SystemInformationBlockType1 field descriptions |
| --- |
| bandwithReducedAccessRelatedInfo Access related information for BL UEs and UEs in CE. NOTE 3. |
| category0Allowed The presence of this field indicates category 0 UEs are allowed to access the cell. |
| cellAccessRelatedInfoList This field contains a list allowing signalling of access related information per PLMN. One PLMN can be included in only one entry of this list. NOTE 4. |
| cellAccessRelatedInfoList-5GC This field contains a PLMN list and a list allowing signalling of access related information per PLMN for PLMNs that provides connectivity to 5GC. One PLMN can be included in only one entry of this list. FFS on the maximum PLMN number of the RAN can configure for 5GC, and the maximum PLMN number across EPC and 5GC |
| cellReservedForOperatorUse-5GC-List this field corresponding to the PLMNs indicated by plmn5GC-BitMap |
| cellReservedForOperatorUse-5GC this field provides the connectivity to 5GC, As defined in TS 36.304 [4]. |
| cellBarred barred means the cell is barred, as defined in TS 36.304 [4]. |
| cellBarred-5GC barred means the cell is barred for connectivity to 5GC, as defined in TS 36.304 [4]. |
| cellIdentity Indicates the cell identity. NOTE 2. |
| cellReservedForOperatorUse As defined in TS 36.304 [4]. |
| cellSelectionInfoCE Cell selection information for BL UEs and UEs in CE. NOTE 3. |
| cellSelectionInfoCE1 Cell selection information for BL UEs and UEs in CE supporting CE Mode B. E-UTRAN includes this IE only if cellSelectionInfoCE is present in SystemInformationBlockType1-BR. NOTE 3. |  |
| csg-Identity Identity of the Closed Subscriber Group the cell belongs to. |
| csg-Indication If set to TRUE the UE is only allowed to access the cell if it is a CSG member cell, if selected during manual CSG selection or to obtain limited service, see TS 36.304 [4]. |
| eCallOverIMS-Support Indicates whether the cell supports eCall over IMS services for UEs as defined in TS 23.401 [41]. If absent, eCall over IMS is not supported by the network in the cell. NOTE 2. |
| eDRX-Allowed The presence of this field indicates if idle mode extended DRX is allowed in the cell. The UE shall stop using extended DRX in idle mode if eDRX-Allowed is not present. |
| fdd-DownlinkOrTddSubframeBitmapBR The set of valid subframes for FDD downlink or TDD transmissions, see TS 36.213 [23]. If this field is not present, the set of valid subframes is the set of non-MBSFN subframes as indicated by mbsfn-SubframeConfigList. If neither this field nor mbsfn-SubframeConfigList is present, all subframes are considered as valid subframes for FDD downlink transmission, all DL subframes according to the uplink-downlink configuration (see TS 36.211 [21]) are considered as valid subframes for TDD DL transmission, and all UL subframes according to the uplink-downlink configuration (see TS 36.211 [21]) are considered as valid subframes for TDD UL transmission. The first/leftmost bit corresponds to the subframe #0 of the radio frame satisfying SFN mod x = 0, where x is the size of the bit string divided by 10. Value 0 in the bitmap indicates that the corresponding subframe is invalid for transmission. Value 1 in the bitmap indicates that the corresponding subframe is valid for transmission. |
| fdd-UplinkSubframeBitmapBR The set of valid subframes for FDD uplink transmissions for BL UEs, see TS 36.213 [23]. If the field is not present, then UE considers all uplink subframes as valid subframes for FDD uplink transmissions. The first/leftmost bit corresponds to the subframe #0 of the radio frame satisfying SFN mod x = 0, where x is the size of the bit string divided by 10. Value 0 in the bitmap indicates that the corresponding subframe is invalid for transmission. Value 1 in the bitmap indicates that the corresponding subframe is valid for transmission. |
| freqBandIndicatorPriority If the field is present and supported by the UE, the UE shall prioritize the frequency bands in the multiBandInfoList field in decreasing priority order. Only if the UE does not support any of the frequency band in multiBandInfoList, the UE shall use the value in freqBandIndicator field. Otherwise, the UE applies frequency band according to the rules defined in multiBandInfoList. NOTE 2. |
| freqBandInfo A list of additionalPmax and additionalSpectrumEmission values, as defined in TS 36.101 [42, table 6.2.4-1] for UEs neither in CE nor BL UEs and TS 36.101 [42, table 6.2.4E-1] for UEs in CE or BL UEs, for the frequency band in freqBandIndicator. If E-UTRAN includes freqBandInfo-v10l0 it includes the same number of entries, and listed in the same order, as in freqBandInfo-r10. |
| freqHoppingParametersDL Downlink frequency hopping parameters for BR versions of SI messages, MPDCCH/PDSCH of paging, MPDCCH/PDSCH of RAR/Msg4 and unicast MPDCCH/PDSCH. If not present, the UE is not configured downlink frequency hopping. |
| hyperSFN Indicates hyper SFN which increments by one when the SFN wraps around. |
| ims-EmergencySupport Indicates whether the cell supports IMS emergency bearer services for UEs in limited service mode. If absent, IMS emergency call is not supported by the network in the cell for UEs in limited service mode. NOTE 2. |
| intraFreqReselection Used to control cell reselection to intra-frequency cells when the highest ranked cell is barred, or treated as barred by the UE, as specified in TS 36.304 [4]. NOTE 2. |
| multiBandInfoList A list of additional frequency band indicators, as defined in TS 36.101 [42, table 5.5-1] that the cell belongs to. If the UE supports the frequency band in the freqBandIndicator field it shall apply that frequency band. Otherwise, the UE shall apply the first listed band which it supports in the multiBandInfoList field. If E-UTRAN includes multiBandInfoList-v9e0 it includes the same number of entries, and listed in the same order, as in multiBandInfoList (i.e. without suffix). See Annex D for more descriptions. The UE shall ignore the rule defined in this field description if freqBandIndicatorPriority is present and supported by the UE. |
| multiBandInfoList-v10j0 A list of additionalPmax and additionalSpectrumEmission values, as defined in TS 36.101 [42, table 6.2.4-1] for UEs neither in CE nor BL UEs and TS 36.101 [42, table 6.2.4E-1] for UEs in CE or BL UEs, for the frequency bands in multiBandInfoList (i.e. without suffix) and multiBandInfoList-v9e0. If E-UTRAN includes multiBandInfoList-v10j0, it includes the same number of entries, and listed in the same order, as in multiBandInfoList (i.e. without suffix). If E-UTRAN includes multiBandInfoList-v10l0 it includes the same number of entries, and listed in the same order, as in multiBandInfoList-v10j0. |
| plmn-IdentityList List of PLMN identities. The first listed PLMN-Identity is the primary PLMN. NOTE 2. |
| plmn5GC-BitMap bitmap of PLMN connected both 5GC and EPC, each bit in the field corresponds to one PLMN in the legacy PLMN list one by one, the LSB of the bitmap corresponding to the 1st PLMN in the 1st plmn-IdentityList included in SIB1, and the subsequent bit of the bitmap indicates the subsequent PLMN in the plmn-IdentityList or subsequent plmn-IdentityList within the same SIB1 if contained except the 5GC PLMN list. Value 1 indicates the corresponding PLMN can connect to 5GC, value 0 indicates the corresponding PLMN can only connect to EPC. |
| p-Max Value applicable for the cell. If absent the UE applies the maximum power according to the UE capability. NOTE 2. |
| q-QualMin Parameter “Qqualmin” in TS 36.304 [4]. If cellSelectionInfo-v920 is not present, the UE applies the (default) value of negative infinity for Qqualmin. NOTE 1. |
| q-QualMinRSRQ-OnAllSymbols If this field is present and supported by the UE, the UE shall, when performing RSRQ measurements, perform RSRQ measurement on all OFDM symbols in accordance with TS 36.214 [48]. NOTE 1. |
| q-QualMinOffset Parameter “Qqualminoffset” in TS 36.304 [4]. Actual value Qqualminoffset = field value [dB]. If cellSelectionInfo-v920 is not present or the field is not present, the UE applies the (default) value of 0 dB for Qqualminoffset. Affects the minimum required quality level in the cell. |
| q-QualMinWB If this field is present and supported by the UE, the UE shall, when performing RSRQ measurements, use a wider bandwidth in accordance with TS 36.133 [16]. NOTE 1. |
| q-RxLevMinOffset Parameter Qrxlevminoffset in TS 36.304 [4]. Actual value Qrxlevminoffset = field value * 2 [dB]. If absent, the UE applies the (default) value of 0 dB for Qrxlevminoffset. Affects the minimum required Rx level in the cell. |
| sib-MappingInfo List of the SIBs mapped to this SystemInformation message. There is no mapping information of SIB2; it is always present in the first SystemInformation message listed in the schedulingInfoList list. |
| si-HoppingConfigCommon Frequency hopping activation/deactivation for BR versions of SI messages and MPDCCH/PDSCH of paging. |
| si-Narrowband This field indicates the index of a narrowband used to broadcast the SI message towards BL UEs and UEs in CE, see TS 36.211 [21, 6.4.1] and TS 36.213 [23, 7.1.6]. Field values (1..maxAvailNarrowBands-r13) correspond to narrowband indices (0..[maxAvailNarrowBands-r13-1]) as specified in TS 36.211 [21]. |
| si-RepetitionPattern Indicates the radio frames within the SI window used for SI message transmission. Value everyRF corresponds to every radio frame, value every2ndRF corresponds to every 2 radio frames, and so on. The first transmission of the SI message is transmitted from the first radio frame of the SI window. |
| si-Periodicity Periodicity of the SI-message in radio frames, such that rf8 denotes 8 radio frames, rf16 denotes 16 radio frames, and so on. |
| si-TBS This field indicates the transport block size information used to broadcast the SI message towards BL UEs and UEs in CE, see TS 36.213 [23, Table 7.1.7.2.1-1] for a 6 PRB bandwidth and a QPSK modulation. |
| schedulingInfoList-BR Indicates additional scheduling information of SI messages for BL UEs and UEs in CE. It includes the same number of entries, and listed in the same order, as in schedulingInfoList (without suffix). |
| si-ValidityTime Indicates system information validity timer. If set to TRUE, the timer is set to 3h, otherwise the timer is set to 24h. |
| si-WindowLength, si-WindowLength-BR Common SI scheduling window for all SIs. Unit in milliseconds, where ms1 denotes 1 millisecond, ms2 denotes 2 milliseconds and so on. In case si-WindowLength-BR-r13 is present and the UE is a BL UE or a UE in CE, the UE shall use si-WindowLength-BR-r13 and ignore the original field si-WindowLength (without suffix). UEs other than BL UEs or UEs in CE shall ignore the extension field si-WindowLength-BR-r13. |
| startSymbolBR For BL UEs and UEs in CE, indicates the OFDM starting symbol for any MPDCCH, PDSCH scheduled on the same cell except the PDSCH carrying SystemInformationBlockType1-BR, see TS 36.213 [23]. Values 1, 2, and 3 are applicable for dl-Bandwidth greater than 10 resource blocks. Values 2, 3, and 4 are applicable otherwise. |
| systemInfoValueTagList Indicates SI message specific value tags for BL UEs and UEs in CE. It includes the same number of entries, and listed in the same order, as in schedulingInfoList (without suffix). |
| systemInfoValueTagSI SI message specific value tag as specified in subclause 5.2.1.3. Common for all SIBs within the SI message other than MIB, SIB1, SIB10, SIB11, SIB12 and SIB14. |
| systemInfoValueTag Common for all SIBs other than MIB, MIB-MBMS, SIB1, SIB1-MBMS, SIB10, SIB11, SIB12 and SIB14. Change of MIB, MIB-MBMS, SIB1 and SIB1-MBMS is detected by acquisition of the corresponding message. |
| tdd-Config Specifies the TDD specific physical channel configurations. NOTE 2. |
| trackingAreaCode A trackingAreaCode that is common for all the PLMNs listed. NOTE2. NOTE 5. |