3GPP TSG-RAN WG2 Meeting	R2-1802444

Athens, Greece, 26th February – 2nd March, 2018

Agenda item:	10.4.1.5.3

Source: 	Qualcomm Incorporated

Title: 	Configuration of power control parameters during handover through CFRA

WID/SID:	NR_newRAT-Core– Release 15

Document for:	Decision

# 1. Introduction

Following text of 38.331 [1] shows that RAN2 has already allowed network the flexibility to configure dedicated time and frequency region to convey CFRA during handover scenarios.

RACH-ConfigDedicated ::=		SEQUENCE {

-- Resources for handover to the cell

cfra-Resources					CFRA-Resources,

-- Subcarrier spacing for msg2 for contention-free RA procedure for handover

rar-SubcarrierSpacing			SubcarrierSpacing

}

-- FFS_CHECK: Isn’t it sufficient to have just one list and the CHOICE inside the list element (around the ssb/csirs)?

CFRA-Resources ::= 				CHOICE {

cfra-ssb-ResourceList			SEQUENCE (SIZE(1..maxRAssbResources) OF CFRA-SSB-Resource,

cfra-csirs-ResourceList			SEQUENCE (SIZE(1..maxRAcsirsResources)OF CFRA-CSIRS-Resource

}

CFRA-SSB-Resource ::= 			SEQUENCE {

ssb								SSB-ID,

ra-PreambleIndex				INTEGER (0..FFS_XX),

-- 	PRACH configuration for SSB configuration (i.e. time and frequency location)

ra-Resources				RA-Resources -- Definition FFS

}

This article focuses on how network could configure power control parameters to perform CFRA through dedicated time and frequency region during handover.

# 2. Description

According to the current version of 38.331, network provides the same configuration regarding different power control parameters, e.g., preambleReceivedTargetPower, powerRampingStep, preambleTransMax, etc., for both CBRA and CFRA.

CFRA through dedicated time and frequency RACH region does not get interfered by UEs that perform contention based random access. Hence, unlike common RACH time-frequency RACH occasions that may contain both CBRA preambles and dedicated CFRA preambles, network can completely control the level of congestion during CFRA through dedicated time and frequency RACH region.

Hence, NR should allow network the flexibility to configure different power control parameters, e.g., preambleReceivedTargetPower, powerRampingStep, preambleTransMax, etc. separately during CFRA through dedicated time and frequency RACH region. This can simply be achieved if network has the flexibility to select these parameters during CFRA.

Observation 1: CFRA through dedicated time and frequency RACH region does not get interfered by UEs that perform contention based random access. Unlike common time-frequency RACH regions, network can completely control the level of congestion during CFRA through dedicated time and frequency RACH region.

Proposal 1: NR gives network the flexibility to configure different power control parameters, , e.g., preambleReceivedTargetPower, powerRampingStep, preambleTransMax, for CFRA and CBRA during handover.

Proposal 2: Adopt following change (shown in yellow marks) in 38.331.

CHANGE START

RACH-ConfigDedicated ::=		SEQUENCE {

-- Resources for handover to the cell

cfra-Resources					CFRA-Resources,

-- Subcarrier spacing for msg2 for contention-free RA procedure for handover

rar-SubcarrierSpacing			SubcarrierSpacing

preambleReceivedTargetPower				ENUMERATED {

dBm-120, dBm-118, dBm-116, dBm-114, dBm-112, dBm 110, dBm-108, dBm-106, dBm-104, dBm-102, dBm-100,dBm-98, dBm-96, dBm-94,dBm-92, dBm-90, dBm-88, dBm-86, dBm-84,dBm-82, dBm-80, dBm-78, dBm-76,dBm-74, dBm-72, dBm-70, dBm-68, dBm-66, dBm-64, dBm-62, dBm-60, dBm-58, dBm-56, dBm-54, dBm-52,dBm-50, dBm-48, dBm-46, dBm-44, dBm-42, dBm-42, dBm-40, dBm-38, dBm-36, dBm-34, dBm-32, dBm-30, dBm-28, dBm-26, dBm-24, dBm-22, dBm-20, dBm-18, dBm-16, dBm-14, dBm-12, dBm-10, dBm-8, dBm-6,dBm-4, dBm-2, dBm-0, dBm2, dBm4, dBm6 } OPTIONAL,

powerRampingStep						ENUMERATED {dB0, dB2, dB4, dB6}	 OPTIONAL

PreambleTransMax ::= ENUMERATED {n3, n4, n5, n6, n7,	n8, n10, n20, n50, n100, n200}

}

CHANGE END

# 3. Conclusion

Observation 1: CFRA through dedicated time and frequency RACH region does not get interfered by UEs that perform contention based random access. Unlike common time-frequency RACH regions, network can completely control the level of congestion during CFRA through dedicated time and frequency RACH region.

Proposal 1: NR gives network the flexibility to configure different power control parameters, , e.g., preambleReceivedTargetPower, powerRampingStep, preambleTransMax, for CFRA and CBRA during handover.

Proposal 2: Adopt following change (shown in yellow marks) in 38.331.

CHANGE START

RACH-ConfigDedicated ::=		SEQUENCE {

-- Resources for handover to the cell

cfra-Resources					CFRA-Resources,

-- Subcarrier spacing for msg2 for contention-free RA procedure for handover

rar-SubcarrierSpacing			SubcarrierSpacing

preambleReceivedTargetPower				ENUMERATED {

dBm-120, dBm-118, dBm-116, dBm-114, dBm-112, dBm 110, dBm-108, dBm-106, dBm-104, dBm-102, dBm-100,dBm-98, dBm-96, dBm-94,dBm-92, dBm-90, dBm-88, dBm-86, dBm-84,dBm-82, dBm-80, dBm-78, dBm-76,dBm-74, dBm-72, dBm-70, dBm-68, dBm-66, dBm-64, dBm-62, dBm-60, dBm-58, dBm-56, dBm-54, dBm-52,dBm-50, dBm-48, dBm-46, dBm-44, dBm-42, dBm-42, dBm-40, dBm-38, dBm-36, dBm-34, dBm-32, dBm-30, dBm-28, dBm-26, dBm-24, dBm-22, dBm-20, dBm-18, dBm-16, dBm-14, dBm-12, dBm-10, dBm-8, dBm-6,dBm-4, dBm-2, dBm-0, dBm2, dBm4, dBm6 } OPTIONAL,

powerRampingStep						ENUMERATED {dB0, dB2, dB4, dB6}	 OPTIONAL

PreambleTransMax ::= ENUMERATED {n3, n4, n5, n6, n7,	n8, n10, n20, n50, n100, n200}

}

CHANGE END

# 4. References

1. 3GPP TS 38.331, V15.0.0 Radio Resource Control (RRC) protocol specification (Release 15).