3GPP TSG-RAN WG2 Meeting 101	R2-1802833

# Athens, Greece, 26 February-02 March, 2018

Agenda Item:	10.4.3.2

Source: 	Huawei, HiSilicon

Title: 	PDSCH and PUSCH UE-specific parameters

Document for:	Discussion and decision

# Introduction

This document addresses an ASN.1 issue regarding the handling of some UE-specific parameters in PDSCH-Config and PUSCH-Config.

Our understanding of the state of BWP- and UE-specific parameters in PDCCH-Config/PDSCH-Config/PUCCH-Config/PUSCH-Config is provided in Annex A (see also [1]).

# Discussion

Since PDSCH-Config and PUSCH-Config are included in the downlink and uplink (respectively) BWP structures, they can accommodate signalling of BWP-specific parameters.  However, signalling UE-specific parameters requires some care.  In some cases e.g. lte-CRS-ToMatchAround in PDSCH-Config, delta signalling provides a solution; the parameter is UE-specific, but it can be modified or replaced whenever the PDSCH-Config is signalled, i.e. at downlink BWP configuration or reconfiguration.

The same situation arises with some parameters in PDCCH-Config (controlResourceSetToAddModList, searchSpacesToAddModList, and slotFormatIndicator) and PUCCH-Config (scramblingID and pucch-PowerControl); in these cases delta signalling can be applied (see ASN.1 issues H313 and H314).

For smaller parameters with less structure, delta signalling has generally not been specified.  Moreover, certain parameters have default values indicated in the RAN1 spreadsheet, which prevents delta signalling from working.  This issue arises with the following parameters, all of which are supposed to be UE-specific:

- mcs-Table in PDSCH-Config: Currently a mandatory parameter.

- rateMatching in PUSCH-Config: Optional with default value.

- mcs-Table in PUSCH-Config: Optional with default value.

- mcs-TableTransformPrecoder in PUSCH-Config: Optional with default value.

- transformPrecoder in PUSCH-Config: Optional with default value.

In the current format, all these parameters will be updated whenever the PDSCH-Config/PUSCH-Config is sent.  Thus the network implementation is responsible for maintaining consistent settings for the UE.  If we keep this structure, there should be some guidance to the UE, so that it is clear that the received value is applied at the UE-specific level and the UE implementation is not responsible for maintaining separate values per BWP.

The alternative would be to disregard the default values, make all the fields OPTIONAL, and allow delta signalling for these parameters.  This approach seems technically feasible but would violate the guidance of the RAN1 spreadsheet regarding default values.

Proposal 1: For the parameters mcs-Table in PDSCH-Config, and rateMatching, mcs-Table, mcs-TableTransformPrecoder, and transformPrecoder in PUSCH-Config, determine which approach to adopt:

- Make the fields OPTIONAL without default values, and allow delta signalling;

- Leave the fields with their current mandatory/optional status including default values, and capture explicitly that the fields are UE-specific.

# Conclusion

Proposal 1: For the parameters mcs-Table in PDSCH-Config, and rateMatching, mcs-Table, mcs-TableTransformPrecoder, and transformPrecoder in PUSCH-Config, determine which approach to adopt:

- Make the fields OPTIONAL without default values, and allow delta signalling;

- Leave the fields with their current mandatory/optional status including default values, and capture explicitly that the fields are UE-specific.

# References

[1] R1-1801347, “Summary of remaining issues on bandwidth part and wideband operation”, Huawei/HiSilicon, RAN1#92

# Annex A: BWP-specific and UE-specific parameters

| PDCCH-Config |  |  |  |
| --- | --- | --- | --- |
| RAN2 parameter name | RAN1 parameter name | Spreadsheet guidance | RAN1 guidance |
| controlResourceSetToAddModList | CORESET | UE specific | UE specific |
| searchSpacesToAddModList | search-space-config | UE specific – but also talks of the number that are configurable per BWP | UE specific |
| downlinkPreemption | Multiple DL preemption parameters | UE specific – but DL-preemption-int indicates “monitoring is done per DL BWP” | Preemp-DL, int-TF-unit is BWP specific |
| slotFormatIndicator | SFI-PDCCH | UE specific | UE specific |
| timing | Multiple timing parameters | BWP specific | BWP specific |

| PDSCH-Config |  |  |  |
| --- | --- | --- | --- |
| RAN2 parameter name | RAN1 parameter name | Spreadsheet guidance | RAN1 guidance |
| codeBlockGroupTransmission | CBG-DL | UE specific | BWP specific |
| maxCodeBlockGroupsPerTransportBlock | CBGs-per-TB-DL | UE specific | BWP specific |
| codeBlockGroupFlushIndicator | CBG-DL-FI | UE specific | BWP specific |
| dmrs-Downlink | Multiple DMRS parameters | UE specific | BWP specific |
| phaseTracking-RS | Multiple PTRS parameters | BWP specific (DL-PTRS-frequency-density-table, DL-PTRS-time-density-table) UE specific (others) | BWP specific |
| tci-States | TCI-States | UE specific | BWP specific |
| xOverhead | Xoh-PDSCH | UE specific | BWP specific |
| vrb-to-PRB-Interleaver | VRB-to-PRB-interleaver | UE specific | BWP specific |
| resourceAllocation | Resouce-allocation-config | UE specific | BWP specific |
| pdsch-SymbolAllocation | PDSCH-symbol-allocation | UE specific | BWP specific |
| rateMatchPatterns | rate-match-PDSCH-resource-set | Cell specific (in Resource-set-cell) and BWP specific (in Resource-set-BWP) | BWP specific |
| lte-CRS-ToMatchAround | Multiple frame structure parameters | UE specific | UE specific |
| RBG size | rbg-size-PDSCH | UE specific | BWP specific |
| mcs-Table | MCS-Table-PDSCH | UE specific | UE specific |
| maxNrofCodeWordsScheduledByDCI | Number-MCS-HARQ-DL-DCI | UE specific | BWP specific |
| nrofHARQ-processesForPDSCH | number-HARQ-process-PDSCH | Configured per cell for a UE | BWP specific |
| harq-ACK-Codebook | HARQ-ACK-codebook | UE specific | BWP specific |
| pdsch-BundleSize | PDSCH-bundle-size | UE specific | BWP specific |
| prbBundlingEnabled | PRB_bundling | UE specific | BWP specific |
| zp-CSI-RS-Resources | ZP-CSI-RS-ResourceConfig | UE specific | BWP specific |

| PUCCH-Config |  |  |  |
| --- | --- | --- | --- |
| RAN2 parameter name | RAN1 parameter name | Spreadsheet guidance | RAN1 guidance |
| resourceSets | PUCCH-resource-set | UE specific | BWP-specific |
| format1 | PUCCH-F1-resource-config | UE specific | BWP-specific |
| format2 | PUCCH-F2-resource-config | UE specific | BWP-specific |
| format3 | PUCCH-F3-resource-config | UE specific | BWP-specific |
| format4 | PUCCH-F4-resource-config | UE specific | BWP-specific |
| schedulingRequestResources | SR-configurations | UE specific | BWP-specific |
| scramblingID | ScramblingID | UE specific | UE specific |
| spatialRelationInfo | PUCCH-SpatialRelationInfo | UE specific | BWP-specific |
| pucch-PowerControl | TPC-PUCCH-RNTI p0-pucch-set pucch-pathlossReference-rs-config num-pucch-pcadjustment-states | UE specific | UE specific |

| PUSCH-Config |  |  |  |
| --- | --- | --- | --- |
| RAN2 parameter name | RAN1 parameter name | Spreadsheet guidance | RAN1 guidance |
| codeBlockGroupTransmission | CBG-UL | UE specific | BWP specific |
| maxCodeBlockGroupsPerTransportBlock | CBGs-per-TB-UL | UE specific | BWP specific |
| dmrs-Uplink | UL-DMRS-config-type UL-DMRS-add-pos Multiple PTRS parameters UL-DMRS-max-len UL-DMRS-Scrambling-ID nDMRS-CSH-Identity-Transform-precoding nPUSCH-Identity-Transform-precoding Disable-sequence-group-hopping-Transform-precoding Sequence-hopping-enabled-Transform-precoding Activate-DMRS-with OCC-Transform-predocing CyclicShift-Transform-precoding groupAssignmentPUSCH-Transform-precoding | UE specific | BWP specific |
| pusch-PowerControl | TPC-PUSCH-RNTI Accumulation-enabled alpha-ue-pusch-msg3 p0-nominal-pusch-withoutgrant p0-push-alpha-setconfig pusch-pathlossReference-rs-config num-pusch-pcadjustment-states deltaMCS-Enabled | UE specific | UE specific |
| frequencyHopping | Frequency-hopping-PUSCH | UE specific | BWP specific |
| rateMatching | LBRM-FBRM-selection | UE specific | UE specific |
| resourceAllocation | Resouce-allocation-config | UE specific | BWP specific |
| mcs-Table | MCS-Table-PUSCH | UE specific | UE specific |
| mcs-TableTransformPrecoder | MCS-Table-PUSCH-transform-precoding | UE specific | UE specific |
| transformPrecoder | PUSCH-tp | UE specific | UE specific |
| rbg-Size | RBG-size-PUSCH | UE specific | BWP specific |
| uci-on-PUSCH | UCI-on-PUSCH | UE specific | BWP specific |
| xOverhead | Xoh-PUSCH | UE specific | BWP specific |
| frequencyHoppingOffsets | Frequency-hopping-offsets-set | UE specific | BWP specific |