3GPP TSG-RAN WG2 Meeting #101	R2-1802865

# Athens, Greece, 26th February - 2nd March 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.314 | CR | 0049 | rev | - | Current version: | 14.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on distribution of total PRB usage |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15 |  |  |  |  |  |  | Date: |  | 2018-2-14 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the SA5 LS R2-1704027, it was mentioned that with the PRB usage distribution measurements, the operator could know whether there are traffic bursts in the network during the monitoring period. However, such measurements are not supported in the current specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Some changes are made in order to support distribution of total PRB usage. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SA5 requirement on distribution of total PRB usage cannot be met. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.x (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

4.1.x	Distribution of PRB usage

The objective of the distribution of PRB usage measurements is to measure the distribution of the usage of time and frequency resources. A use case is cell load balancing, where PRB usage is used for information signalled across the X2 interface. Another use-case is OAM performance observability.

If there is one or more RNs served in a cell, for that cell the eNB performs PRB usage measurements separately for all traffic (including transmissions to/from RNs and UEs directly connected to the eNB) and for RN traffic.

4.1.x.1	Distribution of total PRB usage

Protocol Layer: MAC

| Definition | Distribution of total PRB usage is calculated in the time-frequency domain only. The reference point is the Service Access Point between MAC and L1. The measurement is done separately for: - DL - UL The distribution of PRB usage provides the histogram result of the samples collected during time period T. Considering there are n samples during time T and each sample has the same time period tn, detailed definitions of each sample value at instant n are: Mn=M1nPn*100, where explanations can be found in the table 4.1.x.1-1 below. |
| --- | --- |

Table 4.1.x.1-1

| Mn | Total PRB usage at sample n. Percentage of PRBs used, averaged during time period tn. Value range: 0-100% |
| --- | --- |
| M1n | A count of full physical resource blocks. For the DL, all PRBs used for transmission shall be included. For the UL, all PRBs allocated for transmission shall be included. |
| Pn | Total number of PRBs available during time period tn. For an eNB serving one or more RNs, all PRBs regardless of RN subframe configurations shall be counted. (NOTE) |
| n | The sample with time period tn during which the measurement is performed. |

NOTE: 	It is up to the eNB implementation how to calculate P(n) with respect to PRBs that may be considered not available, e.g. MBSFN subframes and subframes subject to restrictions due to TDM ICIC.