3GPP TSG-RAN WG2 Meeting #101	R2-1803051

Athens, Greece, 26th February – 2nd March, 2018	(Resubmission of R2-1801252)

Agenda Item	: 10.2.11 (Slicing)

Source	: LG Electronics Inc.

Title	: Signalling of network slicing information to RAN

Document for	: Discussion and Decision

1.	Introduction

In the 5G system, a UE may provide slice information for AMF selection during initial registration procedure and for PDU session request. Since an AMF maintains slicing information and updates UEs, we expect that slicing information is generally exchanged via NAS signalling between the network and the UE. In this paper, we discuss the cases that the UE needs to provide slicing information to RAN via RRC signalling.

2.	Discussion

The UE may provide the configured or allowed S-NSSAI(s) as assistant information to the gNB to support the AMF selection during the initial registration procedure. If the gNB is unable to select an AMF using the assistant information or the UE does not provide any slicing information to the gNB, the gNB will select a default AMF.

When the UE provides a full list of S-NSSAI(s), the probability to find the AMF providing the mapping S-NSSAI(s) with the requested S-NSSAI(s) may be higher than that with a subset of the list. In some cases, however, providing a full list of S-NSSAI(s) may not be efficient; the operator may require to use a particular S-NSSAI as assistance information for initial registration. For instance, the UE may be required to provide S-NSSAI(s) for IoT services as assistant information so that the UE activates the IoT related PDU session(s) right after registration. Although the UE is pre-configured with S-NSSAI(s) for IMS services, it could provide the S-NSSAI(s) when the UE activates a PDU session for a voice call.

Proposal 1. The UE may provide either a full list of S-NSSAI(s) or a subset of the list as assistance information based on the operator's requirements.

The allowed network slices for the UE can be changed at any time (e.g. due to registration area change, temporal resource unavailability). If the new allowed NSSAI is provided to the UE, the updated information will be delivered to the UE via NAS signalling using a registration or notification procedure. If some NSSAI used for active PDU session(s) is released, the AMF will inform to the SMF and SMF will release the PDU session(s).

Observation. When the list of slices for a UE is updated, the network updates UE's configuration via NAS signalling.

When the allowed slices for the UE is changed, the gNB also needs to be updated so that the gNB can select a proper resource for supporting network slices and properly configure the UEs. It is likely that the UE informs of the updated slices to the gNB rather than the AMF because it is the UE specific information.

Proposal 2. When the list of slices for a UE is updated, the UE NAS provides the updated slice information to the UE AS so that the UE informs of the update to the gNB. RAN2 should inform CT1 about this assumption.

3.	Conclusion

In this contribution, we discussed signalling of network slice information to RAN. Our proposals are:

Proposal 1. The UE may provide either a full list of S-NSSAI(s) or a subset of the list as assistance information based on the operator's requirements.

Observation. When the list of slices for a UE is updated, the network updates UE's configuration via NAS signalling.

Proposal 2. When the list of slices for a UE is updated, the UE NAS provides the updated slice information to the UE AS so that the UE informs of the update to the gNB. RAN2 should inform CT1 about this assumption.