3GPP TSG-RAN WG2 #101	R2-1803157

Athens, Greece, 26th February – 2nd March 2018

Agenda Item:	10.3.4.2

Source:	Ericsson

Title:	SDAP QFI mapping between AS and NAS

Document for:	Discussion, Decision

# Introduction

In the January 2018-AH meeting RAN2 agreed:

Agreements

=>	Support independent AS and NAS reflective QoS.

=>	From RAN2 perspective supporting up to 64 reflective flows per PDU session per UE is sufficient at the same time, so 6 bits QFI in SDAP.

=>	Ask SA2/CT1 if they expect to use more than 64 reflective flows per PDU session per UE at a time.  Indicate RAN2 agreement and strong need to have 6bits SDAP.   Questions will be included in SA2 LS from main session.

FFS if final QFI in CT1/SA2 is larger than 6 bits, a mechanism to remap NAS QFI to AS QFI may be needed

This contribution discusses about alternative options to implement the QFI on RAN and 5GC. As the QFI should be seen as end-to-end solution, this contribution compares options from UE, RAN and CN perspective.

# Discussion

On the last RAN2 meeting, 6-bit indication of QFI in the SDAP header was proposed. In addition, using explicit bits for NAS reflective QoS as well as AS reflective QoS. From a perspective of code points needed for complete QoS mapping of 5Qis, the QFI value should be at least 7-bits. However, this assumption may not be met. Therefore, alternative solutions should be explored.

A 6-bit QFI only may be available although 7-bits are required

RAN2 is proposing 6-bit QFI even if the SA2 indicated already previously that 7-bits is required

Two main approaches can be seen on this topic. One is to limit the impact of the QFI to RAN, i.e. to have local QFI in RAN. Another option is to push the impact of 6-bit QFI to 5GC and have 6-bit QFI limitation for the whole system. It is worth of noticing that even if RAN tries to absorb the impact of this change, the SMF will still need to respect the limit of 64 configured QFIs per PDU session. In general, having additional layer of mapping on RAN in QFI would unbind the NAS and AS QFI, resulting that 5GC could potentially have support for 8-bit QFI. On the other hand, the 2-level mapping would require that RAN would need to have QFI processing per packet basis (assuming that multiple QFIs are mapped per DRB).

There are two main approaches, either RAN adapts to CN or CN adapts to RAN on 3GPP access case.

Additional impact on 6-bit QFI in 5GC is that QFI value cannot anymore be bound to the 5QI value in 5GC. With the 7-bit QFI, it would be possible to indicate 5QI value directly with the QFI value. If the 6-bit QFI is selected, this is no longer possible for values exceeding 63. One option then is to reshuffle the 5QI table so that all standardized 5QI’s are below 64, but that would impact NR-LTE interworking, and operator adaptation would be required. Another option is to always signal both QFI and 5QI when the 5QI is above 64.

Having 6-bit QFI values on 5GC would, for 5QI’s above 64, prevent usage of 5QI as QFI.

## Option1: 7-Bit QFI in 5GC, Mapping to 6 bits in RAN

One approach is to use 7-bit QFI in 5GC, which gNB would then map into local to RAN 6-bit AS-QFI value. This approach requires that RAN always configures with RRC the UE so that there is a mapping from NAS QFI to RAN QFI. Additional impact of this option is that the NAS reflective QoS behaviour is slightly modified as RAN is required to send the NAS-QFI to AS-QFI mapping to UE whenever new QFI appears on system. Otherwise UE would not be able to resolve the AS-QFI from the NAS-QFI in UL Traffic.

One benefit of this solution is that if QFI value is extended in RAN in the future, there would not be 5GC impact, except for the SMF.

| UE | RRC: UE is configured (per DRB) with mapping from NAS-QFI to AS-QFI DL: UE receives AS-QFI and maps it to NAS-QFI UL: UE uses NAS-QFI on NAS-level and marks NAS-QFI on RAN. (SDAP is responsible of mapping the NAS-QFI to AS-QFI) |
| --- | --- |
| RAN | RRC: Ran configures UE with NAS-QFI to AS-QFI mapping DL: Mapping of AS-QFI to NAS-QFI UL: RAN resolves AS-QFI to NAS-QFI towards UPF. |
| 5GC | SMF: Enforces the limit of 64 QFIs per PDU session UPF: Marks QFI to N3 (NG-U) header |

Option 1: RAN uses 6-bit QFI, 5GC uses 7-bit QFI. RAN maps NAS-QFI to local AS-QFI always. SMF limits the maximum number of QFIs to 64.

## Option 2: 7 Bit QFI in CN, Mapping to 6 bits in RAN when needed

In an alternative option, where RAN uses 6-bit QFI and 5GC uses 7-bit QFI, there is a 1-1 mapping of NAS-QFI to AS-QFI for values that are below or equal to 63. For values exceeding 63, RAN would signal NAS-QFI to AS-QFI mapping. The SMF would need to limit the maximum number of QFI values to 64.

| UE | RRC: UE is configured (per DRB) with mapping from NAS-QFI to AS-QFI for NAS QFI values that exceed 63. UE enables mapping functionality for QFI based on RAN signalling. DL: For QFIs <= 63, NAS-QFI == AS QFI. For QFIs > 63, UE receives AS-QFI and maps it to NAS-QFI UL: For QFIs <= 63, NAS-QFI == AS QFI. For QFIs > 63, UE uses NAS-QFI on NAS-level and marks AS-QFI on RAN based on SDAP mapping. (SDAP is responsible of mapping the NAS-QFI to AS-QFI) |
| --- | --- |
| RAN | RRC: QFIs > 63, Ran configures UE with NAS-QFI to AS-QFI mapping. DL: Mapping of AS-QFI to NAS-QFI UL: RAN resolves AS-QFI to NAS-QFI towards UPF. |
| 5GC | SMF: Enforces the limit of 64 QFIs per PDU session UPF: Marks QFI to N3 (NG-U) header |

Option 2: RAN uses 6-bit QFI, 5GC uses 7-bit QFI. RAN uses same QFI as 5GC for values below or equal to 63 and has explicit mapping for values above 64. SMF limits the maximum number of QFIs to 64

## Option3: Limiting QFI to 6 Bit in CN for 3GPP Access

Alternative approach is to have 6-bit QFI for 3GPP access. This option has most impact on 5GC and it would be up to SA2 to decide whether this approach is used. Additionally, the limit would potentially be only enforced on 3GPP access, i.e. non 3GPP access could still use higher values and have standardized QFI values beyond 63. On this approach, SMF would limit the maximum number of QFIs to 64 for 3GPP access and UPF would mark QFIs up to 64. The 5QI value table would need to be reshuffled and Operators would not be able to have common QoS parameters between LTE and NR without mapping.

| UE | NAS-QFI == AS QFI |
| --- | --- |
| RAN | NAS-QFI == AS QFI |
| 5GC | SMF: Enforces the limit of 64 QFIs per PDU session UPF: Marks 6-bit QFI on N3 for 3GPP access. 5QI table would need to be reorganized and LTE-NR interoperability would need to be resolved. It is up to SA2 to decide whether this option is viable |

Option 3: RAN uses 6-bit QFI, 5GC uses 7-bit QFI. RAN uses same QFI as 5GC for values below or equal to 63 and has explicit mapping for values above 64. SMF limits the maximum number of QFIs to 64

## Option 4: Limiting QFI to 6-bit in 5GC when needed

Another alternative is to enforce QFI limitation only when it is needed. When considering SDAP header format, it has 1-bit for NAS-RQI. The NAS-RQI bit can be used only for QoS Flows where reflective QoS is configured via N2 signalling. I.e. it is fully depending on 5GC configuration whether there is space on RAN2 header or not. Therefore, options exist for 6-bit QFI and 7-bit QFI. The 6-bit QFI would be used for QoS Flows where NAS if flow may be subject for reflective qos, and 7-bit QFI is used otherwise. This adds over the air overhead efficiency as we would not carry empty-bit when there is RQA (Reflective QoS Argument) for a specific QoS Flow. This solution would allow RAN to support either 64 Reflective QoS Flows or maximum of 128 non-reflective QoS Flows per PDU session and implementation would be possible via existing specification mechanisms.

As the NAS reflective QoS support is optional for UEs, UEs that do not implement the Reflective QoS, would only use the 7-bit QFI format with 128 QFI values, there would be no problem on QFI mapping. No extra processing needed.

| UE | NAS-QFI == AS QFI |
| --- | --- |
| RAN | NAS-QFI == AS QFI RAN indicates to UE which QFIs are reflective. |
| 5GC | SMF: Enforces the limit of 64 reflective QoS flows, and 128 QFIs per PDU session. 5GC enforces the limit that For QFIs > 63, reflective QoS is not used in 3GPP access. |

Option 4: RAN uses 6-bit QFI format with reflective QoS flows and 7-bit format with non-reflective QoS Flows. 5GC does not use Reflective QoS with values exceeding 63 in 5QI table.

Option 4 is future proof as it allows 64 reflective and total 128 QFIs to be configure.

Option 4 does support usage of QFI as 5QI (Standardized QFI value)

## Summary of options

Following table summarizes different options based on effort that is required from each entity. While option 3 is tempting from RAN2 point of view, it may not be accepted by SA2. Therefore, the option 4 provides decent compromise on complexity and implementation effort. Figures 1 and 2 present the SDAP data formats with and without RQI. Both data formats would be valid with option 4.

|  | UE | RAN | CN |
| --- | --- | --- | --- |
| Option 1 | High | High | Minimal |
| Option 2 | High | High | Low |
| Option 3 | Medium | Medium | High |
| Option 4 | Low | Low | Low |

Following options are discussed in RAN2:

- Option 1: 	RAN uses 6-bit QFI, 5GC uses 7-bit QFI. RAN maps NAS-QFI to local AS-QFI always. SMF limits the maximum number of QFIs to 64.

- Option 2: 	RAN uses 6-bit QFI, 5GC uses 7-bit QFI. RAN uses same QFI as 5GC for values below or equal to 63 and has explicit mapping for values above 64. SMF limits the maximum number of QFIs to 64

- Option 3: 	RAN uses 6-bit QFI, 5GC uses 7-bit QFI. RAN uses same QFI as 5GC for values below or equal to 63 and has explicit mapping for values above 64. SMF limits the maximum number of QFIs to 64

- Option 4: 	RAN uses 6-bit QFI format with reflective QoS flows and 7-bit format with non-reflective QoS Flows. 5GC does not use Reflective QoS with values exceeding 63 in 5QI table.

Options 1-3 are discussed. Option 4 is preferred because it is providing minimum implementation effort with maximum flexibility

# Text Proposal

### 5.2.1	Uplink

At the reception of an SDAP SDU from upper layer for a QoS flow, the transmitting SDAP entity shall:

-	if there is no stored QoS flow to DRB mapping rule for the QoS flow as specified in the subclause 5.3:

-	map the SDAP SDU to the default DRB;

-	else:

-	map the SDAP SDU to the DRB according to the stored QoS flow to DRB mapping rule;

-	if the DRB to which the SDAP SDU is mapped is configured by RRC [3] with the presence of SDAP header,

- if reflective QoS is configured by RRC for the DRB

-	construct the SDAP PDU as specified in the subclause 6.2.2.2-1;

else:

- construct the SDAP PDU as specified in the subclause 6.2.2.2-2;

-	else:

-	construct the SDAP PDU as specified in the subclause 6.2.2.1;

-	deliver the constructed SDAP PDU to the lower layers.

### 5.2.2	Downlink

At the reception of an SDAP PDU from lower layers for a QoS flow, the receiving SDAP entity shall:

-	if the DRB from which this SDAP PDU is received is configured by RRC [3] with the presence of SDAP header:

-	perform reflective QoS flow to DRB mapping as specified in the subclause 5.3.2;

-	perform RQI handling as specified in the subclause 5.4;

-  if reflective QoS is configured by RRC for the DRB

- retrieve the SDAP SDU from the SDAP PDU as specified in the subclause 6.2.2.2-1;

-  else:

- retrieve the SDAP SDU from the SDAP PDU as specified in the subclause 6.2.2.2-2;

#### 6.2.2.2	Data PDU with SDAP header

Figure 6.2.2.2 – 1 shows the format of SDAP Data PDU of DL with SDAP header being configured.

Figure 6.2.2.2-1: DL SDAP Data PDU format with 6-bit QFI SDAP header

Figure 6.2.2.2 – 2 shows the format of SDAP Data PDU of DL with SDAP header being configured with RDI and 7-bit QFI.

Figure 6.2.2.2-2: SDAP Data PDU format with 7-bit QFI SDAP header

# Conclusion

In section 2 we made the following observations:

Observation 1	A 6-bit QFI only may be available although 7-bits are required

Observation 2	RAN2 is proposing 6-bit QFI even if the SA2 indicated already previously that 7-bits is required

Observation 3	There are two main approaches, either RAN adapts to CN or CN adapts to RAN on 3GPP access case.

Observation 4	Having 6-bit QFI values on 5GC would, for 5QI’s above 64, prevent usage of 5QI as QFI.

Observation 5	Option 4 is future proof as it allows 64 reflective and total 128 QFIs to be configure.

Observation 6	Option 4 does support usage of QFI as 5QI (Standardized QFI value)

Based on the discussion in section 2 we propose the following:

Proposal 1	Following options are discussed in RAN2

Option 1:  RAN uses 6-bit QFI, 5GC uses 7-bit QFI. RAN maps NAS-QFI to local AS-QFI always. SMF limits the maximum number of QFIs to 64.

Option 2:  RAN uses 6-bit QFI, 5GC uses 7-bit QFI. RAN uses same QFI as 5GC for values below or equal to 63 and has explicit mapping for values above 64. SMF limits the maximum number of QFIs to 64

Option 3:  RAN uses 6-bit QFI, 5GC uses 7-bit QFI. RAN uses same QFI as 5GC for values below or equal to 63 and has explicit mapping for values above 64. SMF limits the maximum number of QFIs to 64

Option 4:  RAN uses 6-bit QFI format with reflective QoS flows and 7-bit format with non-reflective QoS Flows. 5GC does not use Reflective QoS with values exceeding 63 in 5QI table.

Proposal 2	Options 1-3 are discussed. Option 4 is preferred because it is providing minimum implementation effort with maximum flexibility