3GPP TSG-RAN WG2 #101 NR	Tdoc R2-1803165

Athens, Greece, 26 Feb - 2 March 2018

Agenda Item:	10.3.1.8

Source:	Ericsson

Title:	Remaining issues for configuredGrantTimer

Document for:	Discussion, Decision

# Introduction

In RAN2 #AH 1801, there were the following agreement with respect to the configuredGrantTimer:

=>  The configuredGrantTimer is restarted when a dynamic grant is received for CS-RNTI/C-RNTI for a (re-)transmission for this HARQ process. For both Type 1 and Type 2.  The timer is re-started upon PUSCH transmission.

and

=>	Only the first transmission in a repetition bundle (re)starts configuredGrantTimer

According to the agreements, the configuredGrantTimer should be started/restarted upon PUSCH transmission using configured grant or upon the reception of a dynamic UL grant whose associated HARQ process is a HARQ process preconfigured for a configured grant.

Some remaining issues, such as how to set the values of the configuredGrantTimer, are still not entirely settled. This paper discusses these remaining issues.

# Discussion

## Unit and values of configuredGrantTimer

For the configuredGrantTimer values, it is helpful to look at the agreed periodicities of configured grants. In RAN1-90bis, there are the following agreements regarding the periodicities of transmission occasions of a configured grant.

Agreements:

- At least support following periodicities of resources for UL transmission without UL grant

  - FFS other values with taking into account the alignment with 14 symbols

| Subcarrier spacing (kHz) | Supported periodicities [ms] |
| --- | --- |
| 15 | 2 symbols, 7 symbols, 1, 2, 5, 10, 20, 32, 40, 64, 80, 128, 160, 320, 640 |
| 30 | 2 symbols, 7 symbols, 0.5, 1, 2, 5, 10, 20, 32, 40, 64, 80, 128, 160, 320, 640 |
| 60 | 2 symbols, 7 symbols (6 symbols for ECP), 0.25,0.5,1,2,5,10,20, 32, 40, 64, 80, 128, 160, 320, 640 |
| 120 | 2 symbols, 7 symbols, 0.125,0.25,0.5,1,2,5,10,20, 32, 40, 64, 80, 128, 160, 320, 640 |

In TS 38.321-f00, the periodicity parameter is in SPS-Config IE. The gNB can configure the periodicity of a configured grant for a UE considering the configured numerology for the UE.

priodicity			ENUMERATED {sym2, sym7, ms0dot125, ms0dot25, ms0dot5, ms1, ms2, ms5, ms10, ms20,

ms32, ms40, ms64, ms80, ms128, ms160, ms320, ms640}				OPTIONAL,

For each numerology, there are extreme small periodicities in 2 or 7 OFDM symbols and larger periodicities in number of milliseconds. The configured grant with small periodicity can be configured for low latency services such as URLLC, while configured grant with large periodicities can be configured with more delay tolerant services such as VoIP or community APP service.

The possible periodicity of a configured grant spreads from a number of OFDM symbols to 640 milliseconds.

When a configured grant with small periodicity is configured for a service with low latency requirement, a short HARQ round trip time may also be desired to enable timely HARQ retransmission upon HARQ transmission failure. The configuredGrantTimer, which is used to prevent overuse of a HARQ process, can be set to equal or larger than the desired HARQ round trip time. A too large value for the configuredGrantTimer should also be avoided due to the HARQ process utilization efficiency can be reduced.

In another aspect, the gNB may want to increase the utility of each HARQ process upon lack of HARQ processes by configuring configuredGrantTimer with a small value so that the UE can reuse the HARQ process more frequently. As cost, the gNB should schedule the retransmissions eatly if there is HARQ transmission failure.

Based on such understanding, it can be observed that:

The configuredGrantTimer value has certain relevance with the configured grant periodicity and the availability of HARQ processes configured for configured grant.

From a standardization effort perspective, it is inefficient to study the configuredGrantTimer values in detail for each numerology and/or each periodicity setting respectively. It is thus preferred to define time units and a list of candidate values so that the gNB can always configure an optimal value for the configuredGrantTimer.  With this, it seems that a good way to define the counting time units of the configuredGrantTimer is in relation to the periodicity of the configured grant.

configuredGrantTimer value unit is OFDM symbol if the configured grant periodicity is in number of OFDM symbols.

configuredGrantTimer value unit is slot if the configured grant periodicity is in number of milliseconds.

A candidate value list is further proposed below:

configuredGrantTimer value list can be down select from [1, 2, 4, 8, 12, 16, 20, 24, 28, 32, 40, 64, 80, 128, 160, 320, 640].

# Conclusion

In section 2 we made the following observations:

Observation 1	The possible periodicity of a configured grant spreads from a number of OFDM symbols to 640 milliseconds.

Observation 2	The configuredGrantTimer value has certain relevance with the configured grant periodicity and the availability of HARQ processes configured for configured grant.

Based on the discussion in section 2 we propose the following:

Proposal 1	configuredGrantTimer value unit is OFDM symbol if the configured grant periodicity is in number of OFDM symbols.

Proposal 2	configuredGrantTimer value unit is slot if the configured grant periodicity is in number of milliseconds.

Proposal 3	configuredGrantTimer value list can be down select from [1, 2, 4, 8, 12, 16, 20, 24, 28, 32, 40, 64, 80, 128, 160, 320, 640].