3GPP TSG-RAN WG2 Meeting #101                                                     				           R2-1803535

Athens, Greece, 26th February - 2nd March 2018

Agenda item:	10.3.4.2

Source:	CMCC

Title:	Shorter QFI in SDAP header

Document for:      Discussion

Introduction

According to current proceedings of 3GPP, both SA2 and RAN2 adopt the concept of QFI (QoS flow ID) to distinguish QoS flows. The related agreements are listed as follows:

-     a)RAN2 aims at designing a 1 byte SDAP header.

-     b) If configured, SDAP header size for a DRB is static (assuming 1 byte header).  The QFI will always be present.

- FFS for bullets a) and b) whether it might be possible to use a shorter id over the radio compared to that received from the CN. This is a stage 3 issue.

=>	From RAN2 perspective supporting up to 64 reflective flows per PDU session per UE is sufficient at the same time, so 6 bits QFI in SDAP.

FFS if final QFI in CT1/SA2 is larger than 6 bits, a mechanism to remap NAS QFI to AS QFI may be needed

In this contribution, we provide considerations on the design of shorter QoS flow ID with a mapping mechanism based on current agreements.

Discussion

Necessity of introducing a shorter QFI in SDAP

Since multiple QoS flows can be mapped to one DRB, thus, SDAP requires an ID to distinguish different QoS flows. According to last several meetings, it is indicated that each SDAP PDU should be transmitted with an ID which distinguishes QoS flows when mapping multiple QoS flows to one DRB.

The number of QoS flows is 7bits which has been determined in SA2. When 5G QoS characteristics is extended or category with more elaborated granularity, the bits number may be increased. However the length of SDAP header is limited to 1 byte, SDAP header can not carry the RQI and RDI separately without any compression.    Shorter QFI over the radio compared to that received from the CN should be considered to decrease the overhead of QFI in SDAP header.

Proposal 1:  Shorter QFI should be considered to decrease the overhead of QFI in SDAP header.

The mapping from QFI to the shorter QFI

The isolation of different DRBs implies that it has already been enough to distinguish QoS flows within each DRB. More specifically, the QoS flow ID can be potentially indicated by two indices: a DRB index and an intra-DRB index (or ID). For each DRB, the number of QoS flows mapped is no more than the total number of QoS flows. Therefore, the length of intra-DRB ID indicating QoS flow is not longer than QoS flow ID in CN.  As a result, this intra-DRB ID can be used as the shorter QFI at SDAP header instead of QoS flow ID provided by CN for overhead reduction.

Note that the length of shorter QFI is dependent on the maximum number of QoS flow allowed concurrently mapping to one DRB, which could be a unified integer configured by RRC when DRB establishes. The parameter setting of shorter QFI length should consider the maximum number of DRBs and maximum number of QoS flows. Shorter QFI length should be the minimum integer n which satisfies that 2^n is equal to or larger than the maximum number of QoS flows mapping to each DRB.

The mapping of QoS flow ID and shorter QFI should be configured through RRC signalling, and both of the transmitting and the receiving side should keep the same mapping relationship. The shorter QFI can only be recognised by SDAP layer and does not affect QoS flow ID in CN. Fig.1 shows a transmission procedure in which the length of shorter QFI could be the same for different DRBs. As long as the number of QoS flows doesn’t reach the maximum number, the mapping relationship should be updated once establishing new QoS flows. Table.1 gives an example on how the mapping mechanism works, in which the length of QFI is 7 bits and the length of shorter QFI is 6 bits.

Fig.1 QoS flow transmission procedure

Table.1 Mapping from QoS flow ID to shorter QFI in SDAP

| QFI (QoS flow ID) | shorter QFI |
| --- | --- |
| 0000001 | 000001 |
| 0001001 | 000011 |
| 0010110 | 001001 |
| 0101011 | 001101 |
| … | … |
| … | … |

Proposal 2: Introduce shorter QFI in SDAP to uniquely indentify QoS flows for each DRB. The mapping of QoS flow ID and shorter QFI should be configured through RRC signalling.

Format design for the shorter QFI in SDAP

According to the agreement at RAN2#98, stating that RAN2 will support a mode in which SDAP header is not present and is configured per DRB. Based on the discussion above, two types of SDAP PDU should be defined, which are illustrated in Fig.2 and Fig.3. Fig.2 shows a possible design of SDAP PDU in transparent mode, which only consists of Data field without SDAP header. Fig.3 shows a possible design of SDAP PDU in non-transparent mode, in which shorter QFI should be needed, and Reflective QoS Indication (RQI) could also be required. Note that whether to use SDAP PDU without or with shorter QFI depends on the network configuration.

Fig.2 SDAP PDU without shorter QFI

Fig.3 An example SDAP PDU with shorter QFI

Proposal 3: Define two types of SDAP PDU ━ “SDAP PDU without header” and “SDAP PDU with shorter QFI”.

Conclusions

In this contribution, we propose an optimization mechanism for SDAP header. The observation and proposals is listed as below:

Proposal 1:  Shorter QFI should be considered to decrease the overhead of QFI in SDAP header.

Proposal 2: Introduce shorter QFI in SDAP to uniquely indentify QoS flows for each DRB. The mapping of QoS flow ID and shorter QFI should be configured through RRC signalling.

Proposal 3: Define two types of SDAP PDU ━ “SDAP PDU without header” and “SDAP PDU with shorter QFI”.

Reference