3GPP TSG-RAN WG2 Meeting #101 	R2-1803553

Athens, Greece, 26th February – 2nd March 2018

Agenda Item:

Source:	Ericsson

Title:	Multiplicity definitions in 38331 (E359)

Document for:	Discussion, Decision

# Introduction

In this document, we discuss some issues on constants in TS38.331.

# Discussion

## Move constant definitions to where they are used

By tradition, RRC specifications have collected ASN.1 constants (maxNrofXxxx) into a common section “Multiplicity and type constraint definitions”. This decouples the definition of the constant from the context of where the constant is used. This has decreased the readability of the specification, having to look at different places in the specification.

Example:

-- ASN1START

-- TAG-REPORT-CONFIG-ID-START

ReportConfigId ::=	INTEGER (1..maxReportConfigId)

maxReportConfigId 	INTEGER ::= 32

-- TAG-REPORT-CONFIG-ID-STOP

-- ASN1STOP

We therefore propose that constant definitions are placed together with the asn.1 definitions where they are used.

Move constant definitions to the part (IE definition) in TS 38331 where the constant is used.

Often, there is only a single use case of a constant, e.g. to set the upper limit of a list. Then, the constant definition can preferably be omitted, and the upper limit is specified directly in the SIZE statement.

One advantage of using a maxNrofXxxx constant is to secure e.g. use of same limit in multiple lists in different parts of the ASN.1. We do not this is a real conflict with the advantage of having the constant definition close to its use.  It is also possible in a later release to introduce a maxNrofXxxx definition, e.g. in case a new list has the same size as an existing list, without compatibility issues.

Delete the constant definitions that are only used once in the ASN.1, and define the limit directly e.g. in the SIZE statement.

## ServCellIndex range vs maxServingCells

The following agreement is currently not correctly covered in TS38331:

Agreements

1:	 The MN provides the SN the range of SCellIndex to be used by signalling a start value and a stop value if the range. 32 SCellIndex range is used between the MN and the SN. The UE uses “Dual Connectivity PHR MAC Control Element supporting 32 serving cells with configured uplink” for PH reporting.

TS38.331 currently reads:

#### ServCellIndex

The IE ServCellIndex concerns a short identity, used to identify a serving cell (i.e. the PCell or an SCell). Value 0 applies for the PCell, while the SCellIndex that has previously been assigned applies for SCells.

ServCellIndex information element

-- ASN1START

-- TAG-SERV-CELL-INDEX-START

ServCellIndex ::=    INTEGER (0..maxNrofServingCells-1)

-- TAG-SERV-CELL-INDEX-STOP

-- ASN1STOP

But maxNrofServingCells reflects the number of cells in a cell group (16).

Use 31 as upper limit on ServcellIndex (directly in IE definition)

Rename maxNrofServingCells to maxNrofServingCellsInCG

## Delete “…Nrof…”

In many cases, “…Nrof…” can be removed from names of constants.

Remove “…Nrof…” from names of several constants.

## Allocating values to constants

Many maxNrofXxx values are missing in TS 38331.

In Appendix, we provide an overview, and propose values on some.

The following should be noted:

- In many cases, the maxNrofXxxx sets the limits on what is possible to convey via RRC signalling. Additional system limits sets the “real” requirements, e.g. on RRM

- To avoid future addition of new lists to “extend” and existing list (e.g. from 256 to 512 list elements) it could in some cases be wise to from the beginning set a very high list size, but limit the actual used size in field description.

RAN2 to discuss and agree on remaining constant values

# Conclusion

In this document, we have raised some issues on use of constants in TS38.331. be avoided.

We ask RAN2 to discuss and agree on the following proposals:

- Move constant definitions to the part (IE definition) in TS 38331 where the constant is used.

- Delete the constant definitions that are only used once in the ASN.1, and define the limit directly e.g. in the SIZE statement.

- Use 31 as upper limit on ServcellIndex (directly in IE definition).

- Rename maxNrofServingCells to maxNrofServingCellsInCG

- Remove “…Nrof…” from names of several constants.

- RAN2 to discuss and agree on remaining constant values.

# References

[1] 	TS 38.331

Appendix

| Area | Name | Current spec value | Proposed value | Description from 38.331 | Comments |
| --- | --- | --- | --- | --- | --- |
| L1 | maxCoReSetDuration | 3 |  | -- Max number of OFDM symbols in a control resource set |  |
| L1 | maxEARFCN | 262143 |  | -- Highest value of extended E-UTRA EARFCN range |  |
| L1 | maxINT-DCI-PayloadSize |  |  | -- Max number payload of a DCI scrambled with INT-RNTI | RAN1 information needed. |
| L1 | maxINT-DCI-PayloadSize-1 |  |  | -- Max number payload of a DCI scrambled with INT-RNTI minus 1 | RAN1 information needed. |
| L1 | maxNrofBWPs | 4 |  | -- Maximum number of BWPs per serving cell |  |
| L1 | maxNrofControlResourceSets | 12 |  | -- Max number of CoReSets configurable on a serving cell |  |
| L1 | maxNrofControlResourceSets-1 | 11 |  |  | RAN1 information needed. |
| L1 | maxNrofCSI-IM-Resources |  |  | -- Maximum number of CSI-IM resources. See CSI-IM-ResourceMax in 38.214. | RAN1 information needed. |
| L1 | maxNrofCSI-IM-Resources-1 |  |  | -- Maximum number of CSI-IM resources minus 1. See CSI-IM-ResourceMax in 38.214. | RAN1 information needed. |
| L1 | maxNrofCSI-IM-ResourcesPerSet |  |  | -- Maximum number of CSI-IM resources per set. See CSI-IM-ResourcePerSetMax in 38.214 | RAN1 information needed. |
| L1 | maxNrofCSI-MeasId |  |  | -- Maximum number of link configurations | RAN1 information needed. |
| L1 | maxNrofCSI-MeasId-1 |  |  | -- Maximum number of link configurations minus 1 | RAN1 information needed. |
| L1 | maxNrofCSI-ResourceConfigurations |  |  | -- Maximum number of resource configurations | RAN1 information needed. |
| L1 | maxNrofCSI-ResourceConfigurations-1 |  |  | -- Maximum number of resource configurations minus 1 | RAN1 information needed. |
| L1 | maxNrofCSI-ResourceSets |  |  | -- Maximum number of resource sets per resource configuration | RAN1 information needed. |
| L1 | maxNrofCSI-ResourceSets-1 |  |  | -- Maximum number of resource sets per resource configuration minus 1 | RAN1 information needed. |
| L1 | maxNrofCSI-ResrouceConfigurations |  |  |  | RAN1 information needed. |
| L1 | maxNrofCSI-RS |  |  |  | RAN1 information needed. |
| L1 | maxNrofCSI-RS-ResourcesPerSet | 8 |  | - Maximum number of CSI-RS resources per resource set |  |
| L1 | maxNrofDL-Allocations |  |  |  | RAN1 information needed. |
| L1 | maxNrofNZP-CSI-RS-Resources-1 |  |  | -- Maximum number of Non-Zero-Power (NZP) CSI-RS resources minus 1 | RAN1 information needed. |
| L1 | maxNrofP0-PUSCH-AlphaSets | 30 |  | -- Maximum number of P0-pusch-alpha-sets (see 38,213, section 7.1) |  |
| L1 | maxNrofP0-PUSCH-AlphaSets-1 | 29 |  | -- Maximum number of P0-pusch-alpha-sets minus 1 (see 38,213, section 7.1) |  |
| L1 | maxNrofPCIsPerSMTC |  |  |  | RAN1 information needed. |
| L1 | maxNrofPhysicalResourceBlocks | 275 |  | -- Maximum number of PRBs |  |
| L1 | maxNrofPhysicalResourceBlocks-1 | 274 |  | -- Maximum number of PRBs |  |
| L1 | maxNrofPUCCH-P0-PerSet | 8 |  | -- Maximum number of P0-pucch present in a p0-pucch set |  |
| L1 | maxNrofPUCCH-PathlossReferenceRSs | 4 |  | -- Maximum number of RSs used as pathloss reference for PUCCH power control. |  |
| L1 | maxNrofPUCCH-PathlossReferenceRSs-1 | 3 |  | -- Maximum number of RSs used as pathloss reference for PUCCH power control minus 1. |  |
| L1 | maxNrofPUCCH-ResourceSets | 4 |  | -- Maximum number of PUCCH Resource Sets |  |
| L1 | maxNrofPUCCH-ResourceSets-1 | 3 |  | -- Maximum number of PUCCH Resource Sets minus 1. |  |
| L1 | maxNrofPUCCH-ResourcesPerSet | 8 |  | -- Maximum number of PUCCH Resources per PUCCH-ResourceSet |  |
| L1 | maxNrofPUCCH-ResourcesPerSet-17 |  |  |  | RAN1 information needed. |
| L1 | maxNrofPUSCH-PathlossReferenceRSs | 4 |  | -- Maximum number of RSs used as pathloss reference for PUSCH power control. |  |
| L1 | maxNrofPUSCH-PathlossReferenceRSs-1 | 3 |  | -- Maximum number of RSs used as pathloss reference for PUSCH power control minus 1. |  |
| L1 | maxNrofQFIs |  |  |  | RAN1 information needed. |
| L1 | maxNrofRateMatchPatterns | 4 |  | -- Max number of rate matching patterns that may be configured |  |
| L1 | maxNrofRateMatchPatterns-1 | 3 |  | -- Max number of rate matching patterns that may be configured minus 1 |  |
| L1 | maxNrofSearchSpaces | 40 |  | -- Max number of Search Spaces |  |
| L1 | maxNrofSearchSpaces-1 | 339 |  | -- Max number of Search Spaces minus 1 |  |
| L1 | maxNrofSlotFormatCombinationsPerSet |  |  | -- Maximum number of Slot Format Combinations in a SF-Set. | RAN1 information needed. |
| L1 | maxNrofSlotFormatCombinationsPerSet-1 |  |  | -- Maximum number of Slot Format Combinations in a SF-Set minus 1. | RAN1 information needed. |
| L1 | maxNrofSlotFormatsPerCombination |  |  |  | RAN1 information needed. |
| L1 | maxNrofSlots | 320 |  | -- Maximum number of slots in a 10 ms period |  |
| L1 | maxNrofSlots-1 | 319 |  | -- Maximum number of slots in a 10 ms period minus 1 |  |
| L1 | maxNrofSpatialRelationInfos |  |  |  | RAN1 information needed. |
| L1 | maxNrofSR-ConfigPerCellGroup | 8 |  | -- Maximum number of SR configurations per cell group | RAN1 information needed. |
| L1 | maxNrofSR-Resoruces |  |  |  | RAN1 information needed. |
| L1 | maxNrofSRS-Resources |  |  | -- Maximum number of SRS resources in an SRS resource set. | RAN1 information needed. |
| L1 | maxNrofSRS-Resources-1 |  |  | -- Maximum number of SRS resources in an SRS resource set minus 1. | RAN1 information needed. |
| L1 | maxNrofSRS-ResourceSets |  |  | -- Maximum number of SRS resource sets. | RAN1 information needed. |
| L1 | maxNrofSRS-ResourceSets-1 |  |  | -- Maximum number of SRS resource sets minus 1. | RAN1 information needed. |
| L1 | maxNrofSRS-ResourcesPerSet |  |  |  | RAN1 information needed. |
| L1 | maxNrofSRS-TriggerStates-1 | 3 |  | -- Maximum number of SRS trigger states minus 1, i.e., the largest code point. |  |
| L1 | maxNrofSSB-Resources-1 | 63 |  | -- Maximum number of SSB resources in a resource set minus 1 |  |
| L1 | maxNrofSSBs |  |  |  | RAN1 information needed. |
| L1 | maxNrofSymbols-1 | 13 |  | - Maximum index identifying a symbol within a slot (14 symbols, indexed from 0..13) |  |
| L1 | maxNrofTCI-States | 64 |  |  |  |
| L1 | maxNrofTCI-States-1 | 63 |  |  |  |
| L1 | maxNrofTCI-StatesPDCCH |  |  |  | RAN1 information needed. |
| L1 | maxNrofUL-Allocations |  |  |  | RAN1 information needed. |
| L1 | maxNrofZP-CSI-RS-Resources | 3 |  | -- Maximum number of Zero-Power (NZP) CSI-RS resources |  |
| L1 | maxNrofZP-CSI-RS-Resources-1 | 2 |  | -- Maximum number of Zero-Power (NZP) CSI-RS resources minus 1 |  |
| L1 | maxQFI |  |  |  | RAN1 information needed. |
| L1 | maxRA-CSIRS-Resources |  |  |  | RAN1 information needed. |
| L1 | maxRA-SSB-Resources |  |  |  | RAN1 information needed. |
| L1 | maxSCSs |  |  |  | RAN1 information needed. |
| L1 | maxSFI-DCI-PayloadSize |  |  | -- Max number payload of a DCI scrambled with SFI-RNTI | RAN1 information needed. |
| L1 | maxSFI-DCI-PayloadSize-1 |  |  | -- Max number payload of a DCI scrambled with SFI-RNTI minus 1 | RAN1 information needed. |
| L2 | maxCellSCG |  | - |  | Same as maxNrofSCells. Can be deleted. |
| L2 | maxDRB |  | 16 | Draft: Max mumber of DRBs | Currently, this sets the max number of DRBs that can be added/modified/deleted. Proposal: 16 |
| L2 | maxLCG-ID | 7 |  | -- Maximum value of LCG ID |  |
| L2 | maxLCH |  | - |  | Currently, this sets the max number of LCHs (RLC Bearers) that can be added/modified/deleted. Deleted in email discussion #5, since same as maxLC-ID. |
| L2 | maxLC-ID |  | 20 | -- Maximum value of Logical Channel ID | Proposed to be set to “max number of DRBs” + “max number of SRBs” = 16 + 4 = 20 |
| L2 | maxNrofTAGs | 4 |  | -- Maximum number of Timing Advance Groups |  |
| L2 | maxNrofTAGs-1 | 3 |  | -- Maximum number of Timing Advance Groups minus 1 |  |
| UE Cap | maxBandComb |  |  | - Maximum number of DL band combinations | Need further RAN2 discussion |
| UE Cap | maxBands |  |  |  | Need further RAN2 discussion |
| UE Cap | maxBasebandProcComb |  |  | -- Maximum number of base band processing combinations | Need further RAN2 discussion |
| UE Cap | maxRAT-CapabilityContainers |  | 8 | -- Maximum number of interworking RAT containers (incl NR and MRDC) | Need further RAN2 discussion |
| UE Cap | maxSecondaryCellGroups |  | 3 |  | Currently, defines the max value of CellGroupID. Proposal: rename to maxCellGroupID (3), meaning values 0-3 are used (4 Cell Groups |
| UE Cap | maxSimultaneousBands |  | 32 | -- Maximum number of simultaneously aggregated bands | Defines the max number of cells (component carriers) in a CA/DC band combination. In LTE, value is used, although 32 is the upper limit after Rel-13 B5C work item. There can be max 16 cells in a cell group. |
| Cell | maxNrofSCells | 15 |  | -- Max number of secondary serving cells per cell group |  |
| Cell | maxNrofServingCells | 16 |  | -- Max number of serving cells (SpCell + SCells) per cell group |  |
| Cell | maxNrofServingCells-1 | 15 |  | -- Max number of serving cells (SpCell + SCells) per cell group minus 1 |  |
| RRM | maxCellPrep |  | 32 |  | In LTE, the corresponding parameter was maxReestabInfo (32). Only used in inter-node message HandoverPreparationInformation. Proposal: - Reuse LTE name and value (32) - use constant local in Inter-node module |
| RRM | maxCellReport |  |  |  |  |
| RRM | maxFreq |  | 8 | DRAFT: Max number of non-serving frequencies possible to include in MeasResultSCG-Failure. | Rename to maxNonServFreqReport, or(preferred) include directly in SIZE definition. |
| RRM | maxNrofAggregatedCellsPerCellGroup |  | 16 | This is related to L1 parameter. Can use same value as maxNrofServingCells (16). | This is related to L1 parameter. Can use same value as maxNrofServingCells (16), or (preferred) reuse this constant. |
| RRM | maxNrofCandidateBeams |  | 16 |  | Defines the max number of PRACH-ResourceDedicatedBFR that can be signalled in BFR config. Expected RAN4 defines performance requirement. RAN1/RAN4 feedback needed. |
| RRM | maxNrofCellMeas |  | 32 | -- Maximum number of entries in each of the cell lists in a measurement object. | 32 is used in LTE. Same can be used here |
| RRM | maxNrofCSI-ReportConfig-1 |  |  |  | Need further RAN2 discussion |
| RRM | maxNrofCSI-Reports |  |  | -- Maximum number of report configurations | Need further RAN2 discussion |
| RRM | maxNrofCSI-RS-CellsRRM |  |  | -- Maximum number of FFS | Need further RAN2 discussion |
| RRM | maxNrofCSI-RS-ResourcesRRM |  | 96 | -- Maximum number of CSI-RS resources for an RRM measurement object | Change CSI-RS-ResourceId-RRM ::= INTEGER (0..maxNrofCSI-RS-ResourcesRRM-1) To CSI-RS-ResourceId-RRM ::= INTEGER (1..maxNrofCSI-RS-ResourcesRRM) |
| RRM | maxNrofCSI-RS-ResourcesRRM-1 |  | - | -- Maximum number of CSI-RS resources for an RRM measurement object minus 1 | Delete. See above |
| RRM | maxNrofCSI-RS-ResourcesToAverage |  |  | -- Max number for the (max) number of CSI-RS to average to determine cel measurement | Need further RAN2 discussion |
| RRM | maxNrofIndexesToReport |  |  |  | Need further RAN2 discussion |
| RRM | maxNrofMeasId |  | 64 | -- Maximum number of configured measurements | 64 is used in LTE |
| RRM | maxNrofObjectId |  | 64 | -- Maximum number of configured measurement objects | 64 is used in LTE |
| RRM | maxNrofPCI-Ranges |  | 8 | -- Maximum number of PCI ranges | In LTE, 4 is used for UTRA PCI ranges |
| RRM | maxNrofQuantityConfig | 2 |  | -- Maximum number of quantity configurations |  |
| RRM | maxNrofReportConfigIdsPerTrigger | 16 |  | -- Maximum number of report configurations per reportTrigger |  |
| RRM | maxNrofSS-BlocksToAverage |  |  | -- Max number for the (max) number of SS blocks to average to determine cell measurement | Need further RAN2 discussion |
| RRM | maxReportConfigId |  | 64 | -- Maximum number of reporting configurations | 32 is used in LTE |
| RRM | maxNrofFailureDetectionResources |  |  | -- Maximum number of failure detection resources | Need further RAN2 discussion |