3GPP TSG-RAN WG2 #101	R2-1803619

Athens, Greece, 26th February – 2nd March 2018

Agenda Item:	10.4.4.3 – NR – Stage-3-CP –Temporary Restrictions

Source:	Qualcomm Incorporated

Title:	Temporary Capability Restriction for standalone NR

Document for:	Discussion, Decision

# Introduction

TS 38.300 states the RAN2#97 meeting agreement , UE access capability can be divided into two parts: static capability and temporary capability restrict. Only static capability is stored in NG core and capability temporary restrict is unseen to the NG core.

7.5	UE Capability Retrieval framework

The UE reports its UE radio access capabilities which are static at least when the network requests. The gNB can request what capabilities for the UE to report based on band information.

When allowed by the network, a temporary capability restriction request maybe sent by the UE to signal the limited availability of some capabilities (e.g. due to hardware sharing, interference or overheating) to the gNB. The gNB can then confirm or reject the request. The temporary capability restriction should be transparent to 5GC. Namely, only static capabilities are stored in 5GC.

In this paper, we propose to agree on further details implementing the text in bold above and also show how these capability restrictions can also be used in SA during hardware sharing, interference or overheating.

.

# Capability Scaling

As per the RAN2 agreements, the UE shall be able to send a “temporary capability restriction request to the gNB”. We propose a scheme where the UE capabilities are categorized into groups as represented graphically in Figure 1.

Figure 1: UE capability groups

Two types of capabilities are proposed.

- The baseline capabilities: Consists of the feature set that is always available, e.g. L2 feature capabilities.

- The set capabilities: Includes a set of groups in which a single group consists of a feature set that can be used simultaneously, e.g. UE categories, CA band combinations, MIMO and Modulation schemes.

For temporary capability restriction, one or multiple sets can be temporarily restricted. The UE can signal the available groups at a given time to the network. In Figure 2, the groups 1, 3, 5 and 7 are signaled as available, reflecting a temporary capability restriction in the UE due to hardware sharing, interference or overheating.

Figure 2: Reduction of the UE capability groups

Below call flow shows the message exchange between UE and gNB during capability restriction. UE during capability enquiry can send the static/baseline capabilities which the UE will always support along with the set capabilities . On detection of overheating, interference, hardware sharing, UE will send the temporary capability group to the gNB to restrict the UE capabilities according to the dynamic capability.

5GC

UE

gNB

UE Capability Enquiry

gNB forwards UE capabilities to core N/W

UE Capability Information

(NR static capabilities)

Temporary Capability Restriction Request

(Capability Group)

RRC Connection Reconfiguration

gNB stores the dynamic UE capabilities and reconfigures UE based on capabilities

Prohibit

timer

RRC Connection Reconfiguration Complete

UE can revert to full capability after prohibit timer expiry if the overheating condition is alleviated

Figure 3:  Temporary capability restriction in SA mode

Observation 1: Temporary capability restriction is needed to alleviate overheating condition caused due to higher data configurations concerning carrier aggregation/dual connectivity, MIMO transmissions, and/or modulation schemes.

Proposal 1: The UE shall support a set of baseline capabilities that are always supported and shall be able to group its other capabilities to enable temporarily restricting its capabilities.

Proposal 2: The UE may revert to a super-set of these restricted capabilities only after expiry of a prohibit timer configured by the network.

# Conclusion

Based on the discussion in section 2 we propose the following:

Observation 1: Temporary capability restriction is needed to alleviate overheating condition caused due to higher data configurations concerning carrier aggregation/dual connectivity, MIMO transmissions, and/or modulation schemes.

Proposal 1: The UE shall support a set of baseline capabilities that are always supported and shall be able to group its other capabilities to enable temporarily restricting its capabilities.

Proposal 2: The UE  may revert to a super-set of these restricted capabilities only after expiry of a prohibit timer configured by the network.