3GPP TSG-RAN WG2 #101	R2-1803626

26th February – 2nd March 2018

Athens, Greece

Source:                    	NTT DOCOMO, INC.

Title:  	[D309/310/311] Sub-carrier spacing for Msg.1/2/3

Document for:        	Discussion and decision

Agenda Item:         	10.4.3.2	ASN.1 issue document

- Introduction

According to the latest ASN.1 structure on RACH-ConifigCommon/Dedicated (see Annex A in this paper), sub-carrier spacing (SCS) are configured for some messages as listed below

Contention-based RA (CBRA):

-	Msg.1;

-	Msg.3.

Contention-free RA (CFRA):

-	Msg.1;

-	Msg.2.

This paper discusses the necessity of dedicated SCS setting for each message.

- Discussion

In the current ASN.1 structure, SCS is configured in severals IEs as listed below. The whole picture of ASN.1 structure from ServingCellConfig to BWP can be found in [1].

-	ServingCellConfigCommon (SCS common and SCS for SSB);

-	SCS-SpecificCarrier (both for DL and UL);

-	TDD-UL-DL-ConfigCommon/Common2;

-	DownlinkBWP-Common;

-	UplinkBWP-Common.

Althogh there is another issue whether SCS has to be configured for all these levels or not (see [1]), this paper assumes that a UE can obtain SCS per BWP upon receiving all the above configurations.

With regards to Msg.3 for CBRA, RACH-ConfigCommon can be setup and released in UplinkBWP-Common. SCS can also be present in UplinkBWP-Common. In that case, it is likely that SCS for Msg.3 in RACH-ConfigCommon is the same as the one in UplinkBWP-Common. As such, SCS for Msg.3 can be inferred from the SCS in UplinkBWP-Common.

With regards to Msg.2 for CFRA, RACH-ConfigDedicated can be delivered together with ServingCellConfigCommon in reconfigurationWithSync. ServingCellConfigCommon can also include DownlinkBWP-Common which also includes SCS. Alternarively, subcarrierSpacingCommon in ServigCellConfigCommon can be utilised, which is intended for SIB1, Msg.2/4 for initial access and SI-messages. Therefore, there is a clue that the UE can learn SCS for Msg.2/3 even without configuring explicitly in RACH-Config. In fact, this approach can be leveraged for PDCCH ordered CFRA for which there is no RRC configuration in advance. Thus, the following is proposed:

Proposal 1:	Remove msg3-SubcarrierSpacing from RACH-ConfigCommon and cfra-msg2-SubcarrierSpacing from RACH-ConfigDedicated (as SCS for Msg.2/3 can be obtained from the other IEs).

Proposal 2:		Msg.2 SCS for PDCCH ordered CFRA follows the one for CBRA.

With regards to Msg.1 for CBRA, since the SCS for Msg.1 CBRA can be different from the one for the other data/control channels, explicit signalling is required in RACH-ConfigCommon.

In contrast, the need of explicit signalling of Msg.1 SCS for CFRA is worthwhile discussing. One approach is that Msg.1 SCS for CFRA is the same as the one for CBRA. In this case, explicit signallingis not needed in RACH-ConfigDedicated. On the other hand, there could be a scenario that Msg.1 SCS for CFRA is different from the one for CBRA, e.g. on the target cell for handover/PSCell change. Given that RAN1 has agreed that dedicated RA resource can be assigned for CFRA, e.g. handover and beam failure recovery, it could be a likely scenario. Nevertheless, for PDCCH ordered CFRA, there is no way to indicate SCS to the UE (unless it is indicated in DCI). Subsequently, the followings are proposed:

Proposal 3:		Msg.1 SCS for PDCCH ordered CFRA follows the one for CBRA.

Proposal 4:	Discuss whether Msg.1 SCS for CFRA is always the same as for CBRA or can be different and so Msg.1 SCS for CFRA is explicitly signalled in RACH-ConfigDedicated.

- Summary and proposal

In summary, the followings were proposed:

Proposal 1:	Remove msg3-SubcarrierSpacing from RACH-ConfigCommon and cfra-msg2-SubcarrierSpacing from RACH-ConfigDedicated (as SCS for Msg.2/3 can be obtained from the other IEs).

Proposal 2:		Msg.2 SCS for PDCCH ordered CFRA follows the one for CBRA.

Proposal 3:		Msg.1 SCS for PDCCH ordered CFRA follows the one for CBRA.

Proposal 4:	Discuss whether Msg.1 SCS for CFRA is always the same as for CBRA or can be different and so Msg.1 SCS for CFRA is explicitly signalled in RACH-ConfigDedicated.

- References

[1] R2-1803746, “[D325] Sub-carrier spacing for serving cell/carrier and BWP,” NTT DOCOMO, INC.

Annex A:
ASN.1 structure on RACH-ConfigCommon/Dedicated