3GPP TSG-RAN WG2 #101	R2-1803722

26th February – 2nd March 2018

Athens, Greece

Source:                    	NTT DOCOMO, INC.

Title:  	[D313] Necessity of LTE CRS rate matching for Other SI

Document for:        	Discussion and decision

Agenda Item:         	10.4.3.2	ASN.1 issue document

- Introduction

Currently, all of the PDSCH parameters are configured per BWP(-Dedicated). This paper discusses whether there are PDSCH parameters which needs to be broadcast.

NOTE:	This issue has already been addressed by the rapporteur of email discussion [NR-AH1801#19]. This document may not have to be treated if the rapporteur proposal is agreed.

- Discussion

The RAN1 LS received at this meeting informs that some of the PDSCH parameters are configured per cell. Amongst them, lte-CRS-ToMatchAround is used for the UE to learn where LTE CRS is present for the rate matching purpose. A practical use case for the LTE CRS rate matching is LTE/NR co-existence where LTE and NR is operated on the same carrier. Likewise, rateMatchPattern is used for the future usage in NR. These parameters are included into rateMatchResourcesPDSCH in PDSCH-Config (see Annex 1).

Given that the main usage is co-existence with LTE and future usage/service in NR, PDSCH rate matching would be needed not only for the dedicated signalling to the UE but also for broadcast signalling delivered on PDSCH, i.e. Other SI (OSI). in that sense, a cell-level PDSCH config. needs to be introduced, which are to be broadcast.

For this rate matching purpose another question can be envisaged whether this information is also needed for the other physical channels/signals, e.g. PDCCH, CSI-RS, etc. RAN1 is still discussing whether the rate matching information is needed for the other channels/signals than PDSCH. RAN2 should wait for the RAN1 decision.

- Summary and proposal

Based on the above discussion, the followings are proposed:

Proposal 1:		lte-CRS-ToMatchAround and rateMatchPattern are provided by SIB1.

Proposal 2:	A cell level PDSCH config (e.g. PDSCH-ServingCellConfig) is introduced for the cell level information to broadcast in SIB1.

- References

[1] R2-1801719, “LS on correction to RRC parameters for NR,” RAN1.

Annex A: ASN.1 structure on PDSCH-Config