3GPP TSG-RAN WG2#101	R2-1804083

# Athens, Greece, 26th Feb. – 2nd Mar. 2018					offline discussion #030

Agenda Item:	10.4.4.2

Source:	Qualcomm Incorporated

Title:	Report on #030: Way Forward after R2-1803749

Document for:	Discussion, Decision

# 1. Introduction

Following the presentation of R2-1803749, RAN2 asked for signalling details. Qualcomm has provided the proposed signalling changes below, and a very lively offline discussion evolved around the understanding and merits of each change.

R2-1803749	Avoiding bridges to nowhere	Qualcomm Incorporated	discussion	Rel-15

=>	Qualcomm will provide a conrtibution to show what the signalling details of a solution based on P3, P4 and P5 option 1, so that it can analysed in the group. R2-1803954 (Offline discussion #30)

# 2. Description of the proposals

While providing the proposals to implement R2-1803749, we realized that we can describe our proposal in two parts, and additionally, we realized that we can do two additional enhancements that are related to the LTE capabilities.

The 4 enhancements are graphically depicted in figure 1:

Figure 1 The 4 proposed enhancements

After offline discussions, only enhancement 1 is proposed below.

## 2.1 Supported Band combination level

A band combination-level summary of DL/UL parameters, that can be mappped to a number of NR band combinations.

Within the combination, the RF bands are mapped sequentially to the band capabilities in this “per band combination structure”.

A suboptimization is added to choose between a saved set of parameters, and a direct listing, this is optional to include, but helps for the cases there is only one configuration for that band combination. Then, we don’t incur the cost of an indirection. This is implemented in this ASN.1 code:

Bands-Configuration-List ::= CHOICE {

nr-BandCombination-index		NR-BandCombination-Index,

nr-BandCombination			NR-BandCombination

}

Another suboptimization helps avoid having to list permutations. Bands within a group can be permutted with their default assignment of band configurations.  This is implemented in this ASN.1 code:

frequencyBandGroup					INTEGER(1..4),

## 2.3 Supported Band combination level

A band-level summary of DL and UL parameters, that can be mapped from the band combination structure.

## 2.4 Supported Band combination level

At the highest level, there may be many LTE combinations that work with many NR combinations. This is a many to many association, that can save a lot of redundancy.  This is achieved with this structure:

-- The UE supports any eutra entry with any NR entry in BandCombinationList

MR-DC-BandCombination ::= SEQUENCE {

eutra-BandCombinationList	(SIZE (1.. maxBandComb)) EUTRA-BandCombination,

BandCombinationList  (SIZE (1..maxBandComb)) OF BandCombination

}

## 2.5 Supported Band combination level

There may be existing combinations that can operate with NR. Why not just refer to them? Adjusting the existing band combinations lists to account for NR is a UE option, enhanced if we relax the fallback rules. There is no need to have baseband capabilities for LTE, and short of restructuring all the LTE parameters, then, we propose to link to existing LTE structures, keeping the door open in the 4th choice of the structure to add a new structure that is optimized, like for NR, either in rel-15 or later. This is achieved in this structure:

EUTRA-BandCombination ::= CHOICE {

index-SupportedBandListEUTRA	INTEGER(1..maxBands),

index-ca-list-r10-r11 CHOICE {

index-SupportedBandCombination-r10 INTEGER (1..maxBandComb-r10),

index-SupportedBandCombination-r11 	INTEGER (1..maxBandComb-r11)

},

index-SupportedBandCombination-r13 	INTEGER (1..maxBandComb-r13),

index-SupportedBandCombination-r15 	INTEGER (1..maxBandComb-r15)  --TBD structure, discuss the need for a new one

}

## 2.4 Misc corrections

Incidental/Misc changes and corrections.

# 3. Proposals

Having gone through the problems with current situation, we do want to optimize the size of the UE capabilities, which is the real problem.

After the offline discussions, we propose:

Proposal 1: Implement RAN1/RAN4 type 3 parameters into the “BPC” structure (the exact name can be changed in the next meeting).

Proposal 2: There shall be explicit linking from the RF band combinations to this structure. The relationship is many to many.  In ASN.1, include a set of BPC indices, in each RF band combination.

Proposal 3: Have an email discussion to adopt the corresponding ASN.1 changes.

# Changes to 38.331 ASN.1

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR |  | rev |  | Current version: | 15.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE capabilities Structure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-02-02 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Pink Changes, 1st optimization:A band combination-level summary of DL/UL parameters, that can be mappped to a number of NR band combinations. A suboptimization is added to chose between a saved set of parameters, anda direct listing, this is optional to include, but helps for the cases there is only one configuration for that band combination. Red Optimizatio, 2nd optimization: A band-level summary of DL and UL parameters, that can be mapped from the band combination structure in pink. A subotimization is to avoid repetition of permutations of bands, when all permutations are possible. Green changes, 3rd optimization: At the highest level, there may be many LTE combinations that work with many NR combinations. This is a many to many association, that can save a lot of redundancy. Blue changes, 4th optimization: There may be existing combinations that can operate with NR. Why not ust refer to them? Adjusting the existing band combinations lists to account for NR is a UE option, enhanced if we relax the fallback rules. There is no need to have baseband capabilities for LTE, and short of restructuring all the LTE parameters, then, we propose to link to existing LTE structures, keeping the door open in the 4th choice of the structure to add a new structure that is optimized, like for NR, either in rel-15 or later. Yellow changes: Incidental/Misc changes and corrections. The updates were made on top of R2-1802969, after accepting all changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

The First Change

## 5.6	UE capabilities

### 5.6.1	UE capability transfer

#### 5.6.1.1	General

#### Editor’s Note: Targeted for completion in June 2018.

#### 5.6.1.2	Initiation

Editor’s Note: Targeted for completion in June 2018.

#### 5.6.1.3	Reception of the UECapabilityEnquiry by the UE

Editor’s Note: Targeted for completion in June 2018.

#### 5.6.1.4	Compilation of band combinations supported by the UE

The UE shall:

1>	if FreqBandList is received:

2>	compile a list of band combinations, candidate for inclusion in the UECapabilityInformation message,  only consisting of bands included in FreqBandList, and prioritized in the order of FreqBandList, (i.e. first include remaining band combinations containing the first-listed band, then include remaining band combinations containing the second-listed band, and so on);

2>	for each band combination included in the candidate list:

3>	if it is regarded as a fallback band combination with the same capabilities of another band combination included in the list of candidates as specified in TS 38.306 [xx]:

4>	remove the band combination from the list of candidates.

2>	include all band combinations in the candidate list into supportedBandCombination;

1>	else:

2>	include all band combinations supported by the UE into supportedBandCombination, excluding fallback band combinations with the same capabilities of another band combination included in the list of band combinations supported by the UE;

#### 5.6.1.5	Compilation of band processing combinations supported by the UE

The UE shall:

1>	for each band combination included in supportedBandCombination:

2>	include the band processing combination supported for the band combination into supportedBandCombinationProcessing, unless it is already included;

2>	if there are the fallback band processing combinations of this band processing combination as specified in TS 38.306 [xx] for which supported band capabilities are different from this band processing combination:

3>	include only these band processing combinations into supportedBandCombinationProcessing;

The Next Change

### 6.3.3	UE capability information elements

#### –	BandCombinationList

The IE BandCombinationList contains a list of NR CA and/or MR-DC band combinations.

BandCombinationList information element

-- ASN1START

-- TAG-BAND-COMBINATION-LIST-START

MR-DC-BandCombinationList ::=	SEQUENCE (SIZE (1..maxBandComb)) OF MR-DC-BandCombination

-- The UE supports any eutra entry with any NR entry in BandCombinationList

MR-DC-BandCombination ::= SEQUENCE {

eutra-BandCombinationList	(SIZE (1.. maxBandComb)) EUTRA-BandCombination,

BandCombinationList 	 (SIZE (1..maxBandComb)) OF BandCombination

}

EUTRA-BandCombination ::= CHOICE {

index-SupportedBandListEUTRA	INTEGER(1..maxBands),

index-ca-list-r10-r11 CHOICE {

index-SupportedBandCombination-r10 INTEGER (1..maxBandComb-r10),

index-SupportedBandCombination-r11 	INTEGER (1..maxBandComb-r11)

},

index-SupportedBandCombination-r13 	INTEGER (1..maxBandComb-r13),

index-SupportedBandCombination-r15 	INTEGER (1..maxBandComb-r15)  --TBD structure, discuss the need for a new one

}

IMPORTS

maxBands,

maxBandComb-r10,

maxBandComb-r11,

maxBandComb-r13

FROM EUTRA-RRC-Definitions;

-- Any element of bands-List can be mapped to any entry in bands-Configuration-List, following the rule included

BandCombination ::= SEQUENCE {

bands-List				(SIZE(1..maxBandComb)) OF BandList,

bands-Configuration-List	(SIZE(1.. maxBandComb)) OF Bands-Configuration-List

}

--Subotimization of the 1st enhancement

Bands-Configuration-List ::= CHOICE {

nr-BandCombination-index		NR-BandCombination-Index,

nr-BandCombination			NR-BandCombination

}

NR-BandCombination-Index :: = INTEGER(1..maxBandComb)

-- nr-BandCombination-index indices SupportedDLParametersPerBandCombination in order of appearance in the list:

SupportedParametersPerBandCombination ::= SEQUENCE (SIZE (1..maxBandComb)) OF ParametersPerBandCombination

NR-BandCombination ::= SEQUENCE {

bandParametersDL				SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParametersDL			OPTIONAL  -- Not included in case of SUL

bandCombinationsUL					BIT STRING (SIZE (1.. maxBandCombUL))   OPTIONAL,

bandCombinationULParametersList		SEQUENCE (SIZE (1..maxBandCombUL)) OF BandCombinationULParameters,

bandCombinationDLParametersList		BandCombinationDLParametersList

}

-- Bandparameters

BandList ::= (SIZE (1..maxSimultaneousBands)) OF BandAndBCS

BandAndBCS ::= SEQUENCE {

frequencyBand						FreqBandInformation,

frequencyBandGroup					INTEGER(1..4),

supportedBandwidthCombinationSet		SupportedBandwidthCombinationSet

}

bandParametersDL				BandParametersDL			OPTIONAL  -- Not included in case of SUL

-- Per Band combination DL parameters

-- BCS is defined by RAN4’s definition per RF CA

SupportedBandwidthCombinationSet ::=	BIT STRING (SIZE (1..maxBandwidthCombSet))

BandCombinationDLParametersList ::= SEQUENCE {

-- Type 2&3 params that are included once per band combination

-- TBD: If this list is long and repetitive we can also create a reference list like below for SupportedParametersPerBand

}

-- UL band combinations (without signalling of frequency bands)

-- Per Band combination UL parameters

BandCombinationUL-PerBC-DL ::= SEQUENCE (SIZE (1..maxBandCombUL)) OF BandParameterCombinationUL

BandParameterCombinationUL ::= SEQUENCE (SIZE (1.. maxSimultaneousBands)) OF PerBandParametersUL

PerBandParametersUL ::= SEQUENCE {

bandParametersUL					BandParametersUL			OPTIONAL  -- Not included in case of DL-only band

}

BandCombinationULParameters ::= SEQUENCE {

multipleTimingAdvances		ENUMERATED {supported}		OPTIONAL,

singleUL-Transmission		ENUMERATED {supported}		OPTIONAL,

intraBandSimultaneousTxRx	ENUMERATED {supported}		OPTIONAL	-- FFS per UE or per band Combination

}

-- Others

BandParametersDL ::= SEQUENCE {

dl-parametersPerBandIndexList		ParametersPerBandIndexList, -- Index list to supportedDLParametersPerBand

scalingFactor0dot75			ENUMERATED {supported}		OPTIONAL, -- FFS dependent on RAN1 confirmation

...

}

BandParametersUL ::= SEQUENCE {

ul-parametersPerBandIndexList		ParametersPerBandIndexList, -- Index list to supportedULParametersPerBand

scalingFactor0dot75			ENUMERATED {supported}		OPTIONAL, -- FFS dependent on RAN1 confirmation

...

}

-- TAG-BAND-COMBINATION-LIST-STOP

-- ASN1STOP

BandCombinationList field descriptions

bandCombinationsUL Bit string with pointers to entries in BandCombinationListUL. Only the UL combinations of the same number of entries as in bandAndParametersDLList can be pointed to.

#### –	FreqBandList

The IE FreqBandList is used to contain list of NR and/or E-UTRA frequency bands for which the UE is requested to provide its supported NR CA and/or MR-DC band combinations (i.e. within the UE capability containers for NR and MR-DC, as requested by E-UTRA).

FreqBandList information element

-- ASN1START

FreqBandList ::=	SEQUENCE (SIZE (1..maxRequestedBands)) OF FreqBandInformation

FreqBandInformation ::= CHOICE {

bandEUTRA				FreqBandIndicatorEUTRA,

bandNR					FreqBandIndicator

}

-- ASN1STOP

#### –	RAT-Type

The IE RAT-Type is used to indicate the radio access technology (RAT), including NR, of the requested/transferred UE capabilities.

RAT-Type information element

-- ASN1START

-- TAG-RAT-TYPE-START

RAT-Type ::= ENUMERATED {nr, eutra-nr, spare2, spare1, ...}

-- TAG-RAT-TYPE-STOP

-- ASN1STOP

#### –	UE-CapabilityRAT-ContainerList

The IE UE-CapabilityRAT-ContainerList contains a list of containers, one for each RAT for which UE capabilities are transferred, if any.

UE-CapabilityRAT-ContainerList information element

-- ASN1START

-- TAG-UE-CAPABILITY-RAT-CONTAINER-LIST-START

UE-CapabilityRAT-ContainerList ::=SEQUENCE (SIZE (0.. maxRAT-CapabilityContainers)) OF UE-CapabilityRAT-Container

UE-CapabilityRAT-Container ::= SEQUENCE {

rat-Type							RAT-Type,

ue-CapabilityRAT-Container			OCTET STRING

}

-- TAG-UE-CAPABILITY-RAT-CONTAINER-LIST-STOP

-- ASN1STOP

UE-CapabilityRAT-ContainerList field descriptions

ue-CapabilityRAT-Container Container for the UE capabilities of the indicated RAT. The encoding is defined in the specification of each RAT: For NR: the encoding of UE capabilities is defined in UE-NR-Capability. For EUTRA-NR: the encoding of UE capabilities is defined in UE-MRDC-Capability

#### –	UE-MRDC-Capability

The IE UE-MRDC-Capability is used to convey the UE Radio Access Capability Parameters for MR-DC, see TS 38.306 [yy].

UE-MRDC-Capability information element

-- ASN1START

-- TAG-UE-MRDC-CAPABILITY-START

UE-MRDC-Capability ::=	SEQUENCE {

measParameters-MRDC			MeasParameters-MRDC,

rf-Parameters-MRDC			RF-Parameters-MRDC,

phyLayerParameters-MRDC		PhyLayerParameters-MRDC	OPTIONAL, -- FFS dependent on other parameters (e.g. L1 feature list)

generalParameters-MRDC		GeneralParameters-MRDC		OPTIONAL,

fdd-UE-MRDC-Capability		XDD-UE-MRDC-Capability		OPTIONAL,

tdd-UE-MRDC-Capability		XDD-UE-MRDC-Capability		OPTIONAL

-- FFS on other parameters

}

RF-Parameters-MRDC ::= SEQUENCE {

supportedBandCombination	MR-DC-BandCombinationList

dynamicPowerSharing			ENUMERATED {supported}		OPTIONAL,

intraBandAsyncFDD			ENUMERATED {supported}		OPTIONAL -- FFS whether intraBandAsyncFDD is included per UE or per band combination

-- FFS on other parameters

}

PhyLayerParameters-MRDC ::= SEQUENCE {

-- FFS on other parameters

}

MeasParameters-MRDC ::= SEQUENCE {

intraCarrierConcurrentMeas	ENUMERATED {supported}	OPTIONAL,

independentGapConfig		ENUMERATED {supported}	OPTIONAL,

sstd-Meas-DC				ENUMERATED {supported}	OPTIONAL

}

GeneralParameters-MRDC ::= SEQUENCE {

splitSRB-WithOneUL-Path		ENUMERATED {supported}	OPTIONAL,

directSN-Addition			ENUMERATED {supported}	OPTIONAL

}

XDD-UE-MRDC-Capability ::= SEQUENCE {

intraCarrierConcurrentMeas	ENUMERATED {supported}	OPTIONAL,

independentGapConfig		ENUMERATED {supported}	OPTIONAL,

sstd-Meas-DC				ENUMERATED {supported}	OPTIONAL,

splitSRB-WithOneUL-Path		ENUMERATED {supported}	OPTIONAL,

directSN-Addition			ENUMERATED {supported}	OPTIONAL

}

-- TAG-UE-MRDC-CAPABILITY-STOP

-- ASN1STOP

#### –	UE-NR-Capability

The IE UE-NR-Capability is used to convey the NR UE Radio Access Capability Parameters, see TS 38.306 [yy].

UE-NR-Capability information element

-- ASN1START

-- TAG-UE-NR-CAPABILITY-START

UE-NR-Capability ::= SEQUENCE {

pdcp-Parameters					PDCP-Parameters,

rlc-Parameters					RLC-Parameters,						-- FFS OPTIONAL

mac-Parameters					MAC-Parameters,						-- FFS OPTIONAL

phyLayerParameters				PhyLayerParameters,

rf-Parameters					RF-Parameters,

measParameters					MeasParameters						OPTIONAL,

fdd-UE-NR-Capability			XDD-UE-NR-Capability				OPTIONAL,

tdd-UE-NR-Capability			XDD-UE-NR-Capability				OPTIONAL,

nonCriticalExtension			SEQUENCE {}							OPTIONAL

}

PhyLayerParameters ::=	SEQUENCE {

-- FFS on other parameters

}

RF-Parameters ::= SEQUENCE {

-- Per band information

supportedBandListNR						SupportedBandListNR,

--NR-“Reference list” per band

supportedParametersPerBandCombination	SupportedParametersPerBandCombination,

supportedDLParametersPerBand			SupportedDLParametersPerBand,

supportedULParametersPerBand			SupportedULParametersPerBand,

-- Actual list, do we need this or part of MR-DC for NSA?

supportedBandCombination			BandCombinationList

}

SupportedBandListNR ::=	SEQUENCE (SIZE (1..maxBands)) OF PerBandNRInfo

ParametersPerBandIndexList::= SEQUENCE (SIZE (1..maxBandConfigurationComb)) OF ParametersPerBandIndex

-- Per Band DL parameters

ParametersPerBandIndex :: = INTEGER(1..maxBandParamtersPerBand)

-- ParametersPerBandIndex indices SupportedDLParametersPerBand in order of appearance in the list:

SupportedDLParametersPerBand ::= SEQUENCE (SIZE (1..maxBandParamtersPerBand)) OF DLParametersPerBand

DLParametersPerBand ::= SEQUENCE {

parametersPerCC		SEQUENCE (SIZE (1..maxNrofCC)) OF DLParametersPerCC,

-- Other type 2&3 params that are per band [AG]

-- FFS on other parameters

}

DLParametersPerCC ::= SEQUENCE {

dl-bandwidthPerCC-List				BandwidthPerCC-List						OPTIONAL,

supportedMIMO-CapabilityDL		MIMO-Capability						OPTIONAL,

supportedModulationOrderDL					ModulationOrder			OPTIONAL,

supportedSubCarrierSpacingList				SupportedSubCarrierSpacingList

-- FFS if supportedSubCarrierSpacingList is included per Band or per CC and whether to separate one for DL and UL.

-- FFS on other parameters

}

-- Per Band UL parameters

-- Per Band combination common IEs, indexed 1..maxBandParamtersPerBand in order of appearance in the list:

SupportedULParametersPerBand ::= SEQUENCE (SIZE (1..maxBandParamtersPerBand)) OF ULParametersPerBand

ULParametersPerBand ::= SEQUENCE {

parametersPerCC		SEQUENCE (SIZE (1..maxNrofCC)) OF ULParametersPerCC

-- Other type 2&3 params that are per band

-- FFS on other parameters

}

ULParametersPerCC ::= SEQUENCE {

Ul-bandwidthPerCC-List			BandwidthPerCC-List					OPTIONAL,

supportedMIMO-CapabilityUL		MIMO-Capability						OPTIONAL,

supportedModulationOrderUL		ModulationOrder						OPTIONAL

}

BandwidthPerCC-List ::= SEQUENCE (SIZE (1..maxNrofBandwidths)) BandwidthPerCC

-- START Per Band parameters

PerBandNRInfo ::=	SEQUENCE {

bandNR							FreqBandIndicator,

supportedMIMO-CapabilityDL		MIMO-Capability						OPTIONAL,

supportedMIMO-CapabilityUL		MIMO-Capability						OPTIONAL

-- FFS on other parameters

}

CA-BandwidthClass ::= ENUMERATED {a, b, c, d, e, f, ...}		-- FFS value ranges

MIMO-Capability ::= SEQUENCE {

-- FFS on the parameters

}

BandwidthPerCC ::= ENUMERATED {TBD}			-- FFS value ranges

ModulationOrder ::= SEQUENCE {

-- FFS on the parameters

}

SupportedSubCarrierSpacingList ::= SEQUENCE {

-- FFS on the parameters

}

PDCP-Parameters ::= SEQUENCE {

dataRateDRB-IP		ENUMERATED {64kbps, spare7, spare6, spare5, spare4, spare3, spare2, spare1}		OPTIONAL, -- Not supported for EN-DC.

supportedROHC-Profiles	SEQUENCE {

profile0x0000		BOOLEAN,

profile0x0001		BOOLEAN,

profile0x0002		BOOLEAN,

profile0x0003		BOOLEAN,

profile0x0004		BOOLEAN,

profile0x0006		BOOLEAN,

profile0x0101		BOOLEAN,

profile0x0102		BOOLEAN,

profile0x0103		BOOLEAN,

profile0x0104		BOOLEAN

},

maxNumberROHC-ContextSessions	ENUMERATED {cs2, cs4, cs8, cs12, cs16, cs24, cs32, cs48, cs64, cs128, cs256, cs512, cs1024, cs16384, spare2, spare1},

uplinkOnlyROHC-Profiles			ENUMERATED {supported}	OPTIONAL,

continueROHC-Context			ENUMERATED {supported}	OPTIONAL,

outOfOrderDelivery				ENUMERATED {supported}	OPTIONAL,

shortSN							ENUMERATED {supported} 	OPTIONAL,

}

RLC-Parameters ::= SEQUENCE {

am-WithShortSN					ENUMERATED {supported}	OPTIONAL,

um-WithShortSN					ENUMERATED {supported}	OPTIONAL,

um-WIthLongSN					ENUMERATED {supported}	OPTIONAL

}

MAC-Parameters ::= SEQUENCE {

lcp-Restriction					ENUMERATED {supported}	OPTIONAL,

skipUplinkTxDynamic				ENUMERATED {supported}	OPTIONAL,

logicalChannelSR-DelayTimer		ENUMERATED {supported}	OPTIONAL,

longDRX-Cycle					ENUMERATED {supported}	OPTIONAL,

shortDRX-Cycle					ENUMERATED {supported}	OPTIONAL,

numberOfSR-Configurations		ENUMERATED {n2, n3, n4, ...}	OPTIONAL, -- FFS value range

numberOfConfiguredGrantConfigurations	ENUMERATED {n2, n3, n4, ...}	OPTIONAL -- FFS value range

}

MeasParameters ::= SEQUENCE {

intraAndInterF-MeasAndReport		ENUMERATED {supported}	OPTIONAL,

eventA-MeasAndReport			ENUMERATED {supported}	OPTIONAL

}

XDD-UE-NR-Capability ::= SEQUENCE {

skipUplinkTxDynamic				ENUMERATED {supported}	OPTIONAL,

logicalChannelSR-DelayTimer		ENUMERATED {supported}	OPTIONAL,

longDRX-Cycle					ENUMERATED {supported}	OPTIONAL,

shortDRX-Cycle					ENUMERATED {supported}	OPTIONAL,

numberOfSR-Configurations		ENUMERATED {n2, n3, n4,…}	OPTIONAL, -- FFS value range

numberOfConfiguredGrantConfigurations	ENUMERATED {n2, n3, n4,…}	OPTIONAL, -- FFS value range

intraAndInterF-MeasAndReport		ENUMERATED {supported}	OPTIONAL,

eventA-MeasAndReport			ENUMERATED {supported}	OPTIONAL

}

-- TAG-UE-NR-CAPABILITY-STOP

-- ASN1STOP

### 6.3.4	Other information elements

## 6.4	RRC multiplicity and type constraint values

### –	Multiplicity and type constraint definitions

-- ASN1START

-- TAG-MULTIPLICITY-AND-TYPE-CONSTRAINT-DEFINITIONS-START

maxBandComb								INTEGER ::=	ffsValue		-- Maximum number of DL band combinations

maxBandComb-r15							INTEGER ::=	ffsValue		-- Maximum number of LTE band combinations

maxBandCombUL							INTEGER ::= FFS		-- Maximum number of UL band combinations

maxBandParamtersPerBand					INTEGER ::= 256

maxBandConfigurationComb						INTEGER ::=	8		-- Maximum number of base band processing combinations

maxBandConfigurationComb				INTEGER ::=	8	-- Maximum number of configurations for a band

maxBandwidthCombSet						INTEGER ::=	8	-- Maximum number of bandwidth combination sets per

-- supported band combination

maxNrofServingCells						INTEGER ::=	16		-- Max number of serving cells (SpCell + SCells) per cell group

maxNrofServingCells-1					INTEGER ::= 15		-- Max number of serving cells (SpCell + SCells) per cell group minus 1

maxNrofSCells							INTEGER ::=	15		-- Max number of secondary serving cells per cell group

maxNrofCellMeas							INTEGER ::=	ffsValue		-- Maximum number of entries in each of the cell lists in a measurement object

maxNrofSS-BlocksToAverage				INTEGER ::= ffsValue		-- Max number for the (max) number of SS blocks to average to determine cell

-- measurement

maxNrofCSI-RS-ResourcesToAverage		INTEGER ::= ffsValue		-- Max number for the (max) number of CSI-RS to average to determine cell

-- measurement

maxNrofDL-Allocations 					INTEGER ::= ffsValue

maxNrofSR-ConfigPerCellGroup			INTEGER ::= 8		-- Maximum number of SR configurations per cell group

maxLCG-ID								INTEGER ::= 7		-- Maximum value of LCG ID

maxLC-ID								INTEGER ::= ffsValue		-- Maximum value of Logical Channel ID

maxNrofTAGs								INTEGER ::=	4		-- Maximum number of Timing Advance Groups

maxNrofTAGs-1							INTEGER ::=	3		-- Maximum number of Timing Advance Groups minus 1

maxNrofBWPs					INTEGER ::= 4		-- Maximum number of BWPs per serving cell

maxNrofSymbols-1							INTEGER ::= 13		-- Maximum index identifying a symbol within a slot (14 symbols, indexed from 0..13)

maxNrofSlots							INTEGER ::= 320		-- Maximum number of slots in a 10 ms period

maxNrofSlots-1							INTEGER ::= 319		-- Maximum number of slots in a 10 ms period minus 1

maxNrofPhysicalResourceBlocks			INTEGER ::= 275		-- Maximum number of PRBs

maxNrofPhysicalResourceBlocks-1			INTEGER ::= 274		-- Maximum number of PRBs

maxNrofControlResourceSets 				INTEGER ::= 12 	-- Max number of CoReSets configurable on a serving cell

maxNrofControlResourceSets-1			INTEGER ::= 11  	-- Max number of CoReSets configurable on a serving cell minus 1

maxCoReSetDuration						INTEGER ::= 3		-- Max number of OFDM symbols in a control resource set

maxNrofSearchSpaces						INTEGER ::= 40		-- Max number of Search Spaces

maxNrofSearchSpaces-1					INTEGER ::= 339		-- Max number of Search Spaces minus 1

maxSFI-DCI-PayloadSize					INTEGER ::= ffsValue		-- Max number payload of a DCI scrambled with SFI-RNTI

maxSFI-DCI-PayloadSize-1				INTEGER ::= ffsValue		-- Max number payload of a DCI scrambled with SFI-RNTI minus 1

maxINT-DCI-PayloadSize					INTEGER ::= ffsValue		-- Max number payload of a DCI scrambled with INT-RNTI

maxINT-DCI-PayloadSize-1				INTEGER ::= ffsValue		-- Max number payload of a DCI scrambled with INT-RNTI minus 1

maxNrofRateMatchPatterns				INTEGER ::= 4		-- Max number of rate matching patterns that may be configured

maxNrofRateMatchPatterns-1				INTEGER ::= 3		-- Max number of rate matching patterns that may be configured minus 1

maxNrofCSI-Reports						INTEGER ::= ffsValue 	-- Maximum number of report configurations

maxNrofCSI-RS-CellsRRM 					INTEGER ::= ffsValue 	-- Maximum number of FFS

maxNrofReportConfigIdsPerTrigger		INTEGER ::= 16	 	-- Maximum number of report configurations per reportTrigger

maxNrofCSI-ResourceConfigurations		INTEGER ::= ffsValue		-- Maximum number of resource configurations

maxNrofCSI-ResourceConfigurations-1		INTEGER ::= ffsValue		-- Maximum number of resource configurations minus 1

maxNrofCSI-ResourceSets					INTEGER ::= ffsValue		-- Maximum number of resource sets per resource configuration

maxNrofCSI-ResourceSets-1				INTEGER ::= ffsValue		-- Maximum number of resource sets per resource configuration minus 1

maxNrofFailureDetectionResources		INTEGER ::= ffsValue		-- Maximum number of failure detection resources

maxNrofNZP-CSI-RS-Resources-1			INTEGER ::= ffsValue		-- Maximum number of Non-Zero-Power (NZP) CSI-RS resources minus 1

maxNrofZP-CSI-RS-Resources				INTEGER ::= 3		-- Maximum number of Zero-Power (NZP) CSI-RS resources

maxNrofZP-CSI-RS-Resources-1			INTEGER ::= 2		-- Maximum number of Zero-Power (NZP) CSI-RS resources minus 1

maxNrofCSI-IM-Resources					INTEGER ::= ffsValue		-- Maximum number of CSI-IM resources. See CSI-IM-ResourceMax in 38.214.

maxNrofCSI-IM-Resources-1				INTEGER ::= ffsValue		-- Maximum number of CSI-IM resources minus 1. See CSI-IM-ResourceMax in 38.214.

maxNrofCSI-IM-ResourcesPerSet			INTEGER ::= ffsValue		-- Maximum number of CSI-IM resources per set. See CSI-IM-ResourcePerSetMax in 38.214

maxNrofSSB-Resources-1					INTEGER ::= 63		-- Maximum number of SSB resources in a resource set minus 1

maxNrofCSI-RS-ResourcesPerSet			INTEGER ::= 8		-- Maximum number of CSI-RS resources per resource set

maxNrofCSI-MeasId						INTEGER ::= ffsValue		-- Maximum number of link configurations

maxNrofCSI-MeasId-1						INTEGER ::= ffsValue		-- Maximum number of link configurations minus 1

maxNrofCSI-RS-ResourcesRRM				INTEGER ::= ffsValue		-- Maximum number of CSI-RS resources for an RRM measurement object

maxNrofCSI-RS-ResourcesRRM-1			INTEGER ::= ffsValue		-- Maximum number of CSI-RS resources for an RRM measurement object minus 1

maxNrofObjectId							INTEGER ::= ffsValue		-- Maximum number of configured measurement objects

maxNrofPCI-Ranges						INTEGER ::= ffsValue		-- Maximum number of PCI ranges

maxReportConfigId					INTEGER ::= ffsValue		-- Maximum number of reporting configurations

maxNrofMeasId							INTEGER ::= ffsValue		-- Maximum number of configured measurements

maxNrofQuantityConfig					INTEGER	::= 2		-- Maximum number of quantity configurations

maxNrofSRS-ResourceSets					INTEGER ::= ffsValue		-- Maximum number of SRS resource sets.

maxNrofSRS-ResourceSets-1				INTEGER ::= ffsValue		-- Maximum number of SRS resource sets minus 1.

maxNrofSRS-Resources					INTEGER ::= ffsValue		-- Maximum number of SRS resources in an SRS resource set.

maxNrofSRS-Resources-1					INTEGER ::= ffsValue		-- Maximum number of SRS resources in an SRS resource set minus 1.

maxNrofSRS-TriggerStates-1 				INTEGER ::= 3		-- Maximum number of SRS trigger states minus 1, i.e., the largest code point.

maxRAT-CapabilityContainers				INTEGER ::= ffsValue		-- Maximum number of interworking RAT containers (incl NR and MRDC)

maxRequestedBands						INTEGER ::= FFS		-- Maximum number of simultaneously requested bands

maxNrofCC								INTEGER ::=	FFS		-- Maximum number of carriers

maxSimultaneousBands					INTEGER ::= ffsValue		-- Maximum number of simultaneously aggregated bands

maxNrofSlotFormatCombinationsPerSet		INTEGER ::= ffsValue		-- Maximum number of Slot Format Combinations in a SF-Set.

maxNrofSlotFormatCombinationsPerSet-1	INTEGER ::= ffsValue		-- Maximum number of Slot Format Combinations in a SF-Set minus 1.

maxNrofPUCCH-ResourceSets				INTEGER ::= 4		-- Maximum number of PUCCH Resource Sets

maxNrofPUCCH-ResourceSets-1				INTEGER ::= 3		-- Maximum number of PUCCH Resource Sets minus 1.

maxNrofPUCCH-ResourcesPerSet			INTEGER ::= 8		-- Maximum number of PUCCH Resources per PUCCH-ResourceSet

maxNrofPUCCH-ResourcesPerSet-1			INTEGER ::= 7		-- Maximum number of PUCCH Resources per PUCCH-ResourceSet minus 1.

maxNrofPUCCH-P0-PerSet					INTEGER ::= 8		-- Maximum number of P0-pucch present in a p0-pucch set

maxNrofPUCCH-PathlossReferenceRSs		INTEGER ::= 4		-- Maximum number of RSs used as pathloss reference for PUCCH power control.

maxNrofPUCCH-PathlossReferenceRSs-1	INTEGER ::= 3		-- Maximum number of RSs used as pathloss reference for PUCCH power control minus 1.

maxNrofP0-PUSCH-AlphaSets				INTEGER ::= 30		-- Maximum number of P0-pusch-alpha-sets (see 38,213, section 7.1)

maxNrofP0-PUSCH-AlphaSets-1				INTEGER ::= 29		-- Maximum number of P0-pusch-alpha-sets minus 1 (see 38,213, section 7.1)

maxNrofPUSCH-PathlossReferenceRSs		INTEGER ::= 4		-- Maximum number of RSs used as pathloss reference for PUSCH power control.

maxNrofPUSCH-PathlossReferenceRSs-1	INTEGER ::= 3		-- Maximum number of RSs used as pathloss reference for PUSCH power control minus 1.

maxEARFCN								INTEGER ::= 262143	-- Highest value of extended E-UTRA EARFCN range

maxBands 								INTEGER ::= ffsValue

maxCellPrep		 						INTEGER ::= ffsValue

maxCellReport		 					INTEGER ::= ffsValue

maxCellSCG		 						INTEGER ::= ffsValue

maxDRB		 							INTEGER ::= ffsValue

maxFreq		 							INTEGER ::= ffsValue

maxLCH									INTEGER ::= ffsValue

maxNrofCSI-RS	 						INTEGER ::= ffsValue

maxNrofAggregatedCellsPerCellGroup 		INTEGER ::= ffsValue

maxNrofCandidateBeams 					INTEGER ::= ffsValue

maxNrofCSI-ReportConfig-1 				INTEGER ::= ffsValue

maxNrofCSI-ResrouceConfigurations 		INTEGER ::= ffsValue

maxNrofPCIsPerSMTC 						INTEGER ::= ffsValue

maxNrofQFIs 							INTEGER ::= ffsValue

maxNrofSR-Resoruces		 		INTEGER ::= ffsValue

maxNrofSlotFormatsPerCombination 		INTEGER ::= ffsValue

maxNrofSpatialRelationInfos 			INTEGER ::= ffsValue

maxNrofSRS-ResourcesPerSet 				INTEGER ::= ffsValue

maxNrofIndexesToReport 					INTEGER ::= ffsValue

maxNrofSSBs 							INTEGER ::= ffsValue

maxNrofTCI-StatesPDCCH 					INTEGER ::= ffsValue

maxNrofTCI-States						INTEGER ::= 64

maxNrofTCI-States-1						INTEGER ::= 63

maxNrofUL-Allocations 					INTEGER ::= ffsValue

maxQFI 									INTEGER ::= ffsValue

maxRA-CSIRS-Resources 					INTEGER ::= ffsValue

maxRA-SSB-Resources 						INTEGER ::= ffsValue

maxSCSs									INTEGER ::= ffsValue

maxSecondaryCellGroups 					INTEGER ::= ffsValue

ffsValue								INTEGER ::= 64

-- IE definitions introduced to not get warning at ASN.1 syntax check

CandidateRS-IndexInfoList ::=	ENUMERATED {ffsTypeAndValue}

CellIdentity ::=				ENUMERATED {ffsTypeAndValue}

CSI-RS-Index ::=					ENUMERATED {ffsTypeAndValue}

FilterCoefficient ::=			ENUMERATED {ffsTypeAndValue}

Hysteresis ::=					ENUMERATED {ffsTypeAndValue}

MeasObjectEUTRA ::=				ENUMERATED {ffsTypeAndValue}

MeasResultListEUTRA ::=			ENUMERATED {ffsTypeAndValue}

MeasResultSSTD ::=				ENUMERATED {ffsTypeAndValue}

PDU-SessionID ::=				ENUMERATED {ffsTypeAndValue}

PhyCellNR ::=					ENUMERATED {ffsTypeAndValue}

PhysCellIdEUTRA ::=				ENUMERATED {ffsTypeAndValue}

PhysCellIdRange ::=				ENUMERATED {ffsTypeAndValue}

P-Max ::=						ENUMERATED {ffsTypeAndValue}

RA-Resources ::=				ENUMERATED {ffsTypeAndValue}

ReportConfigEUTRA ::=			ENUMERATED {ffsTypeAndValue}

RRC-TransactionIdentifier ::=	ENUMERATED {ffsTypeAndValue}

SchedulingRequestId ::=			ENUMERATED {ffsTypeAndValue}

ShortMAC-I ::=					ENUMERATED {ffsTypeAndValue}

SSB-Id ::=						ENUMERATED {ffsTypeAndValue}

TimeToTrigger ::=				ENUMERATED {ffsTypeAndValue}

UECapabilityInformation ::=	ENUMERATED {ffsTypeAndValue}

BW-PerCC ::=		ENUMERATED {ffsTypeAndValue}

FFS_Value ::=		ENUMERATED {ffsTypeAndValue}

FreqBandIndicatorNR ::=		ENUMERATED {ffsTypeAndValue}

MBSFN-SubframeConfigList ::=		ENUMERATED {ffsTypeAndValue}

NZP-CSI-RS-ResourceConfigId ::=		ENUMERATED {ffsTypeAndValue}

SlotFormatIndicator ::=		ENUMERATED {ffsTypeAndValue}

-- TAG-MULTIPLICITY-AND-TYPE-CONSTRAINT-DEFINITIONS-STOP

-- ASN1STOP