3GPP TSG-RAN WG2 #101	R2-1802610

Athens, Greece, 26th February – 2nd March 2018

Agenda Item:	9.3.10

Source:	Ericsson

Title:	RLC UM for NB-IoT open issues

Document for:	Discussion, Decision

# Introduction

In previous meetings there has been discussions on the need for introducing RLC UM to NB-IoT for other cases than for SC-PtM. Discussions have been resolved and RLC UM for DRBs is now added to the specification.

- RAN2#101:

  - RLC UM for DRBs was captured in the running CR to 36.331 (R2-1714258)

    - Based on a draftCR (R2-1713956) from Huawei

  - Open issues:

    - FFS if we use 5bit/10bit SN or just 10bit SN

    - FFS if we support unidirectional

There are still 2 open issues after the latest meeting [1], namely how many bits to use for SN and whether to include support of unidirectional RLC UM. In this paper we present a way forward on these FFSs and a text proposal for TS 36.300 [2] to be added to the running CR.

# Discussion

## Number of bits for SN

The first open issue is whether the SN should be 5 and 10 bits or just 10 bits.  By selecting the first option with both 5 and 10 bits it is possible to configure RLC for different cases. It is good to have the possibility to configure with a minimum of overhead, i.e. 5 bits, which would save 1 byte per TTI. We therefore propose to keep the settings from legacy LTE and keep the possibility to configure the RLC SN with 5 or 10 bits length.

Allow the option of configuring RLC SN with 5 or 10 bits.

## RLC directional setup

In legacy LTE the most common RLC UM mode is probably bidirectional. This since one of the most common uses of RLC UM is VoLTE, which is a bi-directional service.  For NB-IOT there is no VoLTE service. There are instead several other possible services, most of them expected to be almost or entirely unidirectional. That is data is sent from a sensor to a server somewhere on the Internet. Therefore NB-IOT should be possible to configure with unidirectional RLC UM (as well as bidirectional).

Allow the option of configure RLC UM to be unidirectional (or bidirectional).

# Text Proposal

The specification TS 36.300 [2] needs to be updated to support RLC UM for all DRBs.

- 6.2.1	Services and Functions

The main services and functions of the RLC sublayer include:

-	Transfer of upper layer PDUs;

-	Error Correction through ARQ (only for AM data transfer);

-	Concatenation, segmentation and reassembly of RLC SDUs (only for UM and AM data transfer);

-	Re-segmentation of RLC data PDUs (only for AM data transfer);

-	Reordering of RLC data PDUs (only for UM and AM data transfer);

-	Duplicate detection (only for UM and AM data transfer);

-	Protocol error detection (only for AM data transfer);

-	RLC SDU discard (only for UM and AM data transfer);

-	RLC re-establishment, as defined in TS 24.301 [20];

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	Allow the option of configuring RLC SN with 5 or 10 bits.

Proposal 2	Allow the option of configure RLC UM to be unidirectional.

# References

Report of 3GPP TSG RAN WG2 meeting #100 Reno, Nevada, USA, 27 November - 1 December, 2017

(E-UTRAN) Overall description; Stage 2 (Release 15). 3GPP TS 36.300 V15.0.0 (2017-12)