3GPP TSG-RAN WG2 Meeting #101								R2-1802667

Athens, Greece, 26 Feb- 2 Mar 2018

Agenda item:	9.9.2

Source: 	Huawei, HiSilicon

Title: 	Clarification on the MAC CE for new SCell state transition

Document for:	Discussion and decision

1	Introduction

In last RAN2#100 meeting, the following agreements were reached:

Agreements: 1 Transition between legacy SCell deactivated state and fast activation state is via MAC-CE (i.e., similar to legacy). 2 Legacy state transition mechanisms are applicable for transition between legacy SCell activated and SCell deactivated states.

And in the subsequent Email discussion of stage-3 CR after RAN2#100 meeting, a new MAC CE for indicating fast activation state is proposed [1]. But there is still an ambiguous point in the specification. In this document, we continue to discuss the description of newly designed MAC CE.

2	Discussion

The proposed CR is as follows:

Next change

6.1.3.15	New State Transition SCell MAC Control Elements

The New State Transition  SCell MAC control element of one octet is identified by a MAC PDU subheader with LCID as specified in table 6.2.1-1. It has a fixed size and consists of a single octet containing seven C-fields and one R-field. The New State transition  SCell MAC control element with one octet is defined as follows (figure 6.1.3.15-1).

The New State transition  SCell MAC control element of four octets is identified by a MAC PDU subheader with LCID as specified in table 6.2.1-1. It has a fixed size and consists of a four octets containing 31 C-fields and one R-field. The New State transition  SCell MAC control element of four octets is defined as follows (figure 6.1.3.15-2).

For the case with no serving cell with a ServCellIndex [8] larger than 7,New State transition SCell MAC control element of one octet is applied, otherwiseNew State transition SCell MAC control element of four octets is applied.

-	Ci: if there is an SCell configured with SCellIndex i as specified in [8], this field indicates the activation/deactivation status of the SCell with SCellIndex i, else the MAC entity shall ignore the Ci field. The Ci field is set to "1" to indicate that the SCell with SCellIndex i shall be activated. The Ci field is set to "0" to indicate that the SCell with SCellIndex i shall be deactivated;

-	R: Reserved bit, set to “0”.

Figure 6.1.3.15-1: New State transition  SCell MAC control element of one octet

Figure 6.1.3.15-2: New State transition SCell MAC control element of four octets

End

As the new fast activation state is added, there is totally three states of a SCell, i.e. activation state, deactivation state and fast activation state. Based on the sentence of “The Ci field is set to "1" to indicate that the SCell with SCellIndex i shall be activated. The Ci field is set to "0" to indicate that the SCell with SCellIndex i shall be deactivated;”, it is ambiguous which states the SCell will to go after the Ci field is set to "0" . The information we can acquire is just the Scell’s new fast activation state had been deactivated, without the indication of the destination state. Hence, we can combine this new MAC CE and current activation/deactivation MAC CE to implement all state transitions among these three SCell states together. There is no need to indicate an activated “fast activation state” or deactivated “fast activation state” which just results in the confusion.

Proposal 1:  The Ci field is set to "0" to indicate that the SCell with SCellIndex i shall leave the new fast activation state.

3	Conclusions

In this document, we continue to discuss the description of newly designed MAC CE, and we have the following proposal:

Proposal 1:  The Ci field is set to "0" to indicate that the SCell with SCellIndex i shall leave the new fast activation state.

4	References

- R2-1714288 euCA MAC running CR (Rel-15)