3GPP TSG-RAN WG2 meeting #101																	  R2-1803224

Athens, Greece, 26 February - 2 March, 2018

Agenda item:	9.19

Source:	CMCC

Title:	Consideration on Bluetooth/WLAN measurement collection in MDT

Document for:	Discussion and Decision

1	Introduction

In the RAN#78 meeting, a new WID “Bluetooth/WLAN measurement collection in MDT” is approved [1]. The objective of this work item is to introduce Bluetooth (BT) and WLAN measurement collection in MDT to monitor and assess coverage performance of BT and WLAN network and also to provide location information for the associated other MDT measurements.

In this contribution, we would give our considerations on the related functionalities and procedures and analyse the specification impact as well.

2	Discussion

2.1	General considerations

2.1.1	Relation with the existing LTE positioning specifications

The main use of BT/WLAN measurement results is to calculate the location of a UE. These measurement/location information together with other MDT measurement results are then reported to the eNB by the UEs for further network optimization purpose. So here comes the critical question: why we need this functionality to obtain UE location when there already are some LTE Positioning Protocol (LPP) specifications. With LPP specifications, we can obtain UE location first using the existing positioning procedures and then correlate the UE location information with other MDT measurements based on time-stamps. In our understanding, using LPP procedure to obtain UE location is a feasible way but has the following limitations:

1) Modifications to the LPP specifications are required

- In MDT LoggedMeasurementConfiguration message, there are two mandatory fields, absoluteTimeInfo and loggingInterval, where the absoluteTimeInfo indicates the absolute time in the current cell and the LoggingInterval indicates the periodicity for logging measurement results. If we want to correlate the location information obtained by LPP procedures with other MDT measurements, the two mandatory fields should also be introduced in the LPP specifications to ensure the matching effect. This is the first impact on the LPP specifications.

- The second impact is UE minimum memory requirement. In MDT, a UE that supports logged measurements in RRC_IDLE shall also support a minimum of 64kB memory for log storage. To support the new BT/WLAN based RF fingerprint in logged MDT, a minimum memory for location information storage should be introduced in the existing LPP specifications wherein logged positioning is not supported.

- To protect user privacy, it is probably that UEs only measure and report the radio performance of public or operator deployed BT/WLAN APs excluding home APs. The existing LPP mechanism (TS 36.305) is in support of the delivery of WLAN AP list to UE via LPP Provide Assistance Data message. However the delivery of BT beacon list is not supported in the LPP specifications. If we want to indicate UE to measure a specific group of BT beacons, similar signalling message carrying the BT beacon list should be defined in the existing LPP specifications

Above are our concerns on the impacts to the existing LPP specifications.

- Modifications to the LPP specifications are required, for example:

- two mandatory fields should be introduced to ensure the matching between logged MDT measurements and location information;

- a minimum UE memory for location information storage should be introduced;

- signalling message carrying the BT beacon list should be defined to indicate UE to measure a specific group of BT beacons.

2) Additional requirements on UE and network are needed

If the location information is obtained from E-SMLC, it means the UEs and network elements must support MDT functionality and LPP functionality at the same time. Unfortunately, LPP specifications use a different decoder rather than the ASN.1 decoder, which is RTCM decoder. If we introduce RTCM positioning decoder in MDT, the cellular UE chipset company has to implement another set of RTCM decoder. The two decoders (ASN.1 decoder and RTCM positioning decoder) may exist in a single smart phone, bringing additional development cost.

- Additional requirements on UE and network are needed if we use LPP mechanism to obtain UE location.

3) Signalling overhead are added

Using LPP functionality to obtain UE location information means at least four messages would be used to achieve the whole MDT measurement results. One message carrying BT/WLAN measurement results is encoded by LPP and then forwarded by eNB to E-SMLC. Based on these measurement results, E-SMLC would calculate the location of the UE and then feedback a message containing location information to MDT server (TCE). Around the same time, TCE would receive another message carrying other MDT measurement results from UE. Based on time-stamps, the TCE associates location information with these MDT measurements. So we can see the procedure is very complex and the overhead is large when the MDT measurements and UE location information are provided in separate message.

- The procedure is complex and the overhead is large if we use LPP mechanism to obtain UE location.

With the above concerns, we don’t think it is a perfect solution to bundle MDT functionality and LPP functionality together to realize the new feature. So we propose UE reports ASN.1 encoded WLAN/BT measurement results to eNB instead of to E-SMLC. Whether eNB or TCE calculates the UE location depends on implementation.

- UE reports ASN.1 encoded WLAN/BT measurements to eNB instead of to E-SMLC.

But from reducing standardization efforts point of view, we should reuse the measurement configurations already defined in LPP specifications as much as possible.

- Reuse the measurement configurations already defined in LPP specifications as much as possible.

2.1.2	General impacts to the latest MDT specifications

Before we make detailed design, two fundamental issues should be explicit.

Issue 1: How BT/WLAN measurements are reported? In MeasurementReport message or location information IE?

In the new feature, BT/WLAN measurement results is used for positioning purpose instead of for cell reselection or handover purpose. From the intended use point of view, they should be regarded as location information. But literally, they are just measurement results rather than GNSS location coordinates.

For Immediate MDT, the configuration for UE measurements is based on the existing RRC measurement procedures for configuration and reporting. If BT/WLAN measurements is specified to be carried in MeasurementReport message, a new measurement object should be introduced in TS36.331 which would bring a large specification impact. It is unnecessary to make such a great modification for a small feature. So we think at least for Immediate MDT, BT/WLAN measurement results is carried in location information IE.

- For Immediate MDT, BT/WLAN measurements is carried in location information IE.

For logged MDT, such specification impact does not exist since both logged measurement configuration and reporting parameters are fixed which do not depend on dynamic configuration. Therefore, both options (in MeasurementReport message and location information) are feasible. To ensure consistency of UE behaviour in both logged MDT and Immediate MDT, taking BT/WLAN measurements as location information may be better.

- RAN2 to decide the reporting form of BT/WLAN measurements in logged MDT (MeasurementReport message or location information).

Issue 2: Whether the feature is required in both Immediate MDT and logged MDT cases or not?

The goal of the feature is to optimize indoor network performance using high accuracy location information calculated from BT/WLAN measurement results. In indoor environment, UE is probably in a poor coverage area, for example in the basement or at the edge of the rooms, where UE is unable to report BT/WLAN measurement results to network in time. Hence introducing the feature for logged MDT is a must.

As proposed above, for Immediate MDT the BT/WLAN measurement results is carried in location information which is an optional field. If detailed location information (BT/WLAN measurement results) is available, the UE could provide it in RRC measurement report with less power consumption. The impact to UE is less, but the value to operator is meaningful. The operators would have a more objective view of indoor network performance considering not only the MDT measurements coming from idle UEs but also that from connected UEs.

- Both Immediate MDT and logged MDT support the new feature.

2.2	Detailed aspects

2.2.1	General procedures

Figure 1: Procedures for logged MDT

Figure 2: Procedures for immediate MDT

For Logged MDT, Network initiates the procedure to UE in RRC Connected by sending LoggedMeasurementConfiguration message, which is used to transfer configuration parameters. Then the UE may enter into RRC idle state. During connection establishment, the UE may indicate the availability of Logged MDT measurements to E-UTRAN by means of an indicator in RRCConnectionSetupComplete message. If the E-UTRAN wants the MDT measurements, it may command UEInformationRequest to retrieve these measurements from the UE. And for Immediate MDT, the configuration for UE measurements is based on the existing RRC measurement procedures for configuration and reporting with some extensions for location information. In our understanding, the basic MDT measurement configuration and reporting procedures could be followed for indoor location purpose without any enhancement.

To indicate the network problems for a specific location, RLF report is extended in Rel-10 to include the available location information on where RLF detected, and RCEF reporting is introduced in Rel-11 to indicate where the UE RRC Establishment is failed to one eNB. In our view, the enhanced procedures are also meaningful indoors and should also be supported.

- The existing MDT measurement configuration and reporting procedures (i.e., logged MDT, immediate MDT, RLF report and RCEF report) can be used for indoor location purpose.

2.2.2	Measurement collection

Generally we should reuse the measurement configurations already defined in LPP specifications as much as possible. Using BT/WLAN measurements to calculate UE location can be regarded as UE-Assisted positioning mode in which location calculation is done by the network. In LPP specification, BT and WLAN measurement information that may be transferred from the UE to the E-SMLC are listed in Table 1 and 2 respectively. For BT case, the UE would report the MAC addresses of the measured BT beacons and associated RSSI. For WLAN case, the UE would report the BSSID/SSID of the measured WLAN APs, and associated RSSI or RTT.

Table 1: BT measurement information that may be transferred from the UE to the E-SMLC in LPP

| Bluetooth measurement Information |  |
| --- | --- |
| MAC Address | mandatory |
| Received Signal Strength (RSSI) | optional |
| Time Stamp | optional |
| Measurement characteristics | optional |

Table 2: WLAN measurement information that may be transferred from the UE to the E-SMLC in LPP

| WLAN measurement Information |  |
| --- | --- |
| BSSID | mandatory |
| SSID | optional |
| Received Signal Strength (RSSI) | optional |
| Round Trip Time (RTT) | optional |
| Time Stamp | optional |
| Measurement characteristics | optional |

Similarly for the Bluetooth/ WLAN based RF fingerprint in MDT, at least RSSI or RTT and device identifiers should be collected and reported. Besides, time stamp may also useful when other MDT measurements (e.g., measured by eNBs) and Bluetooth/ WLAN measurements (i.e., location information) are provided separately.

- For BT case, the UE should report the MAC addresses of the measured BT beacons and associated RSSI.

- For WLAN case, the UE should report the BSSID/SSID of the measured WLAN APs, and associated RSSI or RTT.

For Logged MDT, the measurement quantities for downlink pilot strength measurement logging are fixed and the principle should still be followed in this feature. Specifically the logged MDT measurement quantities should further include RSSI for Bluetooth beacons and both RSSI and RTT for WLAN APs.

For Immediate MDT, seven measurements (M1-M7) are defined in the specification and which one to use is configured via RRCConnectionReconfiguration message. To support BT/ WLAN measurements, a new measurement M8: RSSI measurement by UE, should be defined.

- For logged MDT, measurement quantities should further include RSSI for Bluetooth beacons and both RSSI and RTT for WLAN APs.

- For Immediate MDT, define a new measurement M8: RSSI measurement by UE.

As we all know, the estimated location would be more accurate with more measurement results. To ensure the location accuracy and further the indoor MDT performance, collections for multiple WLAN APs or Bluetooth beacons are necessary. The number of WLAN APs or Bluetooth beacons to be logged should be limited by a fixed upper limit like other MDT measurement categories. In TS36.331, maximum number of WLAN IDs to report is 32 (maxWLAN-Id-Report-r14). We think this value can also be applied for the WLAN based RF fingerprint. While for Bluetooth beacons, the maximum number has not been specified. But we can follow the upper limit of Bluetooth measurements defined in [3], which is 32.

- The UE could log the measurement results for up to 32 WLAN APs.

- The UE could log the measurement results for up to 32 Bluetooth beacons.

2.2.3	UE capability

Before enabling the BT/WLAN based RF fingerprint function to a specific UE, E-UTRAN need to know whether the UE support Bluetooth /WLAN measurements or not. So 2 bits should be defined for UE capabilities, one is used to indicate the capability for support of Bluetooth measurements, and the other is used to indicate the capability for support of WLAN measurements.

- 2 bits should be defined in UE capability to indicate whether the UE support Bluetooth/WLAN measurements or not.

2.2.4	Impacts to UE memory size

As suggested in the previous section, measurement results for up to 32 WLAN APs and up to 32 Bluetooth beacons could be reported simultaneously. Assuming the WLAN/ Bluetooth measurement Information have the following size:

| Bluetooth measurement Information |  |
| --- | --- |
| MAC Address | 6 byte |
| Received Signal Strength (RSSI) | 1 byte |
| WLAN measurement Information |  |
| BSSID/ SSID | 6 byte +32 byte |
| Received Signal Strength (RSSI) | 1 byte |
| Round Trip Time (RTT) | 3byte + 11 bit |

The maximum memory size consumed for 32 WLAN APs and 32 Bluetooth beacons is 1612 bytes. Considering the minimum memory for log storage is 64kB which was agreed to be expanded from 32kB in RAN2#72bis [5], we think the current UE memory is still abundant.

- Not need to expend UE memory size for log storage.

3	Conclusion

In summary, our considerations and recommendations are list below:

- Modifications to the LPP specifications are required, for example:

- two mandatory fields should be introduced to ensure the matching between logged MDT measurements and location information;

- a minimum UE memory for location information storage should be introduced;

- signalling message carrying the BT beacon list should be defined to indicate UE to measure a specific group of BT beacons.

- Additional requirements on UE and network are needed if we use LPP mechanism to obtain UE location.

- The procedure is complex and the overhead is large if we use LPP mechanism to obtain UE location.

- UE reports ASN.1 encoded WLAN/BT measurements to eNB instead of to E-SMLC.

- Reuse the measurement configurations already defined in LPP specifications as much as possible.

- For Immediate MDT, BT/WLAN measurements is carried in location information IE.

- RAN2 to decide the reporting form of BT/WLAN measurements in logged MDT (MeasurementReport message or location information).

- Both Immediate MDT and logged MDT support the new feature.

- The existing MDT measurement configuration and reporting procedures (i.e., logged MDT, immediate MDT, RLF report and RCEF report) can be used for indoor location purpose.

- For BT case, the UE should report the MAC addresses of the measured BT beacons and associated RSSI.

- For WLAN case, the UE should report the BSSID/SSID of the measured WLAN APs, and associated RSSI or RTT.

- For logged MDT, measurement quantities should further include RSSI for Bluetooth beacons and both RSSI and RTT for WLAN APs.

- For Immediate MDT, define a new measurement M8: RSSI measurement by UE.

- The UE could log the measurement results for up to 64 WLAN APs.

- The UE could log the measurement results for up to 32 Bluetooth beacons.

- 2 bits should be defined in UE capability to indicate whether the UE support Bluetooth/WLAN measurements or not.

- Not need to expend UE memory size for log storage.

References

- RP-172820   New WID: Bluetooth/WLAN measurement collection in MDT

- 3GPP TS 37.320, Evolved Universal Terrestrial Radio Access (E-UTRA); Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; Stage 2 (Release 14)

- 3GPP TS 36.355, Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol (LPP)

- 3GPP TS 36.331, Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification

- R2-110282, On memory size limitation for Logged MDT, NTT DOCOMO, INC., RAN2#72bis