3GPP TSG RAN WG2 Meeting #101    	                            R2-1803442

Athens, Greece, 26th Feb - 2nd March 2018

Agenda Item:	10.4.5.6

Source: 	Sony

Title:	Considerations on Multi-beams Operation & Paging Resource Allocation

Document for:	Discussion/decision

# Introduction

After the RAN#71 meeting, a new WI was agreed [1] main objective of which is to develop a new radio (NR) access technology.

Currently the standardization of NR is on-going in 3GPP Rel-15. The non-standalone EN-DC had its first release in December 2017, whereas NR stand-alone will be ready in June 2018.

Regarding the NR paging mechanism, the work on Idle/Inactive paging has not progressed much yet. Both RAN1 and RAN2 WGs have only made a few agreements so far. The basic paging mechanism in FR1, sub 6 GHz can be expected to have similar behaviour as in LTE. One difference is for FR2, higher frequencies (mmWave), is the usage of multi-beam operation.

In RAN2#100 meeting [2], RAN2 received LS from RAN1 [3]about NR paging mechanism, and responded to it. A summary of RAN1 LS and RAN2 response is as follows.

LS from RAN1 RAN1 is considering the following options on NR paging mechanisms: - Option 1: Paging DCI followed by Paging Message - Note: These do not imply that they are consecutive - Option 2: Paging group indicator triggering UE feedback and Paging DCI followed by Paging Message - Option 3: Paging group indicator and Paging DCI followed by Paging Message - Option 4: Paging DCI indicates use of Option 1 or 2. Q: Per RAN2 definition on Paging Occasion, are the Paging DCI and Paging Message sent in the same Paging Occasion or they can be sent in the different Paging Occasions. RAN2 Response In LTE, one Paging Occasion (PO) is a subframe where there may be P-RNTI transmitted on PDCCH or MPDCCH or, for NB-IoT on NPDCCH addressing the paging message. PO defines a number of slots where the UE has to monitor the PDCCH (TS 38.300 section 9.2.5). RAN2 has not decided whether or not the message is in the same slot(s). RAN2 assume that RAN1 can make this decision. RAN2 think that paging design should consider UE power consumption.

At the RAN1 #90b meeting, the following agreements regarding paging in NR were made.

- At least Option 1 (Paging scheduling DCI followed by Paging Message) is supported

  - From RAN1 perspective, paging scheduling DCI and Paging Message are sent at least in the same slot

  - From RAN1 perspective, NR supports LTE-like UE grouping where UE is specifically configured of its PO/slot. This is considered part of Option 1.

    - Details of UE grouping are up to RAN2

This agreement also implies that the NR paging mechanism is essentially “LTE-like” paging mechanism for both FR1 and FR2. Recently, in RAN1 AH1801 [4], the following agreements were made:

Agreements:

- The association between actual transmitted SSB and the monitoring window of PDCCH containing the Paging DCI and the broadcast OSI DCI can be respectively configured via RMSI.

  - It is up to RAN2 on how to do the above configuration. Send an LS to RAN2 (R1-1801248, which is approved and final LS is in R1-1801280)

The default association between SSB index and monitoring window of PDCCH containing a Paging DCI and a broadcast OSI DCI is same as that between SSB index and its RMSI monitoring window.

In this contribution, we will elaborate on the how to map the multi-beams pattern in the SSB towards the paging timeslot in the paging occasion.

# Discussion

The generic operation of a UE before establishing RRC connection is shown in Figure 1 below. First, in case MT traffic, i.e. downlink traffic initiated from the network, the UE is receiving a paging message at a predetermined time (DRX) followed by random access procedure, and then RRC connection for data communication. However, prior to paging mechanism, the UE needs to synchronize with the gNB. In a multi-beam operation, it is well understood that the UE has also identified its best beam to maintain communication with the gNB. The synchronization signal for each beam is carried within a synchronization signal block (SSB). In this example, there are (L=8) multi-beams actually transmitted SSBs.

Figure 1: UE establishing RRC connection initiated by synchronization process

As for the paging mechanism, the UE has identified its best beam as described above. However, the gNB does not know the best beam to the intended UE being paged. Thus, the gNB is known to perform paging with beam sweeping ,i.e. it has to transmit the same paging message multiple times in each beam The basic principle is that each paging occasion (PO) is subdivided into paging timeslots, in which the different beams to used are spread amongst. The paging message is then repeated once for every beam in the paging occasion see Figure 2 below.

Figure 2: Paging with gNB Tx beamsweeping

In case the UE has been able to synchronize the one of the TX beams from the gNB, there are methods for the UE then to know in which time slot it should listen for the paging message. This means that the UE does not have to be awake for the whole paging occasion, but only for the specific paging timeslot, in which the UE are able to receive the best beam from the gNB. The UE has been implicitly informed by the gNB on the mapping of gNB Tx beam and the paging time slot resources information.

In LTE system, the assumption is the base-station transmits paging message in a paging occasion using omni directional or relatively wider beam antenna. In NR system, the base-station could have up to L=64 narrow-beams for SSB transmission directing to different directions. Considering NR rel.15 (Dec 17) has agreed to adopt “LTE-like” NR paging mechanism then the consequent is the paging message is transmitted repeatedly up to 64 times in various directions. This will consume a lot of paging resources and potentially increase the latency if it is not configured properly.

Observation 1: To apply and follow the SSB beam sweeping pattern can lead to the usage of high number of paging resources.

In order to minimize the size of paging resources, we propose so that the multi beam pattern as in SSB transmissions is mapped to the paging time slot where it can be based on many to one mapping as shown in Figure 3. Here, two SSBs are mapped to one paging timeslot. Hence, the gNB simultaneously transmit two beams for the paging message transmission. In practice, it can also be the case where the gNB uses one wider beams corresponds to the two narrowbeams.

Figure 3: Many (e.g. 2 SSBs) map to one PO.

Proposal 1: There is an association between the number of paging time slot within a PO and the actually transmitted SSBs.

Proposal 2: There is a mapping procedure or relation between the actually transmitted SSBs and the beam direction in each paging time slot. The relations are at least one-to-one mapping and many-to-one mapping.

In addition, the multi beam pattern as in SSB transmissions can be mapped not only in time, but also in frequency, where the mapping rule is the frequency direction first followed by the time direction. This operation is illustrated in Figure 4A. Note, the paging timeslot here, it can be all paging message (paging DCI + paging data) or paging DCI only.

In case the initial bandwidth part (BWP) is wide enough then the FDM of paging time slot is all located within the initial BWP (BWP that contain SSB transmission). It is placed consecutively (no gap) and numbered from the lowest to the highest frequency. (See Figure 4A)

In case the initial BWP is relatively small, it may not be possible to FDM the whole paging message or even to do FDM of the paging data part. In case the paging control (DCI) and paging data are not in the same slot, we can also consider where the paging data part is transmitted in different BWP. It depends on the configuration in the paging control part. Thus, the paging data location can be dynamically arranged by the gNB.

Figure 4: Paging FDM operation and SSB mapping

Proposal 3: The multi beam pattern as in SSB transmissions can be mapped not only in time, but also in frequency, where the mapping rule is frequency direction first followed by time direction. See fig 5A.

Proposal 4: For simplicity, all of the paging DCI and message are in the active initial bandwidth part (BWP). Other options can still be considered and for further studies (FFS). (e.g. utilization of other BWPs for paging transmission).

# Conclusion

In this contribution we have elaborated on the mapping procedure of the multi-beams pattern in the SSB towards the paging timeslot in a paging occasion. Our proposals are summarized below:

Proposal 1: There is an association between the number of paging time slot within a PO and the actually transmitted SSBs.

Proposal 2: There is a mapping procedure or relation between the actually transmitted SSBs and the beam direction in each paging time slot. The relations are at least one-to-one mapping and many-to-one mapping.

Proposal 3: The multi beam pattern as in SSB transmissions can be mapped not only in time direction, but also in frequency direction, where the mapping rule is frequency direction first followed by time direction.

Proposal 4: For simplicity, all of the paging DCI and message are in the active initial bandwidth part (BWP). Other options can still be considered and for further studies (FFS). (e.g. utilization of other BWPs for paging transmission).

# References

- RP-170847, "New WID on New Radio Access Technology ", NTT DoCoMo

- RAN2#100 meeting

- R2-1712118, LS on NR Paging Mechanisms (R1-1719164; contact: Huawei)

- RAN1 AH1801 meeting