3GPP TSG-RAN WG2 #101	R2-1803745

26th February – 2nd March 2018

Athens, Greece

Source:                    	NTT DOCOMO, INC.

Title:  	[D323/324] Contention-based RA resource for Beam Failure Recovery

Document for:        	Discussion and decision

Agenda Item:         	10.4.3.2	ASN.1 issue document

- Introduction

For Beam Failure Recovery (BFR), Contention-Based RA (CBRA) is supported as well as Contention-Free RA (CFRA). In the current BeamFailureRecoveryConfig, not all of the parameters required for CBRA are included. This paper attempts to clarify how CBRA for BFR is configured for a UE.

- Discussion

As per the latest L1 parameter list, the following CBRA parameters for BFR can be set to different value than the one for the other purposes of CBRA:

-	rootSequenceIndex:	only the short sequence is supported for BFR.

-	beamFailureCandidateBeamThreshold:	This threshold is applied for CSI-RS. The threshold for SSB is the same as present in RACH-ConfigCommon (rsrp-ThresholdSSB).

-	Parameters grouped into RACH-ConfigCommonGeneric:

-	zeroCorrelationZoneConfig;

-	preambleRcvedTargetPower;

-	preambleTransMax;

-	powerRampingStep;

-	ra-ResponseWindow.

The rest of the CBRA parameters for BFR are the same as configured in RACH-ConfigCommon, e.g. prach-ConfigurationIndex, msg1-FDM/FrequencyStart, etc. Figure 2-1 roughly illustrates how CBRA resource is configured for BFR and the other purposes.

Figure 2-1: Overall picture on CBRA configuration for BFR and other purposes

Although the benefit of doing so is to leverage some of the parameters configured for the other purposes in terms of CBRA, the current structure may incur an IOT issue in future as to which parameter is applied for BFR as well as the other purposes.

To make the CBRA resource for BFR clearer, an alternative approach can be envisaged such that RACH-ConfigCommon inclues the CBRA parameters applied for BFR as illustrated in Figure 2-2.

Figure 2-2:	Alternative RACH-ConfigCommon structure

One potential drawback of this alternative approach is to increase the broadcast overhead assuming that RACH-ConfigCommon is broadcast somewhere in SIB for NRstandalone. Nonetheless, If RACH-ConfigCommon is broadcast in SIB not required for cell selection (e.g. SIB2 in LTE), the broadcast overhead can be mitigated by setting the longer SI scheduling period.

- Summary and proposal

Based on the analysis described in Section 2, the following is proposed:

Proposal 1:		CBRA parameters specific to BFR are also included in RACH-ConfigCommon, which are:

1) RACH-ConfigCommonGeneric for BFR;

2) rootSequenceIndex-BFR.

Proposal 2:	If proposal 1 is agreed, RACH-ConfigCommonGeneric for BFR and rootSequenceIndex-BFR are removed from BeamFailureRecoveryConfig.

- References

[1] R2-1801719, “LS on correction to RRC parameters for NR,” RAN1.

Annex A:
ASN.1 structure on RACH-ConfigCommon/Dedicated and BeamFailureRecoveryConfig