3GPP TSG-RAN WG2#101	                                                                       R2-1804488

Sanya, China, 16th-20thApril 2018                                        Revision of R2-1802150

Source:	CATT

Title:	Further Clarification on PHR MAC CE for EN-DC

Agenda Item:	10.3.1.12

Document for:	Discussion and Decision

# Introduction

At the previous RAN2 meeting, the PHR MAC CE for EN-DC has been discussed based on [1]and it was concluded that there is no need to clarify further on the UE behavior in the MAC specification. But we think the current MAC specification is still unclear for EN-DC case. In this contribution, we will further discuss this issue.

# Discussion

## Rules for UE determining the CC bitmap length in PHR MAC CE for EN-DC

In previous RAN2 meetings, many agreements were reached related to the PHR MAC CE, listed below:

Agreements on RAN2#99bis UP session - One octet of bitmap is used for indicating the presence of PH per SCell when the highest ScellIndex ofScell with configured uplink is less than 8. Otherwise four octets are used. Editor’s note “it depends on whether we support 32 carriers” - For EN-DC the assumption is that the cell index space is shared between LTE and NR. [CB for CP]

Agreements on RAN2#100 CP session - SCellIndex is unique in the UE (i.e. across NR and LTE SCells) - Each node allocates SCellIndex for its own cells (from a set of SCellIndices available to that node) without involvement of the other node at the time of allocation of the SCell - MN decides the SCellIndex range available for MN and SN, and informs the SN range to the SN. This is a per UE configuration. - The MN provides the SN the range of SCellIndex to be used by signalling a start value and a stop value if the range. 32 SCellIndex range is used between the MN and the SN. The UE uses “Dual Connectivity PHR MAC Control Element supporting 32 serving cells with configured uplink” for PH reporting.

In our understanding, the agreements made on RAN2#99bis UP session do not take the EN-DC case into account. It is just for NR. If we directly apply it to EN-DC case, the length of CC bitmap in the Multiple Entry PHR MAC CE will depend on the highest SCellIndex of SCell with configured uplink. But now referring to the above CP agreement, clearly the highest SCellIndex of SCell with configured uplink can be different from the maximum SCellIndex value of the SCellIndex range allocated by MN. And it could also be viewed as contradicting the other CP agreement that the UE uses “Dual Connectivity PHR MAC Control Element supporting 32 serving cells with configured uplink”.

Observation 1: It is clear that the highest SCellIndex of SCell with configured uplink can be different from the maximum SCellIndex value of the SCellIndex range allocated by MN.

Hence we need to further check how to understand the agreement reached on RAN2#100 CP session. There are two possible interpretations:

- Alt 1: The agreement made on RAN2#100 CP session overrides the agreement made on RAN2#99bis UP session. That is to say for EN-DC, no matter for MN or SN, UE always uses 4 octets CC bitmap in the Multiple Entry PHR MAC CE.

- Alt 2: Since, strictly speaking, NR MAC no longer supports a “Dual Connectivity PHR MAC Control Element”, the agreement made on RAN2#100 CP session is only applicable for LTE MN, and the agreement made on RAN2#99bis UP session applies to NR SN.

The above CP agreement “The MN provides the SN the range of SCellIndex to be used by signaling a start value and a stop value in the range” is currently captured in RRC specification by the I.E. servCellIndexRangeSCG of the SCG-ConfigInfo message as follows:

ConfigRestrictInfoSCG ::=	SEQUENCE {

allowedBC-ListMRDC		BandCombinationIndexList		OPTIONAL,

allowedBPC-ListMRDC		BPC-IndexList				OPTIONAL,

powerCoordination-FR1		SEQUENCE {

p-maxNR			P-Max			OPTIONAL,

p-maxEUTRA		P-Max				OPTIONAL

}																				OPTIONAL,

servCellIndexRangeSCG		SEQUENCE {

lowBound		ServCellIndex,

upBound			ServCellIndex

}																	OPTIONAL,   -- Cond SN-Addition

maxMeasFreqsSCG-NR		INTEGER(1..maxMeasFreqsMN)		OPTIONAL,

...

}

servCellIndexRangeSCG Range of serving cell indices that SN is allowed to configure for SCG serving cells.

In the following, we give two examples of the above configuration usage to illustrate the difference of the above two alternatives. Assuming UE1 is configured with EN-DC, the SCellIndex range of MeNB is [1, 5] and the SCellIndex range of SeNB is [lowBound=6, upBound=32]. And currently, the maximum SCellIndex with configured uplink by SN is 6. The PHR MAC CE formats for these two options are shown in the following Figure:

Figure -1 Illustration of different understandings on the agreement of RAN2#100 CP session

Comparing the two alternatives, restricting the agreement made on RAN2#100 CP session to LTE MN can minimize the impact on LTE specification. However it might be restrictive to impose it on the NR side as well considering the PHR MAC CE overhead. Hence we prefer Alt2.

Proposal 1: Suggest RAN2 to confirm the following rule for UE to determine the length of CC bitmap in PHR MAC CE for EN-DC:

- For LTE MN, UE always uses Dual Connectivity PHR MAC Control Element supporting 32 serving cells with configured uplink for PH reporting.

- For NR SN, UE uses Multiple Entry PHR MAC CE for PH reporting and the length of CC bitmap in the PHR MAC CE depends on the highest SCellIndex of SCell with configured uplink.

## Handling of multiple PHR MAC CE for NR SN

If Proposal 1 is agreed, it is obvious the length of CC bitmap in the Multiple Entry PHR MAC CE for NR SN will be flexible. It depends on the highest SCellIndex of SCell with configured uplink. If it is less than 8, one octet will be used; otherwise four octets are used.

The agreement made on RAN2#100 CP session states that for EN-DC, MN decides the SCellIndex range available for MN and SN and informs the SN range to the SN. If the minimum SCellIndex value of the SCellIndex range of NR SN (namely lowBound) can be assumed to always be larger than the maximum SCellIndex value of the SCellIndex range of LTE MN, then the NR SN can determine the CC bitmap based on knowledge of its own SCell(s) with configured uplink. No enhancement on MAC is needed.

Observation 1: In EN-DC, if we restrict the minimum SCellIndex value of the SCellIndex range of NR SN (namely lowBound) to be always larger than the maximum SCellIndex value of the SCellIndex range of LTE MN, then NR SN can determine the CC bitmap length in Multiple Entry PHR MAC CE without any enhancement.

But this restriction is not assumed so far and it is not precluded by the above RRC configuration of the SN SCellIndexrange with servCellIndexRangeSCG that MN configures SN with e.g. cell index range [lowBound=6, upBound=20] leaving the possibility for MN to configure its own cells with cell indexes picked from [21, 32] range. Furthermore, considering the further extension of NR-NR DC, both MN and SN will need to determine the length of CC bitmap once it receives the Multiple Entry PHR MAC CE. But if we stick to the rule that each node allocates SCellIndex for its own cells (from a set of SCellIndices available to that node) without involvement of the other node at the time of allocation of the SCell, the node with small SCellIndex range will not know the highest SCellIndex configured for UE. Hence it cannot determine which CC bitmap length will be used by UE in the Multiple Entry PHR MAC CE. In order to solve this issue, three options can be considered:

- Option1: Blind decoding.

- Option 2: MN and SN exchange the configured maximum cell index for each UE working under EN-DC.

- Option 3: Introduce two LCIDs for Multiple Entry PHR MAC CE, one is used for Multiple Entry PHR MAC CE with 1-byte CC bitmap; and another is used for Multiple Entry PHR MAC CE with 4-byte PHR MAC CE.

For Option 1, blind decoding will increase the network implementation complexity. Option 2 clearly defeats the purpose of  the agreement made on RAN2#100 meeting that each node allocates SCellIndex for its own cells (from a set of SCellIndices available to that node) without involvement of the other node at the time of allocation of the SCell. Hence Option 3 is the simplest, most future-proof, and the specification effort is acceptable.

Thus, it is suggested to introduce two LCIDs for the Multiple Entry PHR MAC CE, one for 1 octet CC bitmap and one for 4 octets CC bitmap.

Proposal 2: Two LCIDs should be introduced for Multiple Entry PHR MAC CE, one for 1 octet CC bitmap and one for 4 octets CC bitmap.

# Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: Suggest RAN2 to confirm the following rule for UE to determine the length of CC bitmap in PHR MAC CE for EN-DC:

- For LTE MN, UE always uses Dual Connectivity PHR MAC Control Element supporting 32 serving cells with configured uplink for PH reporting.

- For NR SN, UE uses Multiple Entry PHR MAC CE for PH reporting and the length of CC bitmap in the PHR MAC CE depends on the highest SCellIndex of SCell with configured uplink.

Proposal 2: Two LCIDs should be introduced for Multiple Entry PHR MAC CE, one for 1 octet CC bitmap and one for 4 octets CC bitmap.

The corresponding TPs for Proposal 1and 2 are provided in [2][3].

# Reference

- R2-1801539	 PHR MAC CE for EN-DC,  CATT

- R2-1804523 CR to 36.321 on  PHR MAC CE for EN-DC, CATT

- R2-1804524 CR to 36.321 on  PHR MAC CE for EN-DC, CATT