3GPP TSG-RAN WG2 Meeting#101bis		R2-1804678

Sanya, China, 16th – 20th April 2018

Source:	vivo

Title:	Switching BWP during measurement Gap

Agenda Item:	10.3.1.2

Document for:	Discussion and Decision

- Introduction

In TS38.321 [1], if the bwp-InactivityTimer associated with the active DL BWP expires the UE behavior is captured as the below:

5.15 Bandwidth Part (BWP) operation ….. 2> if the bwp-InactivityTimer associated with the active DL BWP expires: 3> if the default-DL-BWP is configured: 4> perform BWP switching to a BWP indicated by the default-DL-BWP. 3> else: 4> perform BWP switching to the initial DL BWP.

In this contribution we discuss whether the UE shall perform BWP switching if bwp-InactivityTimer expires during the measurement gap.

- Discussion

  - Measurement GAP in NR

In NR, when the bwp-InactivityTimer associated with the active DL BWP expires the UE will perform BWP switching. The specification does not consider the measurement gap, the UE will also perform BWP switching if the time point of expiry falls into the period of measurement gap. It is not useful to perform the BWP switching during the measurement gap because no data is sent during the measurement gap. Switching BWP will also impact the inter-frequency measurement gap. One simple way is that if the bwp-InactivityTimer expires during the associated FR measurement gap, the UE can consider the bwp-InactivityTimer expires at the end of the associated FR measurement gap.

Figure1: bwp-InactivityTimer expires during the measurement gap

Proposal: if the bwp-InactivityTimer expires during the associated FR measurement gap, the UE can consider the bwp-InactivityTimer expires at the end of the associated FR measurement gap.

  - Text proposal

5.14	Handling of measurement gaps

During a measurement gap, the MAC entity shall:

1>	not perform the transmission of HARQ feedback, SR, and CSI;

1>	not report SRS;

1>	not transmit on UL-SCH except for Msg3 as specified in subclause 5.4.2.2;

1>	if the bwp-InactivityTimer associated with the active DL BWP expires during the associated FR measurement gap [11]:

2>	consider the bwp-InactivityTimer associated with the active DL BWP expires at the end of the associated FR measurement gap [11].

1>	if the ra-ResponseWindow or the ra-ContentionResolutionTimer is running:

2>	monitor the PDCCH as specified in subclauses 5.1.4 and 5.1.5.

1>	else:

2>	not monitor the PDCCH.

- Conclusion

In this contribution, we provide impact analysis whether the UE shall perform BWP switching if bwp-InactivityTimer expires during the measurement gap and give our text proposal.

Proposal: if the bwp-InactivityTimer expires during the associated FR measurement gap, the UE can consider the bwp-InactivityTimer expires at the end of the associated FR measurement gap.

- References

- TS38.321