3GPP TSG-RAN WG2 Meeting #101bis	R2-1804878

Sanya, China, 16th April - 20nd April 2018

Source:	vivo

Title:	RRC configuration of SRB CA duplication

Agenda Item:	10.3.3.3

Document for:	Discussion and Decision

# Introduction

In the RAN2 discussion on the CA duplication, RAN2 achieved the following consensus:

=>	SRB duplication for CA is supported.  FFS LCID is allocated by RRC signalling and is not fixed.

In this contribution, we discuss in more details on the RRC configuration for SRB CA duplication.

# Discussion

## LCP restriction configuration for SRB CA duplication

Figure 1: SCell failure case of SRB

A specific SCell failure scenario (i.e. SCell-only configuration) as illustrated in Figure 1 is to allow that both P-RLC and S-RLC are configured (i.e. allowed for packet transmission) on only SCells, namely SCell_1 allowed for P-RLC and SCell_2 allowed for S-RLC respectively when CA duplication is activated. When CA duplication is activated, if both P-RLC (i.e. SCell_1) and S-RLC (i.e. SCell_2) encounters the SCell-RLF, the UE is not able to send the uplink RRC message (which is used to report the SCell-RLF of the SRB) via the SRB, as the UE is only allowed to transmit data via SCell_1 and SCell_2 for the SRB. Here we could have the following solutions for this case:

- Option 1: The UE triggers RRC reestablishment for the MCG SRB, or triggers SCG failure for the SCG SRB

- Option 2: The SpCell is always configured for the SRB when the SRB is configured with CA duplication.

According to the offline discussion summary [1], most companies prefer Option 2. For SRB duplication, as the PDCP duplication is always activated once the PDCP duplication is configured for the SRB, there is no primary leg or secondary leg for the SRB. Then the SpCell can be allowed for any leg of the SRB configured with CA duplication. As the LCP restriction is to restrict that the two duplicated packets are transmitted via different carriers, the SpCell should be configured for only one leg for the SRB.

Proposal 1: The SpCell is always included in the allowedServingCells of one LogicalChannelConfig for the SRB when the SRB is configured with CA duplication.

## LCID configuration

According to the RRC configuration in 38.331 [1] as quoted below, it is already possible that more than one RLC entities can be added for one SRB (e.g. SRB1/2/3) within the same cell group, and the logical channel identity are configurable for the extra leg for SRBs.

38.331: CellGroupConfig ::= SEQUENCE { cellGroupId CellGroupId, -- Logical Channel configuration and association with radio bearers: rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLC-ID)) OF RLC-Bearer-Config OPTIONAL, -- Need N rlc-BearerToReleaseList SEQUENCE (SIZE(1..maxLC-ID)) OF LogicalChannelIdentity OPTIONAL, -- Need N -- Parameters applicable for the entire cell group: mac-CellGroupConfig MAC-CellGroupConfig OPTIONAL, -- Need M physicalCellGroupConfig PhysicalCellGroupConfig OPTIONAL, -- Need M -- Serving Cell specific parameters (SpCell and SCells) spCellConfig SpCellConfig OPTIONAL, -- Need M sCellToAddModList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig OPTIONAL, -- Need N -- List of seconary serving cells to be released (not applicable for SpCells) sCellToReleaseList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellIndex OPTIONAL, -- Need N ... } -- The ID of a cell group. 0 identifies the master cell group. Other values identify secondary cell groups. -- In this version of the specification only values 0 and 1 are supported. -- FFS: This should be moved to be own IE section CellGroupId ::= INTEGER (0.. maxSecondaryCellGroups) RLC-Bearer-Config ::= SEQUENCE { -- ID used commonly for the MAC logical channel and for the RLC bearer. logicalChannelIdentity LogicalChannelIdentity, -- Associates the RLC Bearer with an SRB or a DRB. The UE shall deliver DL RLC SDUs received via the RLC entity of this -- RLC bearer to the PDCP entity of the servedRadioBearer. Furthermore, the UE shall advertise and deliver uplink PDCP PDUs of the -- uplink PDCP entity of the servedRadioBearer to the uplink RLC entity of this RLC bearer unless the uplink scheduling -- restrictions ('moreThanOneRLC' in PDCP-Config and the restrictions in LogicalChannelConfig) forbid it to do so. servedRadioBearer CHOICE { srb-Identity SRB-Identity, drb-Identity DRB-Identity } OPTIONAL, -- Cond LCH-SetupOnly reestablishRLC ENUMERATED {true} OPTIONAL, -- Need N rlc-Config RLC-Config OPTIONAL, -- Cond LCH-Setup mac-LogicalChannelConfig LogicalChannelConfig OPTIONAL, -- Cond LCH-Setup ... } LogicalChannelIdentity ::= INTEGER (1..maxLC-ID)

Thus we consider that there is no need to change anything in the RRC specification. RAN2 can confirm that the current RRC configuration can be reused to have a configurable LCID for the SRB configured with CA duplication.

Proposal 2: The LCID of the extra leg for the SRB configured with CA duplication is configurable.

Proposal 3: The current RRC configuration is reused for the configurable LCID of SRB.

# Conclusion

According to the analysis given above, we have the following proposals:

Proposal 1: The SpCell is always included in the allowedServingCells of one LogicalChannelConfig for the SRB when the SRB is configured with CA duplication.

Proposal 2: The LCID of the extra leg for the SRB configured with CA duplication is configurable.

Proposal 3: The current RRC configuration is reused for the configurable LCID of SRB.

# Reference

[1] R2-1801557, vivo, Summary of duplication support for SRBs for CA

[2] 3GPP TS 38.331, “NR; Radio Resource Control (RRC) protocol specification”