3GPP TSG-RAN WG2 Meeting #101bis	R2-1804911

Sanya, China, 16th - 20th April 2018

Agenda item:	10.3.1.7

Source:	Nokia, Nokia Shanghai Bell

Title:	MDBV in Uplink

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

One the characteristics of the 5QI of a Delay Critical GBR flow is the Maximum Data Burst Volume (MDBV), which denotes the largest amount of data that the 5G-AN is required to serve within a period of 5G-AN PDB (i.e. 5G-AN part of the PDB) [23.501]. This contribution discusses how it affects UL operation in NG-RAN.

# 2	Uplink Operation

## 2.1	Radio Admission Control

MDBV will be used by the gNB in radio admission control to assess how many Delay Critical GBR bearers can be supported in parallel in a cell. If Delay Critical GBR bearers started sending more data than initially planned by the MDBV, the gNB would have no other choices but to take the statistical variations into account when performing admission control, thereby reducing not only the number of Delay Critical GBR bearers it allows but any type of bearers for which resources need to be guaranteed. For instance, assuming an allocated cell capacity for DL to serve a Delay Critical GBR flow allows admitting 40 QoS flows at 160 octets MDBV within a 5 ms PDB, if the flows instead generate traffic at 320 octets size, the number of flows that can be served is bound to roughly halve (assuming there are no other radio bounds that would prevent serving flows at that rate).

Observation 1: allowing Delay Critical GBR bearers to exceed the MDBV based on which admission control is based would result in lower capacity for all GBR bearers.

To maximise cell capacity, MDBV enforcement is therefore obviously needed.

Proposal 1: MDBV is enforced.

In the downlink, it is up to the gNB to enforce the MDBV and this has no impact on RAN2. We will therefore focus on uplink operations affecting the UE behaviour in general and LCP in particular.

## 2.2	Baseline Mechanisms

Before checking how MDBV enforcement can be ensured, it is probably useful to remember how uplink data transmission works in NR. In uplink, the UE has a rate control function which manages the sharing of uplink resources between logical channels: the logical channel prioritisation function (LCP). RRC controls LCP by giving each logical channel a priority, a prioritised bit rate (PBR), a buffer size duration (BSD) and a list of possible restrictions (controlling which configured cells, numerologies, PUSCH transmission duration and Configured Grant type the logical channel can use).

With a token bucket algorithm, LCP ensures that the logical channel(s) are served in the following sequence:

1.   All relevant logical channels in decreasing priority order up to their PBR;

2.   All relevant logical channels in decreasing priority order for the remaining resources assigned by the grant.

NOTE:	the bucket size of each logical channel is set to PBR × BSD.

## 2.2	Delay Critical GBR Services

In order to understand how the MDBV impacts UL operation for the support of Delay Critical GBR services (i.e. URLLC services), we need to distinguish two cases:

1.	Scenarios where LCP restrictions can be put in place to isolate Delay Critical GBR services by mapping them on distinct resources (may it be cell, numerology…);

2.	Scenarios where LCP restrictions cannot isolate Delay Critical GBR services from each other and/or from other services.

In the first case, it is obvious that through scheduling (dynamic or SPS), the gNB has direct control of the Delay Critical GBR bearers and can always ensure that the MDBV is never exceeded.

Observation 2: when Delay Critical GBR services are mapped on distinct resources, the gNB can ensure that the MDBV is never exceeded.

Note that for that purpose, it would be useful to confirm that the configuration of allowedServingCells is not limited to radio configurations with duplication.

Proposal 2: confirm that allowedServingCells can be configured regardless of duplication.

In the second case, the gNB has no direct control of the bearers and they can freely compete for the grants. It seems reasonable to assume that Delay Critical GBR bearers will be configured with a high priority and will therefore see their PBR served first. However, in order to guarantee than one Delay Critical GBR bearer (possibly delayed) does not eat up the resources meant for another Delay Critical GBR bearer and that the PDB requirements are fulfilled, the BSD needs to be aligned to the PDB characteristic of the 5QI.

NOTE:	SA2 also defines that packets delayed more than the PDB are added to the PER.

Proposal 3: add ms5, ms10 and ms20 to the bucketSizeDuration in RRC.

Another problem that arises is that when additional resources are granted for the purpose of serving other bearers, there is no possibility to prevent a high priority Delay Critical GBR bearer from sending data beyond MDBV if it has data buffered. While it may be argued that since the resources are anyway used to send data and therefore not wasted for that particular UE, we need to remember the explanation given in section 2.1 above: it will result in lower capacity for other bearers as the gNB will need to allocate more resources to factor the statistical variations in. For instance, considering an URLLC service requiring 160 octets to be sent within 5ms every 1s, a PDB of 5ms, MDBV of 160octets and GBR of 1,280bit/s (160octets/s) would be configured. If the URLLC service ignored the MDBV, the gNB would have to cope with a bit rate up to 256,000bit/s (160octets/5ms).

Observation 3: MDBV cannot be enforced in scenarios where multiple Delay Critical GBR bearers are multiplexed on the same resource.

If RAN2 agrees with the observations, we suggest starting an email discussion until the next meeting to come up with an agreeable solution.

Proposal 3: have an email discussion until the next meeting to select a solution to enforce MDBV in uplink.

# 3	Conclusion

This contribution has discussed MDBV and has made the following observations and proposals.

Observation 1: allowing Delay Critical GBR bearers to exceed the MDBV based on which admission control is based would result in lower capacity for all GBR bearers.

Proposal 1: MDBV is enforced.

Observation 2: when Delay Critical GBR services are mapped on distinct resources, the gNB can ensure that the MDBV is never exceeded in uplink.

Proposal 2: confirm that allowedServingCells can be configured regardless of duplication.

Proposal 3: add ms5, ms10 and ms20 to the bucketSizeDuration in RRC.

Observation 3: MDBV cannot be enforced in scenarios where multiple Delay Critical GBR bearers are multiplexed on the same resource.

Proposal 3: have an email discussion until the next meeting to select a solution to enforce MDBV in uplink.