3GPP TSG-RAN WG2 Meeting #101bis 														R2-1805118

# Sanya, China, 16th – 20th April

Agenda item:	9.20

Source: 	ZTE, Sanechips

Title: 	The impact on user plane to extend the DRB number

Document for:	Discussion and Approval

Introduction

One new WID [1] was approved in RAN#78 to extend the number of DRB to be up to 15. One of the objects is as following:

- Extend the logical channel ID space by extending the MAC header structure (TS 36.321)

In fact current LCH id space is sufficient to introduce extra 7 DRBs. However as pointed in [2] it is a future proof solution to extend the LCH id when more number of DRBs is to be introduced. The first part of this paper will discuss how to extend the LCH id space.

Another issue is whether MAC spec should also extend the number of logical channel group. In NR system the LCH group is extended to 8 groups. The main reason is that more DRBs and more logical channels are introduced in NR system. After introducing more DRBs in LTE, this issue should be also discussed.

Extension of LCH id

Part1: Current LCH id situation, in the DL/UL 9/7 LCH id are left

The table of current LCH id space is shown in the Annex. The common part of the reserved LCH id space between DL and UL is as following:

01101-10011 (13~19)

Because already 8 LCH id are used for 8 DRBs, up to 15 DRBs can be supported by using up all these reserved 7 LCH id for newly introduced 7 DRBs assuming only one LCH id will be configured for one DRB. But it is not always the case. For example for the LTE_URLLC feature duplication in LTE PDCP layer is proposed. It means that two LCH id will be configured for one DRB. If LCH id is not extended then 15 DRBs can be possible only if no DRB is configured with duplication in user plane. For the sake of forward compatibility LCH id should be extended.

Proposal 1: A new table for extended LCH id should be introduced

It is obvious that the extended LCH id can only be used when both UE and network support extended LCH id. Network can only receive UE capability from core network after RRC connection is setup. For RRC connection re-establishment or RRC connection resume procedure network can know UE’s capability once UE is identified after receiving message3 during RACH procedure. From UE point of view, UE can use extended LCH id once it is enabled by network. Again for RRC connection setup procedure the earliest message is the first RRC connection reconfiguration. And for RRC connection re-establishment or resume procedure, it can be enabled in the RRC re-establishment or RRC connection Resume message. For all these 3 initial access procedure extended LCH id can be contained in MAC PDU only after UE is enabled by network via RRC message.

Proposal2: Extended LCH id can be contained in MAC PDU only after UE is enabled by network via RRC message in both uplink and downlink

There are 3 solutions discussed in paper [2].

Solution1: to change the R bit to be V bit to indicate new LCH id table

Solution2: same as solution1 with more R bits resulting from two BYTEs sub-header

Solution3: to rely on a special LCH id to indicate new LCH id space

We prefer solution1 with following modification i.e. to simply use the R bit in the MAC sub-header for MSB bit of the LCH id. One drawback of this solution as indicated by [2] is that there is no more R bit in the MAC sub-header. But we don’t think this is a big problem. There is a good summary of the LCH id left per release in the same paper [3] as following:

Table 1 – Remaining LCIDs in MAC per release.

| Release | Remaining LCIDs |  |
| --- | --- | --- |
|  | DL | UL |
| Rel-11 | 16 | 14 |
| Rel-12 | 15 | 10 |
| Rel-13 | 13 | 10 |
| Rel-14 | 10 | 7 |

In average 2 LCH id are defined for new MAC per release. If a new table containing 32 LCH id is introduced, it can last around 15 more Releases! Even at some point of time all the left LCH ids are consumed for some particular reason still the solution 3 in paper [2] i.e. to rely on a special LCH id to indicate new LCH id space is still feasible but with less R bits. Note after a new table is introduced, still some of the LCH id in the old table can be reserved for future extension in case of overhead critical cases.

Proposal3:  To change the R bit in current MAC sub-header to be M bit and the definition of the M bit  as following:

- M: The M field indicates MSB bit of  LCH id. There is one M field per MAC sub-header. The size of M field is one bit. The M field is set to “0” to indicate MSB bit of LCH id is zero i.e.  table 6.2.1-1 or table 6.2.1-2 is applied for this MAC sub-header otherwise table 6.2.1-1a or table 6.2.1-2a is applied.

As indicated by the definition of the M bit, it is set per MAC sub-header. Once extended LCH id is enabled it is possible to mix MAC sub-header with both legacy LCH id and new LCH id. That also means all the defined LCH id in the table 6.2.1-1 or table 6.2.1-2 can be kept as it is.

Proposal4: Once extended LCH id is enabled MAC sub-header with both legacy LCH id and new LCH id can be mixed within same MAC PDU.

The reserved LCH id in the old table should be still reserved. This is because not all the UE will support extended LCH id. This could be also helpful in case new MAC CE need be used in overhead sensible cases. Then it seems quite obvious that LCH id for more than 8 DRBs should be introduced in the new LCH id table. This also means that a UE supporting more than 8 DRBs should also support extended LCH id. But it doesn’t means a UE supporting extended LCH id should also support more than 8 DRBs i.e. for those UEs extended LCH id could be used for new MAC CE.

Proposal5:  LCH id for more than 8 DRBs should be introduced in new table.

Proposal5a: a UE supporting more than 8 DRBs should also support extended LCH id table

Here is an example of the new LCH id table:

Table 6.2.1-1a Values of LCID for DL-SCH

| Index | LCID values |
| --- | --- |
| 100000-100110 | Identity of the logical channel |
| 100111~111111 | reserved |

Table 6.2.1-2a Values of LCID for UL-SCH

| Index | LCID values |
| --- | --- |
| 100000-100110 | Identity of the logical channel |
| 100111~111111 | reserved |

This value range of new LCH id is different from current value range of logicalChannelIdentity . So it should be reflected in RRC spec also.

Proposal 5b: value range of extended LCH id should be also reflected in RRC specification

Introduce of more LCH group

The reason for NR system to introduce more LCH group is due to the fact that more logical channels will be configured compared to existing LTE system. Now the situation in LTE system seems also to ahead the same direction i.e. more and more logical channel will be configured considering more DRBs will be configured and potential duplication for LTE-URLLC is needed. But we also realize that more LCH groups will demands completely new design of the BSR format including short/truncated BSR format and long BSR format. On other hand there are only two meetings left to finish this WID and significant change of the BSR format will invite more papers which might not be able to be concluded before June. Therefore we slightly prefer to discuss this issue in next release.

Proposal6: RAN2 is kindly asked to discuss whether more LCH group should be introduced in Rel15.

Conclusion and proposals

Proposal 1: A new table for extended LCH id should be introduced

Proposal2: Extended LCH id can be contained in MAC PDU only after UE is enabled by network via RRC message in both uplink and downlink

Proposal3:  To change the R bit in current MAC sub-header to be M bit and the definition of the M bit  as following:

- M: The M field indicates MSB bit of  LCH id. There is one M field per MAC sub-header. The size of M field is one bit. The M field is set to “0” to indicate MSB bit of LCH id is zero i.e.  table 6.2.1-1 or table 6.2.1-2 is applied for this MAC sub-header otherwise table 6.2.1-1a or table 6.2.1-2a is applied.

Proposal4: Once extended LCH id is enabled MAC sub-header with both legacy LCH id and new LCH id can be mixed within same MAC PDU.

Proposal5:  LCH id for more than 8 DRBs should be introduced in new table.

Proposal5a: a UE supporting more than 8 DRBs should also support extended LCH id table

Proposal 5b: value range of extended LCH id should be also reflected in RRC specification

Proposal6: RAN2 is kindly asked to discuss whether more LCH group should be introduced in Rel15.

References

[1] RP-172835	New WI on extended number of bearers for LTE	Samsung

[2] R2-1712955	LCID space extension	Ericsson	discussion	Rel-15	TEI15

ANNEX

Table 6.2.1-1 Values of LCID for DL-SCH

| Index | LCID values |
| --- | --- |
| 00000 | CCCH |
| 00001-01010 | Identity of the logical channel |
| 01011-10100 | Reserved |
| 10101 | Activation/Deactivation of CSI-RS |
| 10110 | Recommended bit rate |
| 10111 | SC-PTM Stop Indication |
| 11000 | Activation/Deactivation (4 octets) |
| 11001 | SC-MCCH, SC-MTCH (see note) |
| 11010 | Long DRX Command |
| 11011 | Activation/Deactivation (1 octet) |
| 11100 | UE Contention Resolution Identity |
| 11101 | Timing Advance Command |
| 11110 | DRX Command |
| 11111 | Padding |
| NOTE: Both SC-MCCH and SC-MTCH cannot be multiplexed with other logical channels in the same MAC PDU except for Padding and SC-PTM Stop Indication |  |

Table 6.2.1-2 Values of LCID for UL-SCH

| Index | LCID values |
| --- | --- |
| 00000 | CCCH |
| 00001-01010 | Identity of the logical channel |
| 01011 | CCCH |
| 01100 | CCCH |
| 01101-10011 | Reserved |
| 10100 | Recommended bit rate query |
| 10101 | SPS confirmation |
| 10110 | Truncated Sidelink BSR |
| 10111 | Sidelink BSR |
| 11000 | Dual Connectivity Power Headroom Report |
| 11001 | Extended Power Headroom Report |
| 11010 | Power Headroom Report |
| 11011 | C-RNTI |
| 11100 | Truncated BSR |
| 11101 | Short BSR |
| 11110 | Long BSR |
| 11111 | Padding |