3GPP TSG-RAN WG2 #101bis	Tdoc R2-1805431

# Sanya, China, 16 – 20 April 2018

Agenda Item:	10.3.4.2

Source:	Ericsson

Title:	6-bit QFI and AS-NAS QFI mapping

Document for:	Discussion, Decision

# Introduction

The last RAN2 meeting agreed:

Agreements:

The presence of SDAP header is configured per DRB for UL or DL independently

For DL SDAP headers, two bits, one bit RQI and one bit RDI

For UL SDAP header at least a QFI field is present.  RQI and RDI are not needed. FFS on the size of QFI depending on DL

In the January 2018-AH meeting RAN2 agreed:

Agreements

=>	Support independent AS and NAS reflective QoS.

=>	From RAN2 perspective supporting up to 64 reflective flows per PDU session per UE is sufficient at the same time, so 6 bits QFI in SDAP.

=>	Ask SA2/CT1 if they expect to use more than 64 reflective flows per PDU session per UE at a time.  Indicate RAN2 agreement and strong need to have 6bits SDAP.   Questions will be included in SA2 LS from main session.

FFS if final QFI in CT1/SA2 is larger than 6 bits, a mechanism to remap NAS QFI to AS QFI may be needed

RAN2 received LS S2-182856 from SA2

For the SDAP sublayer, in order to keep the header contained within one byte, RAN2 has agreed to limit the number of QFI which can be signalled to 64. Therefore, RAN2 would like to understand if SA2 expects to use more than 64 reflective flows per PDU session per UE at a time.

SA2 acknowledges RAN2 decision and would like to inform RAN2 and CT1 that it has agreed to restrict the maximum number of concurrent QoS flows per PDU Session to 64 accordingly. SA2 has begun to discuss the implications from that limitation on control of the QoS flows mechanisms specified in TS 23.501 section 5.7.3.1, however, has not been able to reach a conclusion so far. SA2 will inform RAN2 and CT1 of the outcome of that discussion.

# Discussion

Last RAN2 meeting agreed that RDI and RQI bits are present on DL and not present on UL. The QFI on DL and UL header should be symmetric for reduced complexity.

6-bIt QFI is used for both DL and UL

As the SA2 agreed to restrict the number of concurrent QFIs per PDU session to 64, and a s aresult there is no need to have any 2-level mapping mechanism in AS.

No QFI remapping mechanism between AS and NAS QFI is introduced on RAN

No shorter version of QFI is introduced on RAN

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	6-bIt QFI is used for both DL and UL

Proposal 2	No QFI remapping mechanism between AS and NAS QFI is introduced on RAN

Proposal 3	No shorter version of QFI is introduced on RAN