3GPP TSG-RAN WG2 NR #101b Meeting                                            	R2-1805845

Sanya, China, 16th – 20th April 2018

Agenda item:	10.2.2.2

Source:	Samsung

Title:	Further considerations for bandwidth part

Document for:	Discussion & Decision

# 1. Introduction

RAN#75 in March approved a 5G WID [1] on New Radio access technology, which targets a unified framework for traffic with diverse QoS requirements e.g. enhanced mobile broadband (eMBB), massive machine-type-communications (mMTC), ultra-reliable and low latency communications (URLLC).

This contribution investigates remaining issue for signaling to support wider BW in NR; especially about flexible BW support based on bandwidth part.

# 2. Further issues to complete BWP operation

  - Inactive state

Regarding the following RAN2 agreements;

6     Upon transition to the idle state, UE releases all dedicated BWP configurations (and therefore UE applies the initial BWP configuration from system information of the cell where the UE is camped)

7	Upon transition to the inactive state, UE applies the initial BWP configuration from system information of the cell where the UE is camped.

UE does not release dedicated BWP configurations upon transition to the inactive state. However, it is hardly said that the dedicated BWP configuration is identical when UE moves across cells. It is useful only when UE transits back to active state at the same serving cell, but the gain could be small.

Proposal 1: UE releases all dedicated BWP configurations upon transition to inactive (i.e. no need to keep as UE likely resumes in other cell having different BWP configuration).

  - SUL case

Regarding SUL case, RAN1 discussed how UE can select which UL carrier. According to RAN1 agreement, UE determines a UL carrier based on measurement quality of the serving cell and trigger contention-based RA to perform initial access procedure on the selected UL carrier. For SCell, on the other hand, UE will not execute initial access procedure and network can decide which carrier is suitable to the UE. Therefore it should be supported that network indicates which carrier or which first UL active BWP among two is actually activated.

Proposal 2: If a SCell is configured with each first active UL BWP for both normal UL and SUL carrier, network indicates to UE which carrier or which first UL active BWP is actually activated.

# 3. Summary

Based on the discussion and identified observations in this contribution, we propose followings:

[Inactive state]

Proposal 1: UE releases all dedicated BWP configurations upon transition to inactive (i.e. no need to keep as UE likely resumes in other cell having different BWP configuration).

[SUL]

Proposal 2: If a SCell is configured with each first active UL BWP for both normal UL and SUL carrier, network indicates to UE which carrier or which first UL active BWP is actually activated.

References

[1] RP-170823, New WID Proposal: Work Item on New Radio (NR) Access Technology