3GPP TSG-RAN WG2 Meeting #101bis	R2-1805916

Sanya, China, 16th April – 20th April, 2018

Agenda Item	: 9.14.2 (Early data transmission)

Source	: LG Electronics Inc.

Title	: RRC-MAC interaction for fallback decision

Document for	: Discussion and Decision

1.	Introduction

After the UE initiates early data transmission, the UE may need to fallback to the legacy procedure if the UL grant is small to transmit user data. The UL grant is the UE MAC layer parameter while the UE RRC layer shall fallback to the legacy mechanisms if the grant is small. So, there should be interactions between the UE RRC and MAC layers.

In this contribution, we suggest possible scenarios of RRC and MAC interaction to fallback RRC procedure based on the UL grant.

2.	Discussion

When the UE initiates EDT, the UE may fallback to the legacy procedure depending on the UL grant size received in RA response. For CP solution, the fallback mechanism is simpler than that of UP solution because difference between the EDT and legacy procedures is Msg3. For UP solution, however, the UE may need to restore UE context, update security context, re-suspend radio bearers, etc. if the UE needs to fallback to the legacy mechanisms after resuming everything. Therefore, it might be better to minimize EDT operation until the UE receives the enough UL grant for EDT.

We considered two possible options for fallback scenario due to the small grant.

- Option 1:

The UE RRC layer only informs EDT indication to the lower layers and does not perform further operations. After receiving the indication from the MAC layer whether or not the UL grant is enough for early data transmission, the UE RRC layer restores stored UE AS context, resumes radio bearers using previously received resumeId or DRB Id(s), updates security context for the EDT procedure, and submits RRCConnectionResumeRequest to the lower layers.

- Option 2:

The UE RRC layer performs the resume procedure for EDT after informing of EDT indication to the lower layer. At this point, the UE assumes the EDT procedure would be continued. Then, if the MAC layer informs that the UL grant is small for EDT, the UE restores everything to perform legacy procedures.

<Fig. 1. Option 1 procedure>

<Fig. 2. Option 2 procedure>

Proposal. RAN2 to discuss whether or not Option 1 (resuming contexts after receiving the grant) could be the fallback solution for User Plane EDT.

3.	Conclusion

In this contribution, we discussed RRC and MAC interaction to fallback RRC procedure based on the UL grant. Our proposal is:

Proposal. RAN2 to discuss whether or not Option 1 (resuming contexts after receiving the grant) could be the fallback solution for User Plane EDT.