3GPP TSG-RAN2 Meeting #101bis 	R2-1806065

# Sanya, China, 16-22 April 2018

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.322 | CR | 0135 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of RLC release procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15 |  |  |  |  |  |  | Date: |  | 2018-04-02 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | The RLC entity has the stored RLC SDUs in rerodering buffer at the time of the RLC release. If those stored RLC SDUs are discarded at RLC release, service quality may be degraded. |  |  |  |  |  |  |  |  |
| Summary of change: |  | RLC release procedure is introduced for RLC AM, RLC UM and RLC TM. For RLC AM and RLC UM, in the transmitting side, all remaining RLC SDUs, RLC PDUs and segments RLC PDUs are discarded. In the receiving side, stored RLC SDUs are delivered to the upper layer, and all remaining RLC PDUs are discarded. For RLC TM, all stored RLC SDUs are discarded. Impact analysis: Impacted functionality: RLC release procedure for all RLC modes Inter-operability: If the network is implemented according to this CR while the UE is not, there is no inter-operability issue. If the UE is implemented according to this CR while the network is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RLC release procedure is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS36.323 CR 0229 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Beginning of changes

5.X	Release procedure

RLC release is performed upon request by RRC, and the function is applicable for AM, UM and TM RLC entities.

When RRC indicates that an RLC entity should be released, the UE shall:

-	if it is a transmitting TM RLC entity:

-	discard all RLC SDUs;

-	release the TM RLC entity;

-	if it is a receiving UM RLC entity:

-	when possible, reassemble RLC SDUs from UMD PDUs with SN < VR(UH), remove RLC headers when doing so and deliver all reassembled RLC SDUs to upper layer in ascending order of the RLC SN, if not delivered before;

-	discard all remaining UMD PDUs;

-	release the UM RLC entity;

-	if it is a transmitting UM RLC entity:

-	discard all RLC SDUs;

-	release the UM RLC entity.

-	if it is an AM RLC entity:

-	when possible, reassemble RLC SDUs from any byte segments of AMD PDUs with SN < VR(MR) in the receiving side, remove RLC headers when doing so and deliver all reassembled RLC SDUs to upper layer in ascending order of the RLC SN, if not delivered before;

-	discard the remaining AMD PDUs and byte segments of AMD PDUs in the receiving side;

-	discard all RLC SDUs and AMD PDUs in the transmitting side;

-	release the AM RLC entity.

End of changes