3GPP TSG-RAN WG2 Meeting #101	R2-1806449

Sanya, China, 16th - 20th April 2018	R2-18xxxxx

Agenda item:	10.4.1.8.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Access Control information

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

This is to capture agreements made based on offline discussion on Access Control information.

# 2	Discussion

Two variants of Access Control information encoding in SIB are investigated.

## 2.1 Option 1: Index option

In this option an index contains a reference for the set of barring parameters to be applied is provided for each access category. For example, one can provide following configuration

PLMN_1, Access category_1 uses barring parameter set S_2

PLMN_1, Access category_2 uses barring parameter set S_4,

…

PLMN_2, Access category_1 uses barring parameter set S_2,

PLMN_2, Access category_2 uses barring parameter set S_4,

…

PLMN_N, Access category_M-1 uses barring parameter set S_2,

PLMN_N, Access category_M uses barring parameter set S_4,

The calculations are made in the following way:

- N: Number of PLMNs

- M: Number of Access Categories

- K: Number of sets of barring parameters

The size needed to advertise this option: N*M*log2(K)

## 2.2 Option 2: Bitmap option

In this option there is bitmap for each barring parameter sets that indicates for which Access Categories to given barring parameter set should be applied. For example,

Bitmap B1 is used for barring parameter set S_1

Bitmap B2 is used for barring parameter set S_2

Bitmap B3 is used for barring parameter set S_3

Bitmap B4 is used for barring parameter set S_4

In each bitmap first bit applies to PLMN_1, AC_1, second bit to PLMN_1, AC_2, … M+1th bit applies to PLMN_2, AC1, … and N*Mth bit applies to PLMN_N, AC_M.

The calculations are made in the following way:

- N: Number of PLMNs

- M: Number of Access Categories

- K: Number of sets of barring parameters

The size needed to advertise this option: N*M*K

## 2.3 Size calculations

The table below presents the calculation with different number of access categories and set of barring parameters. The "worst case" presents the maximum size (with 12 PLMNs), while the example column presents the case of a single PLMN.

|  | 63 ACs 8 Sets of Barring |  | 63 ACs 4 Sets of Barring |  | 15 ACs 8 Sets of Barring |  | 15 ACs 4 Sets of Barring |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Worst Case | Example | Worst Case | Example | Worst Case | Example | Worst Case | Example |
| Number of Access Categories | 63 | 63 | 63 | 63 | 15 | 15 | 15 | 15 |
| Number of Set of Barring parameters | 8 | 8 | 4 | 4 | 8 | 8 | 4 | 4 |
| Number of PLMNs | 12 | 1 | 12 | 1 | 12 | 1 | 12 | 1 |
| Option 1: Index option | 2268 | 189 | 1512 | 126 | 540 | 45 | 360 | 30 |
| Option 2: Bitmap option | 6048 | 504 | 3024 | 252 | 1440 | 120 | 720 | 60 |

# 3	Conclusions

Observation 1: The size of the advertised information is significantly smaller if the number of Access Categories are decrease from 63 to 15.

Observation 2: The size of the advertised information is significantly smaller if the number of sets of barring parameters is limited.

Observation 3: The size of the information to be advertised is smaller with option 1.

Proposal 1: It is proposed to further discuss whether the number of Access Categories can be limited in Rel-15 to reduce the signalling overhead.

Proposal 2: It is proposed to limit the number of sets of barring parameters to a small number. The maximum number of barring parameter sets is FFS.

Proposal 3: It is proposed to adopt option 1

# 4	Annex

The attached excel table was used for the calculations.