3GPP TSG-RAN WG2 Meeting #101bis	R2-1806495

# Sanya, China, 16 – 20 April 2018

Title:	LS on Removal of csi-RS-for-tracking from TCI-State

Release:	Release 15

Work Item:	NR_newRAT-Core

Source:	TSG RAN WG2

To:	TSG RAN WG1

Cc:	-

Contact Person:

#### Name:	Sergio Parolari

E-mail Address:	Sergio.Parolari@zte.com.cn

1. Overall Description:

RAN2 has discussed the content of the current ASN.1 definition for the TCI state IE and wonders whether the csi-RS-for-tracking IE (defined as NZP-CSI-RS-ResourceSetId) should be considered as a possible reference signal in the QCL-Info field.

#### –	TCI-State

The TCI-State IE associates one or two DL reference signals with a corresponding quasi-colocation (QCL) type.

# TCI-State information element

-- ASN1START

-- TAG-TCI-STATE-START

TCI-State ::= 				SEQUENCE {

tci-StateId					TCI-StateId,

qcl-Type1					QCL-Info,

qcl-Type2					QCL-Info																					OPTIONAL,	-- Need R

nrofPTRS-Ports				ENUMERATED {n1, n2}																			OPTIONAL,	-- Need R

...

}

QCL-Info ::=				SEQUENCE {

-- The carrier which the RS is located in. If the field is absent, it applies to the serving cell in which the TCI-State is configured

cell						ServCellIndex																				OPTIONAL,	-- Need R

-- The DL BWP which the RS is located in.

bwp-Id						BWP-Id			OPTIONAL, -- Cond CSI-RS-Indicated

-- Cond NZP-CSI-RS-Indicated: mandatory if csi-rs or csi-RS-for-tracking is included, absent otherwise

referenceSignal				CHOICE {

csi-rs						NZP-CSI-RS-ResourceId,

ssb							SSB-Index,

-- A set of CSI-RS resources for tracking

csi-RS-for-tracking			NZP-CSI-RS-ResourceSetId

},

qcl-Type					ENUMERATED {typeA, typeB, typeC, typeD},

...

}

-- TAG-TCI-STATE-STOP

-- ASN1STOP

2. Actions:

RAN2 invites RAN1 to check whether the csi-RS-for-tracking IE should actually be part of the TCI-State IE (and in case provide justification and/or reference to a section of their specification) or whether it can be removed and placed somewhere else, if needed.

3. Date of Next TSG-RAN WG2 Meetings:

TSG-RAN WG2 #102		2018.05.21 – 2018.05.25		Busan, South Korea

TSG-RAN WG2 #AH1807	2018.07.02 – 2018.07.06		Montreal, Canada