3GPP TSG-RAN WG2 Meeting #101bis		R2-1804895

Sanya, China, April 16-20, 2018

Agenda Item:	9.12.2

Source: 		Intel Corporation

Title: 		SUL and AUL coexistence

Document for:			Discussion and Decision

# Introduction

RAN1 agreed that:

Additional HARQ processes are not introduced for AUL transmission (i.e. AUL supports 16 HARQ processes just like Re-14 eLAA) - The HARQ process IDs allowed for AUL operation are UE specifically RRC configured - All 16 HARQ-processes can be used for scheduled transmissions

Based on the RAN1 agreement, there is no AUL specific HARQ process ID. Therefore, both AUL and SUL share the 16 HARQ process IDs.

RAN 2 agreed that:

In the LAA autonomous UL access, HARQ processes are not tied to TTIs.

Based on the RAN1 and RAN2 agreement, this means eNB does not know a-priori the HARQ-ID the UE chooses for AUL transmission. And vice-versa UE does not know a-priori the HARQ-ID the eNB chooses for SUL grant. Therefore, some rules need to be in place to mitigate the possible collision of HARQ-ID.

# Discussion

In order to see the conflict of HARQ process between SUL and AUL, let’s examine the following timeline where AUL and SUL coexistence may occur:

Figure 1

## Scenario 1

UE receives SUL grant for the retransmission of AUL enabled HARQ process ID#k before or the same subframe as the HARQ NACK. There are 2 cases that need to be considered here depending on the contents of the SUL grant:

Case A: SUL grant with the DCI indicating same AUL enabled HARQ process ID, same TBS, and NDI non-toggled and before timer X expires or stopped.

Case B: SUL grant with matched AUL enabled HARQ process ID, but different TBS size with NDI toggled.

For Case A, as described from our contribution [1] from last meeting, the UE should follow the SUL grant for the AUL retransmission.

For Case B, this can happen when eNB doesn’t see the AUL subframe at n+4 and decided to schedule a new transmission. In this case the SUL grant should be discarded.

Proposal 1: If a SUL is to be used by the UE for a AUL retransmission, the SUL  grant with DCI indicating same HARQ process ID, same TBS, and NDI non-toggled must be received before timer X expires, or in the same subframe of the HARQ NACK feedback for the AUL (re)transmission. Otherwise (including TBS mismatch, NDI toggled), the SUL DCI (indicating the same HARQ process ID as the AUL retransmission) will be discarded.

## Scenario 1+:

UE receives SUL grant for the retransmission of AUL enabled HARQ process ID#k before or the same subframe as the HARQ NACK

Similar as Scenario 1, but AUL subframes are available from n+11. In this case, since a valid SUL grant (i.e. same TBS and NDI non-toggled for the AUL HARQ process and received before HARQ NACK and Timer X expiry or stopped) comes before or the same subframe as HARQ NACK, hence the retransmission of the HARQ process should follow the SUL grant.

Proposal 2: UE should not use an available AUL subframe for AUL retransmission if it has received a valid SUL grant (i.e. same TBS and NDI non-toggled for the AUL enabled HARQ process and received before HARQ NACK and Timer X expiry or stopped)

## Scenario 2:

SUL grant for the HARQ process ID#k is receives after HARQ NACK or Timer X expiry

When a SUL grant for an AUL enabled HARQ process is received after the HARQ NACK feedback of the AUL enabled HARQ process is received or Timer X expiry, the UE should discard the SUL

Proposal 3: UE should discard the SUL grant with DCI indicating the AUL enabled HARQ process if it is received after the HARQ NACK feedback for the AUL enabled HARQ process.

## Scenario 3:

SUL grant for the HARQ process ID#k is received during UL HARQ RTT Timer

By definition UL HART RTT timer specifies the minimum amount of subframe(s) before an UL HARQ retransmission grant is expected by the MAC entity. Therefore, a SUL grant comes in while the UL HART RTT timer is running most likely is as a result of a race condition between timer x and eNB generating a SUL grant. To avoid a double transmission of the same TB, the best way is to discard the SUL grant.

Proposal 4: UE shall discard a SUL grant to the AUL enabled HARQ process when the UL HARQ RTT timer is running.

## Scenario 4:

UE receives HARQ ACK for the AUL enabled HARQ process ID#k and SUL grant for the same HARQ process is received after.

When a SUL grant for an AUL enabled HARQ process is received after the HARQ ACK feedback of the AUL enabled HARQ process is received, the UE MAC can use the HARQ process for the next AUL transmission or it may be used by the eNB for new transmission over SUL. For example, the AUL transmission with HARQ-ID X has been ACKed. At N-4, a SUL grant with HARQ-ID X and the next AUL opportunity is at N. Since the SUL comes in 4 subframes before the AUL opportunity, the UE shall then follow the SUL grant with the HARQ ID X, as per the agreement that SUL transmission always overrides the AUL transmission. In fact, the UE shall accept any SUL grant with the HARD ID X after the HARQ ACK for the HARQ ID X and before N-4 as long as the HARQ ID X is not used.  However, if the SUL grant with HARQ ID X is received less than 4 subframes before the AUL subframe in which the UE MAC has selected HARQ ID X, in order to prevent SUL and AUL collision, the SUL grant with HARQ ID X after n-3 shall be ignored

Proposal 5: UE shall follow the SUL grant for a new transmission if the HARQ process for the SUL grant received is not used by AUL transmission/retransmission. If the SUL grant for a HARQ process is received less than 4 subframes before the AUL subframe in which the UE MAC has selected the HARQ process for the AUL transmission, the UE should ignore the SUL grant.

# Conclusions

We have the following proposals:

Proposal 1: If a SUL is to be used by the UE for a AUL retransmission, the SUL  grant with DCI indicating same HARQ process ID, same TBS, and NDI non-toggled must be received before timer X expires, or in the same subframe of the HARQ NACK feedback for the AUL (re)transmission. Otherwise (including TBS mismatch, NDI toggled), the SUL DCI (indicating the same HARQ process ID as the AUL retransmission) will be discarded.

Proposal 2: UE should not use an available AUL subframe for AUL retransmission if it has received a valid SUL grant (i.e. same TBS and NDI non-toggled for the AUL enabled HARQ process and received before HARQ NACK and Timer X expiry or stopped)

Proposal 3: UE should discard the SUL grant with DCI indicating the AUL enabled HARQ process if it is received after the HARQ NACK feedback for the AUL enabled HARQ process.

Proposal 4: UE shall discard a SUL grant to the AUL enabled HARQ process when the UL HARQ RTT timer is running.

Proposal 5: UE shall follow the SUL grant for a new transmission if the HARQ process for the SUL grant received is not used by AUL transmission/retransmission. If the SUL grant for a HARQ process is received less than 4 subframes before the AUL subframe in which the UE MAC has selected the HARQ process for the AUL transmission, the UE should ignore the SUL grant.

# References

[1]	R2-1802892, Remaining Issues on HARQ aspects on AUL, Intel Corporation, 3GPP TSG-RAN WG2 #101, Athens, Greece, Feb 2018.