3GPP TSG-RAN WG2#102 	R2-1806994

# Busan, Korea, 21th-25th May 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 0118 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Remaining correction on beam failure detection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-05-10 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RAN2#101, UP session, it was agreed to configure the beam failure detection timer (beamFailureDetectionTimer) in number of periods of the BFD RS, i.e. in beam failure instance notification periods. Since this leads to the timer expiring exactly when beam failure instance might be received it raises an ambiguity in the UE behaviour which is undefined in that case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Section 5.17, it is clarified that the BFI_COUNTER is reset once the timer has expired, which necessarily locates the event after the timer expiry and therefore after a potential reception of a beam failure instance notification. Impact analysis Impacted functionality: Beam failure detection. Inter-operability: - If the network is implemented according to the CR and the UE is not, the UE behavior remains undefined and is therefore left to UE implementation how to resolve the ambiguity. - If the UE is implemented according to the CR and the network is not, there is no impact since the functional impact is only in the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE behavior is undefined when the timer expires exactly when a beam failure instance is received, which is not a rare event. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | This CR is implemented on top of rapporteur’s latest running CR 57 rev2 in R2-1806229 |  |  |  |  |  |  |  |  |

Start of change

## 5.17	Beam Failure Detection and Recovery procedure

The MAC entity may be configured by RRC with a beam failure recovery procedure which is used for indicating to the serving gNB of a new SSB or CSI-RS when beam failure is detected on the serving SSB(s)/CSI-RS(s). Beam failure is detected by counting beam failure instance indication from the lower layers to the MAC entity.

RRC configures the following parameters in the BeamFailureRecoveryConfig for the Beam Failure Detection and Recovery procedure:

-	beamFailureInstanceMaxCount for the beam failure detection;

-	beamFailureDetectionTimer for the beam failure detection;

-	candidateBeamThreshold: an RSRP threshold for the beam failure recovery;

-	preamblePowerRampingStep: preamblePowerRampingStep for the beam failure recovery;

-	preambleReceivedTargetPower: preambleReceivedTargetPower for the beam failure recovery;

-	preambleTransMax: preambleTransMax for the beam failure recovery;

-	ra-ResponseWindow: the time window to monitor response(s) for the beam failure recovery using contention-free Random Access Preamble;

-	prach-ConfigIndex: prach-ConfigIndex for the beam failure recovery;

-	ra-ssb-OccasionMaskIndex: ra-ssb-OccasionMaskIndex for the beam failure recovery;

-	ra-OccasionList: ra-OccasionList for the beam failure recovery.

The following UE variables are used for the beam failure detection procedure:

-	BFI_COUNTER: counter for beam failure instance indication which is initially set to 0.

The MAC entity shall:

1>	if beam failure instance indication has been received from lower layers:

2>	start or restart the beamFailureDetectionTimer;

2>	increment BFI_COUNTER by 1;

2>	if BFI_COUNTER >= beamFailureInstanceMaxCount:

3>	initiate a Random Access procedure (see subclause 5.1) on the SpCell by applying the parameters configured in BeamFailureRecoveryConfig.

1>	if the beamFailureDetectionTimer has expired:

2>	set BFI_COUNTER to 0.

1>	if the Random Access procedure is successfully completed (see subclause 5.1):

2>	consider the Beam Failure Recovery procedure successfully completed.

End of change