3GPP TSG-RAN WG2 Meeting #102	R2-1807027

# Busan, Republic of Korea, 21st – 25th May 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 0125 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CBRA BFR on SpCell |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-05-09 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1. As agreed at RAN2-101, the RA configuration in BeamRecoveryConfig is not applicable for contention based random access. Instead, the UE shall apply the normal RA configuration (in rach-ConfigCommon) for contention based beam failure recovery. However, this is not clear from the current MAC specification. The beam failure recovery procedure should be clarified for the case of contention-based random access. 2. When performing CBRA BFR as a fallback from CFRA, it is not clear which parameters to use, the ones in beamFailureRecoveryConfig or rach-ConfigCommon. The MAC Entity shall use the ones in beamFailureRecoveryConfig. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. The absence of beamRecoveryConfig is interpreted by the UE to perform BFR using CBRA. 2. It is clarified that the MAC Entity shall use the parameters in beamFailureRecoveryConfig. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CBRA on BFR on SpCell is not supported properly since it remains unclear which RA configuration the UE shall apply in this case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.17, 5.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 5.17	Beam Failure Detection and Recovery procedure

The MAC entity may be configured by RRC with a beam failure recovery procedure which is used for indicating to the serving gNB of a new SSB or CSI-RS when beam failure is detected on the serving SSB(s)/CSI-RS(s). Beam failure is detected by counting beam failure instance indication from the lower layers to the MAC entity.

RRC configures the following parameters in the BeamFailureRecoveryConfig for the Beam Failure Detection and Recovery procedure:

-	beamFailureInstanceMaxCount for the beam failure detection;

-	beamFailureDetectionTimer for the beam failure detection;

-	beamFailureCandidateBeamThreshold: an RSRP threshold for the beam failure recovery;

-	preamblePowerRampingStep: preamblePowerRampingStep for the beam failure recovery;

-	preambleReceivedTargetPower: preambleReceivedTargetPower for the beam failure recovery;

-	preambleTxMax: preambleTxMax for the beam failure recovery;

-	ra-ResponseWindow: the time window to monitor response(s) for the beam failure recovery using contention-free Random Access Preamble.

The following UE variables are used for the beam failure detection procedure:

-	BFI_COUNTER: counter for beam failure instance indication which is initially set to 0.

The MAC entity shall:

1>	if beam failure instance indication has been received from lower layers:

2>	start or restart the beamFailureDetectionTimer;

2>	increment BFI_COUNTER by 1;

2>	if BFI_COUNTER = beamFailureInstanceMaxCount + 1:

3>	if beamFailureRecoveryConfig is configured:

4>	initiate a Random Access procedure (see subclause 5.1) on the SpCell by applying the parameters preamblePowerRampingStep, preambleReceivedTargetPower, and preambleTxMax configured in beamFailureRecoveryConfig.

3>	else:

4>	initiate a Random Access procedure (see subclause 5.1) on the SpCell;

1>	if the beamFailureDetectionTimer expires:

2>	set BFI_COUNTER to 0.

1>	if the Random Access procedure is successfully completed (see subclause 5.1):

2>	consider the Beam Failure Recovery procedure successfully completed.