3GPP TSG-RAN WG2 #102	R2-1807101

# Busan, Republic of Korea, 21st – 25th May 2018			(revision of R2-1804990)

Agenda Item:	10.4.1.6.2

Source:	Ericsson

Title:	Essential System Information in NR

Document for:	Discussion, Decision

# 1	Introduction

In LTE a UE is required to acquire at least the MIB, SIB1 and SIB2 in order to camp in a cell. If it is not able to do so it should consider the cell as barred, as described in sub-clause 5.2.2.5 (“Essential system information missing”) in TS 36.331.

This contribution discusses the corresponding requirements for considering a cell applicable for camping in NR. A corresponding Text Proposal to 38.331 is also provided.

# 2	Discussion

## 2.1	Essential system information in LTE

As specified for LTE in 36.331 (sub-clause 5.2.2.5), a UE in RRC_IDLE or in RRC_CONNECTED with timer T311 running needs to acquire at least the MIB, SIB1 and SIB2 in order to camp in the cell. Otherwise it will consider the cell as barred and thus not be able to camp in it. Whether it is allowed to perform cell reselection to another cell on the same frequency depends on whether it was able to read the SIB1 or not and, if yes, what the intraFreqReselection parameter (in SIB1) is set to. If the UE managed to acquire the SIB1 it would thus check the intraFreqReselection parameter to determine whether cell reselection within the same frequency is allowed or not. However, if the UE was not able to acquire the SIB1 (or even the MIB) it would consider cell reselection within the same frequency as allowed.

## 2.2	Essential system information in NR

For NR a UE needs to acquire the minimum SI in order to camp on a cell. At the RAN2#101 meeting it was stated, as working assumption, that the minimum SI consists of the MIB and the SIB1. These should thus be considered as the essential system information in NR.

In NR the cellBarred and intraFreqReselection parameters are both included in the MIB. This means that a UE that has acquired the MIB, but is not able to acquire the SIB1, should then check the intraFreqReselection parameter in the MIB to determine whether cell reselection within the same frequency is allowed or not. If the UE is not able to acquire the MIB it should however consider cell reselection within the same frequency to be allowed, just as in LTE (already captured in the text proposal from email discussion [101bis#14][NR] SI procedure text).

A UE in RRC_CONNECTED with timer T311 running, i.e. which is performing an RRC re-establishment procedure, also needs to acquire the MIB and SIB1 of the cell in order to access it. These are thus essential also for such a UE, which should be specified in sub-clause 5.2.2.5 in TS 38.331.

Specify that only the MIB and the SIB1 are essential system information for the UE.

Specify that the essential system information is related to also a UE in RRC_CONNECTED with timer T311 running.

A Text Proposal to sub-clause 5.2.2.5 in TS 38.331 [1], capturing the above proposals, is included in section 3. The TP is based on the one that is the outcome of the email discussion [101bis#14][NR] SI procedure text, with the track changes from that TP being accepted. It is proposed to include the TP in section 3 into TS 38.331.

Include the Text Proposal in section 3 into sub-clause 5.2.2.5 in TS 38.331.

# 3	Text Proposal to TS 38.331

Note that the change marks are relative to the outcome of the email discussion [101bis#14][NR] SI procedure text.

START OF CHANGES

#### 5.2.2.5	Essential system information missing

The UE shall:

1>	if in RRC_IDLE or in RRC_INACTIVE or in RRC_CONNECTED while T311 is running:

2>	if the UE is unable to acquire the MIB:

3>	consider the cell as barred in accordance with TS 38.304 [X]; and

3>	perform barring as if intraFreqReselection is set to allowed;

2>	else if the UE is unable to acquire the SIB1 :

3>	consider the cell as barred in accordance with TS 38.304 [X].

END OF CHANGES

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Specify that only the MIB and the SIB1 are essential system information for the UE.

Proposal 2	Specify that the essential system information is related to also a UE in RRC_CONNECTED with timer T311 running.

Proposal 3	Include the Text Proposal in section 3 into sub-clause 5.2.2.5 in TS 38.331.

# References

3GPP TS 38331; Radio Resource Control (RRC) protocol specification; V15.1.0 (2018-03)