3GPP TSG-RAN WG2 #102	Tdoc R2-1807143

Busan, S. Korea, May 21st – 25th 2018	(re-submission of R2-1805398)

Agenda Item:	10.4.1.4.5

Source:	Ericsson

Title:	Remaining issues in inter-RAT measurements

Document for:	Discussion, Decision

# 1	Introduction

Concerning inter-RAT mobility from NR to LTE, the following agreements were made at RAN2 #99 [1]:

Agreements:

1:	The following events for inter RAT measurements from NR to LTE are supported:

Event B1: Inter RAT neighbour becomes better than threshold

Event B2: PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2

2: Periodical inter-RAT measurement report configuration shall be supported in NR.

3: The measurement object for LTE in NR specification could reuse structure of measurement object for LTE frequency defined in LTE specification.

4: introduce separate MeasResults IE for NR and LTE.

An open issue is still the structure of the measurement object. In addition, there is an open issue regarding the different types of cells, as defined in section 5.5.1 of TS38.331 (in the same way as in 36.331 for LTE) [2]:

The measurement procedures distinguish the following types of cells:

1.	The serving cell(s) - these are the PCell and one or more SCells, if configured for a UE supporting CA.

2.	Listed cells - these are cells listed within the measurement object(s).

3.	Detected cells - these are cells that are not listed within the measurement object(s) but are detected by the UE on the carrier frequency(ies) indicated by the measurement object(s).

For NR measurement object(s), the UE measures and reports on the serving cell(s), listed cells and/or detected cells.

Editor’s Note: FFS Whether the definitions of serving cells, listed cells and detected cells in 38.331 are also applicable for E-UTRAN measurement object(s).

In this paper, we address these two open issues regarding inter-RAT measurements.

# 2	Discussion

## 2.1	Cell Types

The measurement procedures distinguish between serving, listed, and detected cells. The question is which of these are to be measured and reported in the case of E-UTRA measurements in NR. For LTE intra-RAT measurements, all three cell types are measured and reported. We therefore think that, for NR inter-RAT measurements, the UE should measure and report both listed and detected cells.

For E-UTRA inter-RAT measurements in NR, UE should measure and report both listed and detected cells.

## Measurement object structure

The E-UTRA measurement object in LTE is specified in TS36.331 and has the following structure [3]:

MeasObjectEUTRA ::=					SEQUENCE {

carrierFreq							ARFCN-ValueEUTRA,

allowedMeasBandwidth				AllowedMeasBandwidth,

presenceAntennaPort1				PresenceAntennaPort1,

neighCellConfig						NeighCellConfig,

offsetFreq							Q-OffsetRange				DEFAULT dB0,

-- Cell list

cellsToRemoveList					CellIndexList				OPTIONAL,		-- Need ON

cellsToAddModList					CellsToAddModList			OPTIONAL,		-- Need ON

-- Black list

blackCellsToRemoveList				CellIndexList				OPTIONAL,		-- Need ON

blackCellsToAddModList				BlackCellsToAddModList		OPTIONAL,		-- Need ON

cellForWhichToReportCGI				PhysCellId					OPTIONAL,		-- Need ON

...,

[[measCycleSCell-r10				MeasCycleSCell-r10		OPTIONAL,		-- Need ON

measSubframePatternConfigNeigh-r10	MeasSubframePatternConfigNeigh-r10	OPTIONAL							-- Need ON

]],

[[widebandRSRQ-Meas-r11				BOOLEAN	OPTIONAL		-- Cond WB-RSRQ

]],

[[	altTTT-CellsToRemoveList-r12	CellIndexList				OPTIONAL,		-- Need ON

altTTT-CellsToAddModList-r12	AltTTT-CellsToAddModList-r12	OPTIONAL,		-- Need ON

t312-r12						CHOICE {

release							NULL,

setup							ENUMERATED {ms0, ms50, ms100, ms200,

ms300, ms400, ms500, ms1000}

}														OPTIONAL,		-- Need ON

reducedMeasPerformance-r12		BOOLEAN 					OPTIONAL,	 	-- Need ON

measDS-Config-r12				MeasDS-Config-r12			OPTIONAL		-- Need ON

]],

[[

whiteCellsToRemoveList-r13		CellIndexList				OPTIONAL,		-- Need ON

whiteCellsToAddModList-r13		WhiteCellsToAddModList-r13	OPTIONAL,		-- Need ON

rmtc-Config-r13				RMTC-Config-r13			OPTIONAL,		-- Need ON

carrierFreq-r13					ARFCN-ValueEUTRA-v9e0		OPTIONAL			-- Need ON

]],

[[

tx-ResourcePoolToRemoveList-r14	Tx-ResourcePoolMeasList-r14		OPTIONAL,	-- Need ON

tx-ResourcePoolToAddList-r14	Tx-ResourcePoolMeasList-r14		OPTIONAL,	-- Need ON

fembms-MixedCarrier-r14				BOOLEAN					OPTIONAL			-- Need ON

]]

}

In our view, the following parameters are undoubtedly needed for inter-RAT measurements in NR and should thus be included in the new measurement object:

- carrierFreq

- allowedMeasBandwidth

- presenceAntennaPort1

- offsetFreq

- cellsToRemoveList and cellsToAddModList

- blackCellsToRemoveList and blackCellsToAddModList

- widebandRSRQ-Meas

- measDS-Config

- whiteCellsToRemoveList and whiteCellsToAddModList

The following parameters are already supported in LTE and they might be beneficial to have similar behaviour for inter-RAT measurements from NR compared to intra-RAT measurement within LTE.

- altTTT-CellsToRemoveList and altTTT-CellsToAddModList

- t312

- reducedMeasPerformance

Parameters that are probably not needed:

- cellForWhichToReportCGI: This parameter can be moved to the reportConfigNR as has been mentioned in ANR related discussions.

- measSubframePatternConfigNeigh

- fembms-MixedCarrier: This will just inform the UE that the configured carrier is an MBMS related carrier. This could be useful for some slices! (?) However, purely from the RRM measurements purpose this is not required. Hence remove?

- neighCellConfig: used to provide the information related to MBSFN and TDD UL/DL configuration of neighbour cells, relative to the serving cell. Since in our case the serving cell will be an NR cell, this IE is not needed. Information about the configuration of neighbour cells can be provided for example through measSubframePatternConfigNeigh.

- measCycleSCell: used to indicate measurement cycle for SCell configured on the frequency of the MO, and thus not needed with NR serving cell.

- rmtc-Config: used to configure RSSI and channel occupancy measurement for LAA, and thus not needed.

- tx-ResourcePoolToRemoveList and tx-ResourcePoolToAddList: parameters used for V2X sidelink communication, and thus not needed.

Include the following parameters in the E-UTRA measurement object for inter-RAT measurements in NR.

  - carrierFreq

  - allowedMeasBandwidth

  - presenceAntennaPort1

  - offsetFreq

  - cellsToRemoveList and cellsToAddModList

  - blackCellsToRemoveList and blackCellsToAddModList

  - widebandRSRQ-Meas

  - measDS-Config

  - whiteCellsToRemoveList and whiteCellsToAddModList

  - altTTT-CellsToRemoveList and altTTT-CellsToAddModList

  - t312

  - reducedMeasPerformance

Adopt the changes to ASN.1 structure in NR.

#### –	MeasObjectEUTRA

The IE MeasObjectEUTRA specifies information applicable for E-UTRA cells.

MeasObjectEUTRA information element

-- ASN1START

MeasObjectEUTRA ::=					SEQUENCE {

carrierFreq							ARFCN-ValueEUTRA,

allowedMeasBandwidth				AllowedMeasBandwidth,

presenceAntennaPort1				PresenceAntennaPort1,

offsetFreq							Q-OffsetRange				DEFAULT dB0,

-- Cell list

cellsToRemoveList					CellIndexList				OPTIONAL,		-- Need ON

cellsToAddModList					CellsToAddModList			OPTIONAL,		-- Need ON

-- Black list

blackCellsToRemoveList				CellIndexList				OPTIONAL,		-- Need ON

blackCellsToAddModList				BlackCellsToAddModList		OPTIONAL,		-- Need ON

widebandRSRQ-Meas					BOOLEAN	OPTIONAL		-- Cond WB-RSRQ

altTTT-CellsToRemoveList			CellIndexList				OPTIONAL,		-- Need ON

altTTT-CellsToAddModList			AltTTT-CellsToAddModList	OPTIONAL,		-- Need ON

t312							CHOICE {

release							NULL,

setup							ENUMERATED {ms0, ms50, ms100, ms200,

ms300, ms400, ms500, ms1000}

}														OPTIONAL,		-- Need ON

reducedMeasPerformance			BOOLEAN 					OPTIONAL,	 	-- Need ON

measDS-Config					MeasDS-Config				OPTIONAL		-- Need ON

whiteCellsToRemoveList			CellIndexList				OPTIONAL,		-- Need ON

whiteCellsToAddModList			WhiteCellsToAddModList		OPTIONAL,		-- Need ON

}

CellsToAddModList ::=				SEQUENCE (SIZE (1..maxCellMeas)) OF CellsToAddMod

CellsToAddMod ::=	SEQUENCE {

cellIndex							INTEGER (1..maxCellMeas),

physCellId							PhysCellId,

cellIndividualOffset				Q-OffsetRange

}

BlackCellsToAddModList ::=			SEQUENCE (SIZE (1..maxCellMeas)) OF BlackCellsToAddMod

BlackCellsToAddMod ::=	SEQUENCE {

cellIndex							INTEGER (1..maxCellMeas),

physCellIdRange						PhysCellIdRange

}

AltTTT-CellsToAddModList ::=	SEQUENCE (SIZE (1..maxCellMeas)) OF AltTTT-CellsToAddMod

AltTTT-CellsToAddMod ::=	SEQUENCE {

cellIndex							INTEGER (1..maxCellMeas),

physCellIdRange						PhysCellIdRange

}

WhiteCellsToAddModList ::=			SEQUENCE (SIZE (1..maxCellMeas)) OF WhiteCellsToAddMod

WhiteCellsToAddMod ::=	SEQUENCE {

cellIndex							INTEGER (1..maxCellMeas),

physCellIdRange						PhysCellIdRange

}

-- ASN1STOP

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	For E-UTRA inter-RAT measurements in NR, UE should measure and report both listed and detected cells.

Proposal 2	Include the following parameters in the E-UTRA measurement object for inter-RAT measurements in NR.

a.	carrierFreq

b.	allowedMeasBandwidth

c.	presenceAntennaPort1

d.	offsetFreq

e.	cellsToRemoveList and cellsToAddModList

f.	blackCellsToRemoveList and blackCellsToAddModList

g.	widebandRSRQ-Meas

h.	measDS-Config

i.	whiteCellsToRemoveList and whiteCellsToAddModList

j.	altTTT-CellsToRemoveList and altTTT-CellsToAddModList

k.	t312

l.	reducedMeasPerformance

Proposal 3	Adopt the changes to ASN.1 structure in NR.

# References

38.300, 3GPP; Technical Specification Group Radio Access Network; NR; NR and NG-RAN Overall Description; Stage 5 (Release - 15), V15.1.0, 2018-03.

38.331, 3GPPP; Technical Specification Group Radio Access Network; NR; Radio Resource Control (RRC) Protocol Specification, Release 15, V15.1.0, 2018-03.