3GPP TSG-RAN WG2 #102	R2-1807193

# Busan, Republic of Korea, 21st – 25th May 2018

Agenda Item:	10.3.4.3

Source:	Ericsson

Title:	SDAP QFI mapping between AS and NAS

Document for:	Discussion, Decision

# Introduction

The last RAN2 meeting agreed:

Agreements:

The presence of SDAP header is configured per DRB for UL or DL independently

For DL SDAP headers, two bits, one bit RQI and one bit RDI

For UL SDAP header at least a QFI field is present.  RQI and RDI are not needed. FFS on the size of QFI depending on DL

In the January 2018-AH meeting RAN2 agreed:

Agreements

=>	Support independent AS and NAS reflective QoS.

=>	From RAN2 perspective supporting up to 64 reflective flows per PDU session per UE is sufficient at the same time, so 6 bits QFI in SDAP.

=>	Ask SA2/CT1 if they expect to use more than 64 reflective flows per PDU session per UE at a time.  Indicate RAN2 agreement and strong need to have 6bits SDAP.   Questions will be included in SA2 LS from main session.

FFS if final QFI in CT1/SA2 is larger than 6 bits, a mechanism to remap NAS QFI to AS QFI may be needed

RAN2 received LS S2-182856 from SA2

For the SDAP sublayer, in order to keep the header contained within one byte, RAN2 has agreed to limit the number of QFI which can be signalled to 64. Therefore, RAN2 would like to understand if SA2 expects to use more than 64 reflective flows per PDU session per UE at a time.

SA2 acknowledges RAN2 decision and would like to inform RAN2 and CT1 that it has agreed to restrict the maximum number of concurrent QoS flows per PDU Session to 64 accordingly. SA2 has begun to discuss the implications from that limitation on control of the QoS flows mechanisms specified in TS 23.501 section 5.7.3.1, however, has not been able to reach a conclusion so far. SA2 will inform RAN2 and CT1 of the outcome of that discussion.

‘Since then, RAN2 also received the following reply, S2-184359:

….

SA2 has re-discussed this topic and would like to confirm that limiting the QFI to 6 bits (i.e. 64 values) in both NAS and AS is considered adequate for rel.15 5GS system.

In SA2#127, following discussions on some companies’ proposal, SA2 concluded that a potential extension of the QFI beyond 6 bits may be considered in a future release (e.g. in Rel-16). SA2 would like to stress that Release 15 should be defined such that future extensibility should not be precluded.

…

# Discussion

RAN2 have so far confirmed the use of agreed 6-bit QFI use, and also that RDI and RQI bits are present on DL, while leaving details on UL open. For the latter, the QFI on DL and UL header should be symmetric for reduced complexity.

6-bit QFI is used for both DL and UL

As SA2 confirms the agreement to restrict the number of concurrent QFIs per PDU session to 64.

Furthermore, from the LS responses above, it is also clear that the current 6-bit UL SDAP header design fulfils the criteria for QFI usage in NR RAN. With regards to future extensibility, it is not clear from a RAN2 point of view in which circumstances an extended QFI should be supported in future, if possible extensions impact CN, RAN or both and to what extent. Thus, defining adequate requirements and details for such expected support is not feasible at this point in time but should be planned for in Rel-16. I.e. no 2-level mapping mechanism in AS is specified in Rel-15.

No QFI remapping mechanism between AS and NAS QFI is introduced on RAN for Rel-15

Note that the above resulting Rel-15 design does not preclude a future re-use with support for more QFIs for instance in cases NAS reflective QoS is not used. For example, a 6-bit QFI could still be used for QoS Flows where the flow is subject to NAS reflective QoS, and 7-bit QFI used otherwise [1].

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	6-bIt QFI is used for both DL and UL

Proposal 2	No QFI remapping mechanism between AS and NAS QFI is introduced on RAN

# References

R2-1803157, SDAP QFI mapping between AS and NAS, Option 4, Ericsson